Global ITS Communication Requirements (DRAFT)

August 15, 2012

Table of Contents

Introduction	3

1	Vehicle / ITS Station-to-Infrastructure Short-Range Communications	4

2	Vehicle / ITS Station-to-Infrastructure Wide-Area Communications	8

3	Wide-Area Communications: Infrastructure to Multiple Vehicles (Broadcast)	12

4	Vehicle-to-Vehicle Communications	15

Appendix A: Questionnaire Responses	18

Introduction

This is a compilation of requirements based on replies to a questionnaire on communications requirements of high-priority ITS applications in different countries and regions. The information from that questionnaire has been reorganized to highlight the communications requirements themselves. Each section indicates the type of communications, the applications and regions that have indicated a need for them, and the requirements themselves. Specific values for requirements, when they have been stated, have been selected from the questionnaire response summary.

The original text of the questionnaire response summary is attached as Appendix A.

# Vehicle / ITS Station-to-Infrastructure Short-Range Communications

## Related Applications

Air traffic management, ship management

Arab region

CCTV surveillance

Arab region

Electronic toll collection (ETC)

China

Emission control

Arab region

- Fleet management, hazardous material tracking, vehicle monitoring (See also Section 2, Section 3)

Arab region, China, Europe, Japan

Improving emergency response

Arab region

- Loading zone management, border crossing management

Arab region, China, Europe, Japan

Road hazard warning (e.g., road workers, accident)

Arab region, Europe

Smart pedestrian crossings (e.g. automatic warning of pedestrians for drivers, prolonging crossing times for elderly users, and changing pedestrian prioritization)

Arab region, Europe

Speed warnings / regulation enforcement / signage reinforcement

Arab region, Europe

Traffic signal violation detection and enforcement

Arab region, Europe

Traveler information, urban multi-modal information

Arab region

## Requirements

### Addressing

Applications such as speed warning and emission control require point-to-multipoint addressing.

### Bandwidth / Throughput

Medium to high bandwidth is needed for the majority of these applications.

Multiple channel support is required for applications such as vehicle monitoring.

### Connection Type (Persistent or Not)

Applications such as speed warnings and emission control require both types of connection.

### Coverage

TBD

### Data Integrity

Applications such as speed warning and signage reinforcement require highly robust signals, as indicated in the Arab region’s requirements for signage reinforcement:

“Data to be transmitted such that when in radio range (say 250m) any approaching vehicle will receive the signal reinforcement even in the event of 80% message loss (note that transmitters can be placed above cars such as to maximize coverage and optimize communication path integrity)”

See also Reliability, below.

### Directionality

TBD

### Latency

Applications such as speed warning and road hazard warning require very low latency or 100 ms or less.

Applications such as fleet management and loading zone management require maximum latency of 500 ms.

Most of these applications require both real-time and stored transmission.

### Pre-emption Priority of Applications

Safety and law enforcement applications require pre-emption priority.

### Quality of Service

High level of QoS is needed for applications such as air traffic management, speed warning, and road hazard warning.

### Range

Range for these applications varies from tens of meters to one kilometer.

### Reliability

Services such as ETC require high reliability defined in terms of requirements such as:

IPTD (IP Packet Transfer Delay)

IPLR (IP Packet Loss Ratio)

IPER (IP Packet Error Ratio)

Sleep mode, transmission restoration, and multilink support are required for applications such as fleet management, hazardous material tracking, loading zone management, and border crossing management.

### Reserved Bandwidth Needed

5.9 GHz spectrum might be used for some public safety applications.

### Security and Privacy

High security is needed for applications such as air traffic management, ETC, speed warning, road hazard warning, and emission control.

Features needed include security against data interception and manipulation:

Authentication: To make acceptance authentication for received data, to prove the validity of the identity and data source, the certification methods includes: MAC verification, PKC certification and C/R authentication

Privacy control: To avoid illegal monitoring of sensitive information by means of information encryption, encryption methods includes: DEA, FEAL, RAS etc.

Data integrity control: CRC check; MAC verification

### Topology

Services such as fleet management, loading zone management, and border crossing management require broadcast capability to inform ITS stations of their availability. They also require unicast capability to enable I2V data transmission.

## Related Standards

General Architecture of Intelligent Transportation System Based on Telecommunication Networks (Draft 1.0), CCSA TC10 WG2.

Intelligent transport system - Electronic toll collection - System architecture (GB 20135-2006-T)

Electronic toll collection - Dedicated short range communication - Part 1: Physical layer (GBT 20851.1-2007)

Electronic toll collection - Dedicated short range communication - Part 2: Data link layer (GBT 20851.2-2007)

Electronic toll collection - Dedicated short range communication - Part 3: Application layer (GBT 20851.3-2007)

Electronic toll collection - Dedicated short range communication - Part 4: Equipment application (GBT 20851.4-2007)

Existing ISO, IEEE and ETSI standards for Vehicle to Vehicle and Vehicle to Infrastructure communication

IETF specifications for Internet communications

3GPP and other mobile standards for communication

ETSI ITS and ISO CALM

GNSS system for operating vehicles – Technical specification for vehicle terminals (JT/T794-2011)

GNSS for operating vehicles – Technical specification for platform (JT/T796-2011)

Classified criteria for security (GB17859-1999)

## Standards Gaps

See Section 3.2.8 of Appendix A.

# Vehicle / ITS Station-to-Infrastructure Wide-Area Communications

## Related Applications

Cooperative navigation (probe traffic data, etc.)

Europe

- Disaster information

Japan

- Evacuation route information

Japan

- Fleet management, hazardous material tracking, vehicle monitoring (See also Section 1, Section 3)

Arab region, China, Europe, Japan

Floating car (probe) data

Europe

- “I am alive” message

Japan

- Loading zone management, border crossing management (See also Section 1)

Arab region, China, Europe, Japan

- Location-based services / infotainment / insurance / financial services (See also Section 4)

Arab region, Europe, Japan

Management of electric vehicles and electric vehicle charging spots

Japan

- Navigation, including call-center assisted navigation

Europe, China, Japan

- Person finder

Japan

- Refugee information

Japan

Vehicle diagnostic information

China

- Weather information

Arab region

## Requirements

### Addressing

TBD

### Bandwidth / Throughput

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information do not require high bandwidth.

Infotainment, navigation, and vehicle monitoring applications require moderate to high bandwidth. Some require multiple channels (audio, video, etc.)

### Connection Type (Persistent or Not)

TBD

### Coverage

TBD

### Data Integrity

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information require robust communication.

### Directionality

TBD

### Latency

Applications such as fleet management, loading zone management, cooperative navigation, location-based services, and infotainment services require maximum latency of 500 ms.

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information require low latency.

### Pre-emption Priority of Applications

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information require pre-emption priority.

### Quality of Service

Most of these applications require a high level of QoS.

### Range

TBD

### Reliability

Most of these applications require high reliability.

### Reserved Bandwidth Needed

TBD

### Security and Privacy

Applications including vehicle monitoring, vehicle diagnostic information, and management of electric vehicles require protection against unauthorized access to the CAN bus. Security measures include authentication for the OBE and SIM card, private data transmission, OBE-side encryption, and OBE-side data verification.

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information require security ranging from moderate to high.

Applications such as management of electric vehicles and charging spots and vehicle monitoring require moderate or high security, including encryption and VPN capability.

Navigation, including call-center assisted navigation, requires security measures including encryption, verification, long-term storage, and user profile protection.

Location-based services, infotainment, insurance, and financial services also require strong security and privacy measures.

### Topology

Applications such as floating car (probe) data require the capability for the ITS station to broadcast or geocast information.

Applications such as location-based services and some infotainment services require the roadside unit to have the capability to broadcast availability of the service.

Applications such as disaster information, evacuation route information, “I am alive” message, person finder, refugee information, and weather information require both one-way, wide-area broadcast and two-way transmission.

## Related Standards

ISO TC204/CALM

IEEE 802.11p

WAVE/DSRC

3GPP

3GPP2

ETSI TC ITS

Vehicle Telematic Service Requirement and General Framework (YDB 064-2011), CCSA TC10 WG2

GNSS system for operating vehicles – Technical specification for vehicle terminals (JT/T794-2011)

GNSS for operating vehicles – Technical specification for platform (JT/T796-2011)

Classified criteria for security (GB17859-1999)

Standards suggested for infotainment applications:

In-vehicle information network architecture;

The interface specifications between mobile communication equipments (cellular phone, DSRC etc.) and the in-vehicle information network (LAN)

The interface specifications of gateway between the in-vehicle information network (LAN) and vehicle CAN bus

The interface specifications between in-vehicle information network (LAN) and information terminals (nomadic and mobile devices)

The firewall function of the gateway between the in-vehicle information network (LAN) and vehicle CAN bus

## Standards Gaps

See Section 2.1.8, Section 3.1.8, and 5.1.8 of Appendix A.

# Wide-Area Communications: Infrastructure to Multiple Vehicles (Broadcast)

## Related Applications

- Disaster information (See also Section 2)

Japan

- Fleet management, hazardous material tracking, vehicle monitoring (See also Section 1, Section 2)

Arab region, China, Europe, Japan

- “I am alive” message (See also Section 2)

Japan

- ITS station lifecycle management

Europe

Refugee information (See also Section 2)

Japan

## Requirements

### Addressing

TBD

### Bandwidth / Throughput

Applications such as disaster information, “I am alive” message, and refugee information do not require high bandwidth.

Fleet management and vehicle monitoring applications require moderate to high bandwidth. Some require multiple channels (audio, video, etc.)

### Connection Type (Persistent or Not)

TBD

### Coverage

TBD

### Data Integrity

Applications such as disaster information, “I am alive” message, and refugee information require robust communication.

### Directionality

TBD

### Latency

Applications such as fleet management and ITS station life cycle management require maximum latency of 500 ms.

Applications such as disaster information, “I am alive” message, and refugee information require low latency.

### Pre-emption Priority of Applications

Applications such as disaster information, “I am alive” message, and refugee information require pre-emption priority.

### Quality of Service

Most of these applications require a high level of QoS.

### Range

TBD

### Reliability

Most of these applications require high reliability.

### Reserved Bandwidth Needed

5.9 GHz spectrum might be used for some public safety applications.

ITS station life cycle management might use 5.9 GHz.

### Security and Privacy

Applications including vehicle monitoring require protection against unauthorized access to the CAN bus. Security measures include authentication for the OBE and SIM card, private data transmission, OBE-side encryption, and OBE-side data verification.

Applications such as disaster information, “I am alive” message, and refugee information require security ranging from moderate to high.

Applications such as vehicle monitoring require moderate or high security, including encryption and VPN capability.

ITS station lifecycle management requires security to prevent unauthorized code or data from being installed on the ITS station.

### Topology

Applications such as disaster information, “I am alive” message, and refugee information require both one-way, wide-area broadcast and two-way transmission.

## Related Standards

GNSS system for operating vehicles – Technical specification for vehicle terminals (JT/T794-2011)

GNSS for operating vehicles – Technical specification for platform (JT/T796-2011)

Classified criteria for security (GB17859-1999)

## Standards Gaps

See Section 3.1.8 of Appendix A.

# Vehicle-to-Vehicle Communications

## Related Applications

- Ad hoc, multi-hop V2V safety communications

Japan

- Blind spot warning / Lane change warning

U.S.A.

- Do not pass warning

U.S.A.

- Emergency brake light warning

Europe, U.S.A.

Emergency vehicle warning

Arab region, Europe

- Forward collision warning

Arab region, Europe, U.S.A.

- Intersection movement assist

U.S.A.

- Left turn across path / opposite direction

U.S.A.

- Location-based services / infotainment / insurance / financial services

Arab region, Europe, Japan

Motorcycle approaching warning

Europe

Slow vehicle warning

Europe

Wrong-way driving warning

Europe

## Requirements

### Addressing

Ad-hoc, multi-hop V2V safety communications requires a routing protocol among vehicles.

### Bandwidth / Throughput

Safety applications do not require high bandwidth.

Infotainment, navigation, and vehicle monitoring applications require moderate to high bandwidth. Some require multiple channels (audio, video, etc.)

### Connection Type (Persistent or Not)

TBD

### Coverage

TBD

### Data Integrity

Safety applications require high data integrity.

### Directionality

Safety applications require signals to be broadcast from the threat vehicle to multiple surrounding vehicles.

### Latency

Because of the short distances and high speeds of vehicles in threat situations, safety applications require very low latency. ETSI has suggested maximum latency of 100 ms.

Location-based services and infotainment services require maximum latency of 500 ms.

### Pre-emption Priority of Applications

Safety applications require pre-emption priority.

### Quality of Service

Ad-hoc, multi-hop V2V safety communications requires a high level of QoS.

### Range

Longitudinal detection distance for safety applications is defined by USDOT as a maximum 300 m from the front bumper of the host vehicle.

### Reliability

Safety applications must function even when the line of sight between the host vehicle and the threat vehicle is blocked by other vehicles, inclement weather, or terrain.

### Reserved Bandwidth Needed

U.S. safety applications will use the 5.9 GHz band.

### Security and Privacy

Ad-hoc, multi-hop V2V safety communications requires encryption and VPN capability.

Security for ad-hoc, multi-hop V2V safety communications must be high.

Location-based services, infotainment, insurance, and financial services also require strong security and privacy measures.

### Topology

Safety applications require signals to be broadcast from the threat vehicle to multiple surrounding vehicles.

## Related Standards

ISO TC204/CALM

IEEE 802.11p

WAVE

DSRC

ITS-F/IVC

C2CCC

ETSI TG37

## Standards Gaps

TBD

Appendix A: Questionnaire Responses

Collaboration on ITS Communication Standards

Communications requirements of high-priority ITS applications

This is a compilation of replies to a questionnaire on communications requirements of high-priority ITS applications in different countries and regions.

Last update: 	21 June 2012

Collaboration website:	http://itu.int/en/ITU-T/extcoop/cits/

Table of Contents

1.	Arab region	18

1.1	Traffic Management, Operation / Vehicle Services / Emergency / National Security domains: Policing/enforcing traffic regulations: Law enforcement applications	19

1.2	Traffic Management & Control: Traffic Incident Management (TIM)	21

1.3	Traveller Information &  Traffic Management & Operations (Transport Infrastructure maintenance management): Infrastructure signage reinforcement	23

1.4	Vehicle services: Traveller Assistance Navigation/ Assistive driver awareness	24

1.5	Multimodal Travel Information:  Urban Multi-Modal Routing	26

1.6	Vehicle Services: Intersection Collision Warning/Avoidance	28

1.7	Transport related personal safety- Public travel security: CCTV surveillance	29

1.8	Infotainment: content sharing applications and services (advertisements, entertainment)	30

1.9	Passenger Information System	32

1.10	Air Traffic Management System and Data Processing	34

1.11	Vessel Traffic System	36

1.12	Meteorological Information System for Road and Rail Traffic	38

2.	China (CCSA)	40

2.1	Voice navigation	41

2.2	Operation data report of the vehicle	44

3.	China (ITSC)	47

3.1	Dynamic monitoring for operating Vehicles	48

3.2	ETC (Electronic Toll Collection)	53

4.	Europe (ETSI)	55

4.1	Co-operative ITS – General characteristics and requirements	56

4.2	Co-operative road safety applications: Driving assistance – co-operative awareness	58

4.3	Co-operative road safety applications: Driving assistance – Road hazard warning	61

4.4	Co-operative traffic efficiency: Speed management	65

4.5	Co-operative traffic efficiency: Co-operative navigation	67

4.6	Co-operative local services: Location based services	70

4.7	Global internet services: Communities services	73

4.8	Global internet services: ITS station life cycle management	76

5.	Japan (TTC)	78

5.1	Utilization of In-Vehicle Total Information & Communication Environment	79

5.2	Ad-hoc network between vehicle to vehicle communications for disaster response and resilience	81

5.3	Disaster information	83

5.4	Alive information (“I am alive” message)	84

5.5	Person finder	85

5.6	Evacuation route information	86

5.7	Refugee information	87

5.8	Management of Electric Vehicles and Electric Vehicle charging spots	88

6.	United States of America (Department of Transportation)	90

6.1	Forward-collision warning	91

6.2	Emergency Brake-Light Warning / Emergency Electronic Brake Lights	92

6.3	Intersection Movement Assist	93

6.4	Blind Spot Warning / Lane Change Warning	94

6.5	Do Not Pass Warning	95

6.6	Left Turn Across Path / Opposite Direction	96

1. Arab region

    - Contributors

Team leader:

- Muna Hamdi, iMobility:  FutureVision Group & Virtual Research Centre – University of Huddersfield, UK, munahamdi@mghfuturevision.co.uk

IMFV team:

- Hany K. Hassan, Rakedet, LLC, Hany.K.Hassan@rakedet.com, Egypt

- Scott W Cadzow, Cadzow Communications Consulting Ltd., scott@cadzow.com, United Kingdom

- Ali Abdelfattah, AlBalad AlAmeen - Makkah Municipality, aliitsman@hotmail.com, Saudi Arabia

- Slaheddine Maaref, Tunisie Télécom, Slaheddine.Maaref@tunisietelecom.tn, Tunisia

- Farouk Kamoun, SESAME University, farouk.kamoun@sesame.com.tn, Tunisia

- Hajer Tounisi, Higher School of Communication of Tunis, Hajer.Tounsi@supcom.rnu.tn, Tunisia

Note: Applications 1.9-1.12 were contributed by Wissem Mahjoub, Ministry of Transport, Tunisia.

For general comments, also see input document 4 to Munich meeting of the Collaboration, 26 June 2012.

  - Traffic Management, Operation / Vehicle Services / Emergency / National Security domains: Policing/enforcing traffic regulations: Law enforcement applications

    - Description

This is a group of applications that help in maintaining law and order (in relation to transport) reducing accidents, fatalities and casualties.

Examples of highest priority applications within this group include:

- Speed regulation enforcement

- Red light violation detection and enforcement

- Access control (lane control)

- Monitoring and tracking the transportation of hazardous material

- Improving emergency response

- Emission control

Examples within this group related to Safety enhancements for vulnerable road users  (ISO 14813-1) include:

- Smart pedestrian crossings (e.g. automatic warning of pedestrians for drivers, prolonging crossing times for elderly users, and changing pedestrian prioritization)

- Vehicle speed warning systems

- Vehicle presence detection

- Automatic advice to drivers by vulnerable road users (e.g. presence of road maintenance workers)

Finally in this group are applications related to National Security such as international border commercial vehicle goods carriage clearance (this may be related to digital tachograph capabilities for vehicle and driver identification and extended to cover validation and verification of the contents of the vehicle load area).

Implicit in this set of applications are vehicle and driver identification schemes with verification at checkpoints.

    - Communications requirements: functional

- DSRC for public safety and private operation

- Communication Mode:  C2V, R2V

- Addressing: Point to Multipoint

- Transmission type - Real Time & Storage

- Communication services: Connection oriented (i.e. link remains persistent over many transmitted packets) & Connectionless (i.e. link is established for each packet)

    - Communications requirements: performance

- Range:  short to long (i.e. vehicle identification over a range of a few 10s of meters, data validation may be from point of data collection to central control point 100s of km distant)

- Latency: Very low (i.e. detection has to be accurate to prevent claim of data manipulations),

- Rate: medium to high (i.e. few 10s of bits of data for vehicle identification, many kBytes of data for manifest transfer all to be transferred in very short time period requiring high data transfer rate). Note also that a single identification or measuring point may transfer data pertaining to many entities in the same time period.

- Reliability: high (this is system critical data and needs to be contained in a resilient and reliable system)

- Priority: high

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Authenticity of driver/vehicle/authority

- Authority verification of driver/vehicle/authority

- Integrity of data (e.g. vehicle manifests)

    - Security threats and risks

- Masquerade of authority

- Masquerade of driver

- Masquerade of vehicle

- Manipulation of vehicle manifest

- Manipulation of vehicle speed

- Repudiation of activity of driver/vehicle/authority

- False violation claim

- Confidentiality of manifests

    - Communications standards

- Existing ISO, IEEE and ETSI standards for Vehicle to Vehicle and Vehicle to Infrastructure communication

- IETF specifications for Internet communications

- 3GPP and other mobile standards for communication

    - Communications standards gaps

- System integration standards (may be addressed in ETSI EP E2NA and ETSI TC ITS)

    - Comments

An example given by Tunisie Télécom on a current pilot study on using mobile communication to send information on injured people in accident - This is also may be considered as part of Priority Application 4 - Traveller Assistance Navigation.

  - Traffic Management & Control: Traffic Incident Management (TIM)

    - Description

TIM can be achieved through automatic accident detection and alerting combined with vehicle location identification. Once an accident is detected, using in-vehicle crash sensors, GPS and wireless communications, an alert with the accident location is sent automatically to the authorities as well as to the ambulance service.

    - Communications requirements: functional

- Communication infrastructure

- Deliverability of alerts

    - Communications requirements: performance

- Availability of service

- Instantaneous delivery of alerts

    - Security priority (1=lowest; 5=highest)

- 3

    - Communications requirements: security

- Authentication

- Authorization

- Encryption

    - Security threats and risks

- Masquerade of entity

- Manipulation of data

- Eavesdropping of data by unauthorized entity

    - Communications standards

- GSM

- UMTS

- LTE

- DSRC/WAVE

    - Communications standards gaps

    - Comments

The application can be considered as a hybrid of intelligent infrastructure and intelligent vehicle application. Up to now, such application does not exist in the Arab-world. Hence, the usage of an already existing infrastructure, such as cellular networks, would significantly facilitate the deployment.

  - Traveller Information &  Traffic Management & Operations (Transport Infrastructure maintenance management): Infrastructure signage reinforcement

    - Description

Each item of road furniture giving information in graphical form with or without accompanying text shall be developed to reinforce the visual format with authoritative indication by radio signaling of the signage. This shall be linked to centralized traffic management for authoritative road sign changes (e.g. variable speed limits).

    - Communications requirements: functional

- To be incorporated to existing infrastructure (e.g. using RDS TM channel)

- May use IEEE 802.11 at 5.9GHz (as defined in ETSI and ISO)

- Communications topology = directional broadcast (line of sight for signage location)

    - Communications requirements: performance

- Data to be transmitted such that when in radio range (say 250m) any approaching vehicle will receive the signal reinforcement even in the event of 80% message loss (note that transmitters can be placed above cars such as to maximize coverage and optimize communication path integrity)

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Authoritative signature and encryption at transmission

- Verification of signature at reception

- Audit log of receipt of signal

    - Security threats and risks

- Messages purporting to come from authority that are injected to the system leading to inappropriate driver behavior

    - Communications standards

- ETSI ITS and ISO CALM

    - Communications standards gaps

- TBD

    - Comments

As a means to get ITS in the field this can be introduced independently of vehicles and can be addressed by city planners and made a requirement for all road signs as part of planning arrangements. Once available this set of infrastructure ITS elements can be made bi-directional to allow the "Intelligent" part of ITS to operate.

  - Vehicle services: Traveller Assistance Navigation/ Assistive driver awareness

    - Description

Using infrastructure and vehicle mounted ITS-Stations to give intelligent transport system analysis to transport system users.

Allowing the traveler of making informed decision by providing local information such parking, restricted access, route, work zones, traffic status, etc.

For example this can be used for Road – Rail crossing warning applications [4] for enhancing safety of road users at signalized railway crossings: The system provides an in-vehicle visual and/or auditory alert to road users that an activated railway signal crossing is ahead. A signal can be transmitted from the crossing, which is detected by the vehicle, with other information such as the presence of a train.

Mode may be broadcast (advice to all irrespective of role and location), geocast (specific information within a particular geo-location), multicast (advice to specific groups of users), and unicast (advice to specific ITS-S). Provides "Intelligence" in ITS by collating DENM/CAM data from many sensors with high level view from system operator.

    - Communications requirements: functional

- All modes as above

- DSRC– collecting information from passing vehicles or informing passing vehicle about local conditions

    - Communications requirements: performance

- Timing close to real-time command and control

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Safety based security (i.e. using safe/critical environment design goals to maximize security)

    - Security threats and risks

- All CIA threats apply

    - Communications standards

- ETSI Local Dynamic Map;

- ETSI DENM;

- ETSI CAM, DSRC /WAVE

    - Communications standards gaps

- TBD

    - Comments

Extends existing co-operative ITS (vehicle to vehicle centric) to full "Intelligent" command and control system using ITS-S as part of prosumer (user as both consumer of data and producer of data) network.

  - Multimodal Travel Information:  Urban Multi-Modal Routing

    - Description

This is an essential element for effective use of all transport resources and should include all modes (e.g. bus, car, train, tram, boat, pedestrian, bicycle) with social network integration. The multi-modal work of project i-Tour (as an example) could form the basis of the modeling and algorithm development.

http://www.itourproject.com/web/

    - Communications requirements: functional

- Full modal using 3G (cellular data), WiFi and all other sensor networks

    - Communications requirements: performance

- Could be divided into two sections: data collection + data sharing via social network

  - The social network is to be accessed via mobile applications, i.e. internet, hence 3G cellular standard

  - Data collection part could be obtained via 1) public transport websites, who are already providing transit schedules or 2) via road sensors, i.e. in this case DSRC standard can be used

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Privacy protection across multiple application and mode invocations

- Full CIA protection

    - Security threats and risks

- Masquerade

- Data manipulation

- Interception

    - Communications standards

- None available for specific application

    - Communications standards gaps

- TBD

    - Comments

Effective use of transport resources can only be integrated to society if society is fully aware and able to make choices based on accurate, timely data that works with their social/societal use of transport. Smart transport is part of smart societies.

  - Vehicle Services: Intersection Collision Warning/Avoidance

    - Description

Intersection collision warning systems are mainly used in un-signalized intersections. The system uses roadside sensors to monitor vehicles approaching intersections and warn vehicles via alerting signals whether on digital road signs or to in-vehicle device.

    - Communications requirements: functional

- Supporting data transmission from sensors to in-vehicle devices (or to road digital signs)

- Ensuring coverage

- Availability

- Reliability

    - Communications requirements: performance

- Quality of data transmission (in terms of bit error rate)

- Latency

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- Authentication

- Authorization

- Encryption

    - Security threats and risks

- Risks are mainly summarized in digital hacking to the data transmission network

    - Communications standards

- DSRC / WAVE

    - Communications standards gaps

    - Comments

The application does not exist in the Arab-world, however we see it very important application that targets safety issue.

  - Transport related personal safety- Public travel security: CCTV surveillance

    - Description

Threat examples: Crowd control, theft, unauthorised entry, Back-up system:

Municipality of Makkah project:  Plan to install 500 to 2000 CCTV in the Makkah region and around "Haram" area to monitor traffic, pedestrian and visitor activities during Hajj and Omra.

Potential threat or activity to be monitored and/or recorded, Determination of the zones or objects requiring surveillance, Determination of the number of cameras, and their locations, Intended target(s) of the surveillance and the frame rate appropriate to the target’s speed within the location under surveillance, Response required on detection of an event, Manner in which images will be viewed and recorded, How data will be exported from the system to permanent record, Individuals who will require access to the recorded data, Selection of cameras and equipment depending on the operating environmental conditions, Control center configuration including secure location of control equipment

Event log-system history is held and retained in secure method, Recording settings and duration of retention of recorded images documented, Indication of video loss, tampering and failure can be properly conveyed to remote Center.

    - Communications requirements: functional

    - Communications requirements: performance

- High transmission rate

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

Project has not started – nothing to add.

  - Infotainment: content sharing applications and services (advertisements, entertainment)

    - Description

To help increase market penetration, a second class of applications may be developed that offer an immediate benefit to potential customers: the so-called deployment applications. These applications are taken from the domains of entertainment, information access, Internet applications, and increased driving comfort applications

There are a set of applications including content sharing, videoconferencing, life video streaming, gaming, etc. These on-the-road services can greatly increase drivers and/or passengers’ convenience, comfort and productivity. In fact, Business activities should be performed, as usual, in a vehicular environment, realizing the notion of mobile office.

Some concrete examples that we can envisage are: a real time parking navigation system, digital billboards for advertisement or peer-to-peer file transfer.

    - Communications requirements: functional

- Communication Mode: Internet access is needed to download data from Internet servers (V2I), but also peer to peer opportunistic networking among vehicles is needed to distribute the file after some vehicle has downloaded it (V2V).

- Point to Multipoint & Point to Point 2 Way interactive transmissions:

- Transmission type - Real Time & Storage: traffic conditions (traffic load) and user demands (infotainment category and satisfaction level) can vary in both spatial and temporal domains.

- Communication services: Connectionless

- To achieve an end to end connectivity from Vanet to Internet with respect to the applications requirements, several points must be investigated: Geo-addressing, location and discovery services, QoS Geo-routing, seamless connectivity.

    - Communications requirements: performance

- Low Latency

- Medium to High Rate (it depends on the application)

- Reliability : Medium to high reliability

- Priority: Low or high depending on the application

- Service differentiation is necessary by assigning different priorities to different traffic types. For example video service streaming is assigned a high priority or email is assigned a low priority. A fair channel access is needed to guarantee a fair share of resource assigned to each vehicle.

- Some applications are more critical and need eventually resource reservation.

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- Securing geo-routing protocols

- Securing location services (Privacy of location and motion patterns)

- Privacy preservation (name of driver/passenger)

    - Security threats and risks

- Modification attacks; relay attacks

- DDOS attacks

    - Communications standards

- Wave

- IEEE 802.11p

- EDCA

- IPv6

- C2C architecture

- Clustering

    - Communications standards gaps

- Adaptive resource reservation approaches to support different on-the-road applications

- Integration of infotainment applications and road safety applications

    - Comments

  - Passenger Information System

    - Description

Establishment of a complete solution of a Passenger Information System for the public land transport companies in application of Public Private Partnership (PPP) mode. This project consists to implement an integrated, multi-channel and multi-modal solution to provide customers dynamic information in real time. This information is mainly:

- At all stations: the display of waiting time and / or times of passage, destination of the line, etc...

- On board of means of transport (bus, train and metro): dynamic announcement (visual and audible) indicating: destination, next stop, etc…

    - Communications requirements: functional

- GPS and other geo localization devices

- Multimedia Display panels in stations and onboard of public transportations vehicles

- Connection between stations and central servers using the public network telecommunication

    - Communications requirements: performance

- Reliable communications equipment

- Reliable public network telecommunication system with a large coverage across the country

- High resolution GPS coverage across the country

- Quality and the availability of the service

- Real time communication

- Minimizing Risks, such as:

  - Devices risks (failure, deterioration, …)

  - Human risks (theft, deterioration, degradation, …)

  - Communication risks (lack of coverage, communication break, …)

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- Proof against data manipulation (i.e. integrity checks of data or system in transit and in storage)

- Secure data transmission to prevent interception by unauthorized parties

    - Security threats and risks

- Insertion of invalid data to the system

- Missing data

    - Communications standards

- Existing 2G, 3G, WLAN and PLAN standards

    - Communications standards gaps

- tbd

  - Air Traffic Management System and Data Processing

    - Description

Supply, installation and commissioning of a system of processing and visualization of surveillance data and flight control purposes en-route and approach control and aerodrome international airports of Tunis Carthage, TAV Monastir and TAV Ennfidha airports (ATMS / DP: Air Traffic Management System and Data Processing). Integration and processing of monitoring data from the former radar sites and those that will be installed under the project to extend radio coverage of the Tunisian airspace.

    - Communications requirements: functional

- The supply and installation of applications and software suitable for the operation of data processing and monitoring of flight (ATMS / DP)

- Flight Data surveillance system (servers, management stations flight data, radar control stations, printers, active and passive equipment for local area networks, receivers GPS equipment interfacing system ATMS / DP to remote sites, stations administration and supervision system, ...)

    - Communications requirements: performance

- Reliable communications equipment

- Reliable radar communication

- Reliable data transportation

- High level of QoS.

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Proof against data manipulation (i.e. integrity checks of data or system in transit and in storage)

- Secure data transmission to prevent interception by unauthorized parties

    - Security threats and risks

- Interoperability risks between different kind of devices

- Communications risks  such as lost connections

- Devices risks

- Human risks (lack of training, handling errors, …)

- Data transportation risks such as data loss

    - Communications standards

- tbd

    - Communications standards gaps

- tbd

    - Comments

  - Vessel Traffic System

    - Description

Supply and installation of VTS (Vessel Traffic System) port to ports of Bizerte, Sousse, Sfax, Gabes and Zarzis and the upgrade of the VTS system already installed at the port of La Goulette.

- Improving conditions of safety and security on the waterways ports ensuring the identification of vessels approaching the ports and landing on the docks

- Water monitoring, berths, docks and harbors ports of Bizerte, Sousse, Sfax, Gabes and Zarzis to ensure effective monitoring of maritime traffic and lighting in these areas

- Prevention of risk of collision and grounding of ships and the organization of relief vessels in distress

- Assistance by the use of navigation system as confirmation of the positions of ships, signaling the presence of obstacles in their vicinity

- Navigational guidance by indicating the course and speed to follow

- Upgrade the already installed  La Goulette port VTS to benefit from technological change

    - Communications requirements: functional

- Port radar

- Processing system imaging radars

- System for recording and archiving of VHF radio communications and telephones

- Station AIS (Automatic Identification System);

- Equipment and software necessary for the commissioning of VTS ports

- Equipment and software needed to upgrade the VTS system already operational at the port of La Goulette

- The civil works required for the commissioning of the VTS

- Development work necessary to theaters systems

- The integration of different components of the solution to activate the various VTS systems in question

    - Communications requirements: performance

- Reliable communications equipment

- Reliable radar communication

- Reliable data transportation

- High level of QoS

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Security of data transportation

    - Security threats and risks

- Communications risks

- Devices risks

- Human risks (lack of training, handling errors, …)

- Data transportation risks

- Interoperability risks between different kind of devices

    - Communications standards

- tbd

    - Communications standards gaps

- tbd

    - Comments

  - Meteorological Information System for Road and Rail Traffic

    - Description

Acquiring and implementing  a set of meteorological information displays (air temperature, relative humidity, wind force, etc..) For users, installed on roadsides, highways and roads iron. These displays will be connected to the system of the National Institute of Meteorology (INM) to exploit the range of products and services provided by it.

    - Communications requirements: functional

- A comprehensive study on various aspects of the project

- Acquisition of the displays and basic software needed

- Acquisition of network equipment

- Installation and testing

    - Communications requirements: performance

- Reliable communications equipment

- Reliable radar communication

- Reliable data transportation

- High level of QoS

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Security of the devices

- Security of data transportation

- Security of the communications

- Quality of service

    - Security threats and risks

- Communications risks

- Devices risks

- Human risks (lack of training, handling errors, …)

- Data transportation risks

- Interoperability risks between different kind of devices

    - Communications standards

- tbd

    - Communications standards gaps

- tbd

    - Comments

2. China (CCSA)

    - Contributors

- Kevin Li Jianyu, CCSA (Reported by China Unicom), lijianyu@chinaunicom.cn, China

  - Voice navigation

    - Description

The driver initiates a voice call to the call centre through the OBE (On Board Equipment) requiring the navigation of the route. With the help of the navigation platform, the operator calculates the best match route from the starting point (typically the current position of the vehicle) to the end point, and sends the result to the terminal.

    - Communications requirements: functional

- OBE:

  - 2G/3G/LTE single mode or a combination of modes;

  - Voice connection between the terminal and the call centre;

  - Simultaneous transmission for voice and data;

  - Processing and presenting of the received route navigation data;

- Call centre:

  - User identification

  - Service authentication

  - Processing and replying of the navigation request

- Navigation platform:

  - Locating the vehicle

  - Inquiring and calculating the best matched route

    - Communications requirements: performance

- Voice service:

  - QoS index of a basic call should be satisfied

- Data service:

  - IPTD (IP Packet Transfer Delay)

  - IPDV (IP Packet Delay Variation)

  - IPLR (IP Packet Loss Ratio)

  - IPER (IP Packet Error Ratio)

    - Security priority (1=lowest; 5=highest)

- 3

    - Communications requirements: security

- Identity security:

  - Mutual authentication between SIM card and the OBE

- Data security:

  - Encryption mechanism for the OBE and the navigation platform

  - Verification mechanism for received data integrity

  - Long term storage for data related to security management and voice navigation service

  - Protection of user profile related to user privacy

    - Security threats and risks

- Network congestion caused by simultaneous voice call access requirements

- Single service bottleneck caused by single point of failure from the call centre

- Disclosure of user privacy from the call centre

- Bad user experience of the navigation service due to the long processing delay

    - Communications standards

- Vehicle Telematic Service Requirement and General Framework (YDB 064-2011), CCSA TC10 WG2

    - Communications standards gaps

- Gaps in functional requirements:

  - Unclear definition of service logical unit from platform aspect

  - Some functions are not considered

- Gaps in performance requirements:

  - The value of QoS index of a basic call is not specified

  - Data service indicators are not specified

    - Comments

- Comments to the gaps in functional requirements:

  - To add a new unit of 'Service Management Platform', which is responsible for user management, terminal management, service management, the requiring data transmission of self-service voice navigation and navigation data distribution

  - Call centre is only responsible for voice call related handling process, and realize the differentiated services based on the user types and service attributes together with service management platform

  - Besides, to add voice forwarding function in the call centre

  - The navigation platform focuses on inquiry and calculation function related to user position and navigation information

  - To add voice identification module in the OBE, together with service management platform, the module could initiate coded voice data to search the route information in the navigation platform, and acquire the result from the service management platform, totally without the call centre involved

- Comments to the gaps in performance requirements:

  - Need for analyzing the indicators of voice and data service from the aspect of service continuity

  - Need for estimating the influence factor of those indicators in the scenario of heterogeneous network access

  - Operation data report of the vehicle

    - Description

The OBE (On Board Equipment) acquires the operation data of the vehicle through CAN-BUS and reports to the service management platform periodically. The service management platform forwards the data to its related vehicle manufacturer or vehicle maintenance organization, also informs the vehicle owner of the repair and maintenance requirements of his vehicle based on the feedback from the manufacturer or the organization.

    - Communications requirements: functional

- OBE:

  - To acquire the sensor data from the engine, the gearbox, the tire pressure and the airbag through LIN-BUS and CAN-BUS

  - Uploading the data to the service management platform periodically or as required

- Service management platform:

  - To Provide the access interface of uploaded vehicle data to the manufacturer or the organization; to support automatic and manual triggering modes for vehicle maintenance notification

  - To support customizable notification of vehicle maintenance, i.e. SMS for cell phone, email and SMS for the OBE.

    - Communications requirements: performance

- High reliability:

  - In order to protect the BUS communication, the OBE should be switched off and the link between the terminal and the BUS should be also cut off automatically when the terminal fails

  - Need for hardware CRC check to control the lowest data error rate

  - Need for hardware automatic retransmission function to ensure the transmission reliability

- Consistency:

  - The data acquired by the OBE should be adapted and converted into the format of the protocol, to ensure the data format consistency in the service management platform

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- Anti-electromagnetic interference:

  - The communication between the OBE and CAN-BUS should be interference-proof

- Identification security:

  - Authentication mechanism for the OBE

  - Mutual authentication between SIM card and the OBE

- Data security:

  - Need of privacy mechanism in data transmission process

  - The OBE could encrypt the data

  - The OBE should has verification mechanism to check whether the received data is modified during transmission process

    - Security threats and risks

- Errors in the data acquisition and uploading process due to the internal electromagnetic interference of the vehicle

- Data lost or recovery fault in the data uploading process due to the stall of the vehicle

    - Communications standards

- Vehicle Telematic Service Requirement and General Framework (YDB 064-2011), CCSA TC10 WG2

    - Communications standards gaps

- Communication reliability assurance mechanism is not perfect

- Indicators of performance are not specified the requirements of monitoring service are not considered

    - Comments

- To solve the problem of data lost or recovery fault in the data uploading process due to the stall of the vehicle or the interference factor, the repeat transmission function of the OBE is needed; the verification function of data rationality and integrity for the service management platform is also needed; the repeat transmission could be initiated either from the terminal or from the platform; functional requirements of reliability and consistency should be further elaborated, the related indicators should be specified;

- Due to the urgent need of monitoring service for the bus and the school bus, it is suggested to consider the requirement of the monitoring service. Two possible choices could be considered: Adding the function requirements of GPS information of the vehicle, static image inside the vehicle, real-time video acquisition inside or outside of the vehicle; or setting up a new use case for monitoring service.

3. China (ITSC)

    - Contributors

- Kevin Li Jianyu, ITSC (Reported by China Unicom), lijianyu@chinaunicom.cn, China

NOTE: The following applications are collected by China Unicom through its industry research and F2F interview with some ITSC experts. The materials provided in the following paper are for technical reference only as they are not ITSC officially authorized.

  - Dynamic monitoring for operating Vehicles

    - Description

All operating vehicles need to install satellite positioning device, to acquire and process the information and data including driver information, vehicle operation status, loading status, operating data, charging and settlement data, multimedia information (audio, image and video) according to the specific four categories of the operating vehicles (passenger vehicle, CDG (Carriage of Dangerous Goods), freight vehicle and taxi). So as to identify and control the influencing factors of road safety and to eventually achieve the scientific and secure development of the road transportation.

    - Communications requirements: functional

- OBE (On Board Equipment): The functions below are the collection of all the operation vehicles.

  - Communication mode:

    - To support 2G/3G and BeiDou satellite communication

    - Packet data transmission mode preferred, short message as the secondary mode, BeiDou as the backup mode in case of no telecommunication links, all the modes could be switched automatically between each other according to the priority and availability;

  - Data uploading:

    - Fault data in self-check process;

    - Positioning information of vehicle (in addition, the supplementary data storage and report mechanism should be considered when the communication link fails)

    - Driver's profile

    - Vehicle operation status

    - Loading status

    - Operating data

    - Charging and settlement data

    - Multimedia information (image, audio, video (Real time/non-real time))

    - Travelling data records(real time)

    - Audio monitoring data

    - Alert information (also should include the on-demand alert information sent to the mobile phone via short message)

    - Preemptive answer to the dial-a-cab service

    - Response of the vehicle scheduling instructions

  - Data downloading:

    - Remote update of terminal firmware

    - Scheduling information

    - Logistics information

    - Dial-a-cab order

    - Vehicle scheduling order

  - Phone call:

    - CS domain phone call, including call restriction

    - Audio storage

    - Address book management

    - Dial-back

    - Automatic hang-up (no requirement of simultaneous transmission for voice call and data uploading)

  - Sleep mode:

    - Sleep mode should be opened after vehicle stalling, with communication module kept on, the frequency of data uploading can be set remotely or be reduced automatically according to the predefined parameters

  - Multi-link support:

    - The terminal should support at least two remote connections, which are the connection to the main monitoring center and the connection to the backup center

    - The terminal could automatically connect to the backup center when the connection to the main center fails

  - Transmission restore at break-points:

    - The terminal supports this function, sends and receives the data in the batch mode

- Platform:

  - Dedicated or VPN access mode

  - Vehicle inquiry based on SIM card ICCID or IMSI

  - Audio monitoring

  - Monitoring release

  - Wireless communication connections

  - Image snapshot

  - Call of vehicle travelling data

  - Video monitoring

    - Communications requirements: performance

- OBE

  - Positioning capability: In the operating mode, the parameters of the position data reporting are the following:

    - Minimum interval: 5s

    - Maximum interval : 60s;

    - Minimum distance: 100m;

    - Maximum distance: 1000m;

    - Period between real time location request receiving and responding from a terminal to the platform: 10s

  - Image information: Minimum requirement of two camera channels, or one camera channel for the taxi, 320×240 (QVGA) or 352×288 (CIF) resolution, minimum sampling interval: 5s

  - Audio information: Minimum requirement of one audio input collection channel, coding accuracy: 16bit, coding format: ITU-T G.726 or ITU-T G.711

  - Video information: Simultaneous acquisition of minimum two camera channels (except the taxi), resolution: 352×288 (CIF), minimum frame rate: 1FPS, coding format of video acquisition: ITU-T H.264

  - GNSS module: Minimum number of reception channels: 12, sensitivity: above -130dBm; horizontal accuracy: no bigger than 15m, height accuracy: no bigger than 30m, speed accuracy: no bigger than 2m/s, differential positioning accuracy (optional): 1m-5m, minimum location update frequency: 1Hz,  capture time after warm start: no more than 10s

  - Well functioned during the ignition interference test; time interval of dynamic vehicle data reporting in non sleep mode: no more than 60s

- Platform:

  - Average response time of every transaction should be no more than five times of the average response time of single user

  - Processing time of emergency and alerting information: no more than 10min

  - Preferential displaying of alerting information or  alarm processing information

  - Terminal access capability: at least 1000 terminal supported

  - Monitoring capability: over 1000 dynamic targets supported

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- OBE: To support the following functions bearing by wireless network:

  - Remote registering and logout

  - Firmware update

  - Modification and inquiry of firmware parameters

  - Terminal reset or restore the factory settings

- Platform:

  - To meet the safety requirements level 3 or higher of GB 17859

  - Encrypted storage of critical data and user password; reception and transmission data

  - Log recording: minimum of 183 days

  - Backup platform supports automatic switching when the main platform fails

  - Encrypted transmission for data exchange between platforms

    - Security threats and risks

- Certification and authentication:

  - Certification mechanism between the driver and the OBE to guarantee the legal usage of the car by the certified driver

  - Mutual authentication between SIM card and the OBE

- Service bearing:

  - Service application experience varies due to the different network bearing according to the different services

  - Potential risks in service continuity assurance during the handover process between heterogeneous networks

- Potential impacts to continuity assurance service (i.e. vehicle location uploading service) caused by voice call service due to the no guarantee of simultaneous transmission for voice call and data uploading

    - Communications standards

- GNSS system for operating vehicles – Technical specification for vehicle terminals (JT/T794-2011)

- GNSS for operating vehicles – Technical specification for platform (JT/T796-2011)

- Classified criteria for security (GB17859-1999)

    - Communications standards gaps

- Gaps in functional requirements:

  - Need to identify the specific services of dynamic monitoring services, to determine the priority assurance mechanism for the parallel ongoing services

  - Unclear logical relationship between each functional entities of the platform

- Gaps in performance requirements:

  - The number of video acquisition channels is relatively low, which would influence some special service requirements in future

    - Comments

- Comments to the gaps in functional requirements:

  - To refine service features and requirements according to the different types of operating vehicles

  - To Identify the priority of different services and the scenario of multi-service concurrency

  - Refining the platform architecture and the relationship between each entities of the platform so as to elaborate the communication requirement

- Comments to the gaps in performance requirements:

  - To refine service requirements according to the different types of operating vehicles so as to elaborate the performance requirement

  - ETC (Electronic Toll Collection)

    - Description

ETC system consists of the vehicle mounted OBE (On Board Equipment) and RSE (Roadside Equipment) installed in the toll lane. The non-stop toll collection is realized by the data exchange by the communication links between OBE and RSE. The user acquires the ETC service through the agreed method of payment. The statistics, settlement and management are based on the transaction data list.

    - Communications requirements: functional

- In order to timely acquire inherent attribute of the vehicle (vehicle type, owner, plate etc.), operation information of the road and toll collection information, the dedicated short range communication (DSRC) method is used between RSU and OBE

- The RSU could identify and classify the vehicle automatically, then deduct the tolls from the ETC card according to the accounting standards

- When there is a need for evacuation in the important regional event, the traffic management sub system could send the toll change request to reduce or cancel the toll temporarily which will facilitate the fast evacuation of the road

- OBE and RSU are also used in road administration department for data collection purpose of the road usage

    - Communications requirements: performance

- ETC belongs to real-time interactive service with the performance requirements of

  - IPTD (IP Packet Transfer Delay)

  - IPLR (IP Packet Loss Ratio)

  - IPER (IP Packet Error Ratio)

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Data transmission security between each entities: To ensure that the data during transmission is not being monitored, tampered, replaced and damaged, including:

  - Authentication: To make acceptance authentication for received data, to prove the validity of the identity and data source, the certification methods includes: MAC verification, PKC certification and C/R authentication

  - Privacy control: To avoid illegal monitoring of sensitive information by means of information encryption, encryption methods includes: DEA, FEAL, RAS etc.

  - Data integrity control: CRC check; MAC verification

    - Security threats and risks

- Untimely reading and processing of the information due to the over-speed of the vehicle

- Errors in user data or toll transaction data due to the trustless wireless communication link

    - Communications standards

- General Architecture of Intelligent Transportation System Based on Telecommunication Networks (Draft 1.0), CCSA TC10 WG2.

- Intelligent transport system - Electronic toll collection - System architecture (GB 20135-2006-T)

- Electronic toll collection - Dedicated short range communication - Part 1: Physical layer (GBT 20851.1-2007)

- Electronic toll collection - Dedicated short range communication - Part 2: Data link layer (GBT 20851.2-2007)

- Electronic toll collection - Dedicated short range communication - Part 3: Application layer (GBT 20851.3-2007)

- Electronic toll collection - Dedicated short range communication - Part 4: Equipment application (GBT 20851.4-2007)

    - Communications standards gaps

- It is mentioned that the road usage data could be collected by ETC service, but the technical solutions, detailed performance requirements, service flow and data format are not specified.

    - Comments

- To consider extending the usage scenarios of ETC to AVI (Automatic Vehicle Identification), vehicle access control, urban road charging, road usage data collection etc.

4. Europe (ETSI)

    - Contributors

- Martin Arndt, ETSI, martin.arndt@etsi.org

Source: ETSI TR 102 638 V1.1.1 (2009-06): Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Definitions

Co-operative ITS – General characteristics and requirements

    - Description

General characteristics and requirements for co-operative ITS applications/use cases, based on ETSI TR 102 638 V1.1.1 (2009-06) Annex B3 and B4.

The availability of system technical capabilities will condition the selection of the co-operative ITS applications/use cases belonging to the basic set.

    - Communications requirements: functional

- Radio communication capabilities:

  - The radio communication single hop coverage (distance, area)

  - The radio communication frequency channels being used (for interoperability purpose).

  - The available bandwidth and bit transfer rate

  - The radio communication channel robustness which can be characterized by its packet error rate

  - The contribution of the road infrastructure to compensate for signal propagation difficulties in areas populated by radio obstacles (trees, buildings, etc.)

- Network communication capabilities:

  - Broadcasting capabilities

  - Multicasting capabilities

  - Ad-hoc networking capabilities

  - Geocasting capabilities

  - Unicasting capabilities

  - Congestion control capabilities

  - Messages priorities management capabilities

  - Channels/connectivity management capabilities

  - Capability to gain at least one IPv6 globally valid address for Internet - based applications

  - Transparent management of changes in the point-of-attachment to the Internet for all the involved in-vehicle devices

    - Communications requirements: performance

- Vehicle communication performances (maximum latency time, maximum round trip delays, etc.).

- Vehicle communication system reliability and dependability performances (radio coverage, BER, black zones, etc.).

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

- Respect of life privacy

- Confidentiality

- Resistance to external attacks

- Authenticity of received data

- Data and system integrity, etc.

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Co-operative road safety applications: Driving assistance – co-operative awareness

    - Description

The primary objective of applications in the active road safety class is the improvement of road safety. However, it is recognized that in improving road safety they may offer secondary benefits which are not directly associated with road safety.

This application includes the following use cases:

- Emergency vehicle warning

- Slow vehicle indication

- Intersection collision warning

- Motorcycle approaching indication

    - Communications requirements: functional

- Emergency vehicle warning:

  - Capability for an emergency vehicle to broadcast V2X co-operative awareness messages with relevant emergency signs being activated

  - Capability for all vehicles and relevant road side units to receive and process V2X co-operative awareness messages

- Slow vehicle indication:

  - Capability for a slow vehicle to broadcast V2X co-operative awareness messages

  - Capability for any vehicle to receive and process V2X co-operative awareness messages containing a “slow vehicle” type

- Intersection collision warning:

  - Capability for vehicles to broadcast V2X co-operative awareness messages and to receive and process V2X co-operative awareness messages

  - Road side unit to be installed if line of sight between vehicles is obstructed. RSU needs to be capable to relay signal or to detect and signal a collision risk

  - Accurate positioning of vehicles on digital maps

- Motorcycle approaching indication:

  - Capability for motorcycle to broadcast V2X co-operative awareness messages indicating its motorcycle status

  - Road side unit to be installed if line of sight between vehicles is obstructed. RSU must be capable to relay signal or to detect and signal a collision risk

  - Capability for any vehicle to receive and process V2X co-operative awareness messages and provide warnings to drivers

    - Communications requirements: performance

- Emergency vehicle warning:

  - Minimum frequency of V2X co-operative awareness messages issued by the emergency vehicle: 10 Hz

  - Critical time requirement: Latency time less than 100 ms

- Slow vehicle indication:

  - Minimum frequency of the periodic message: 2 Hz

  - Critical time (latency time less than 100 ms)

- Intersection collision warning:

  - Minimum frequency of the periodic message: 10 Hz

  - Critical time (latency time less than 100 ms)

- Motorcycle approaching indication:

  - Maximum latency time: 100 ms

  - Minimum frequency of V2V co-operation awareness messages: 2 Hz

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

- Emergency vehicle warning:

  - Protection and authentication of the CAM message

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Co-operative road safety applications: Driving assistance – Road hazard warning

    - Description

The primary objective of applications in the active road safety class is the improvement of road safety. However, it is recognized that in improving road safety they may offer secondary benefits which are not directly associated with road safety.

This application includes the following use cases:

- Emergency electronic brake lights

- Wrong way driving warning

- Stationary vehicle

  - Accident

  - Vehicle problem

- Traffic condition warning

- Signal violation warning

- Roadwork warning

- Collision risk warning

- Decentralized floating car data –

  - Hazardous location

  - Precipitations

  - Road adhesion

  - Visibility

  - Wind

    - Communications requirements: functional

- Emergency electronic brake lights:

  - Capability for a vehicle, from the emergency electronic brake lights activation, to broadcast in V2X decentralized environmental notification messages

  - Capability for concerned vehicles to receive and process V2X decentralized environmental notification messages

- Wrong way driving warning:

  - Capability for a vehicle, to detect that it is driven in a wrong way and to broadcast in V2X decentralized environmental notification messages its current “wrong way heading” status

  - Capability for concerned vehicles to receive and process V2X decentralized environmental notification messages

- Stationary vehicle warning:

  - Capability for a vehicle, from detecting a dangerous immobilization, to broadcast in V2X decentralized environmental notification messages its current immobilization situation

  - Capability for concerned vehicles (on the same road, and same heading of the immobilized vehicle) to receive and process the V2X decentralized environmental notification messages

- Traffic condition warning:

  - Capability for a vehicle, from detecting a traffic jam, to broadcast/geocast in V2X decentralized environmental notification messages the traffic jam notification

  - Capability for concerned vehicles to receive and process the V2X decentralized environmental notification messages

  - Capabilities for all vehicles crossing the car signalling a traffic jam to store and forward received V2X decentralized environmental notification messages according to their geocasting parameters

- Signal violation warning:

  - Capability for a road side unit to broadcast (upon reception of a V2X co-operative awareness message) V2X decentralized environmental notification messages indicating signal violation to all surrounding vehicles

  - Capability for a concerned vehicle to receive and process V2X decentralized environmental notification messages and provide warning to drivers according to the collision risk level

- Roadwork warning:

  - Capability for a road side unit to broadcast I2V decentralized environmental notification messages providing the status of local roadwork

  - Capabilities for vehicle to receive and process I2V decentralized environmental notification messages and inform drivers accordingly

  - Capabilities for concerned vehicles to store and forward according to geocasting messages cancel rules, I2V decentralized environmental notification messages

- Decentralized floating car data:

  - Capability for a vehicle, from detecting a local danger or event (fog, heavy rain, slippery road heavy wind, traffic jam, etc.), to broadcast/geocast in V2X decentralized environmental notification messages the local danger notification

  - Capability for concerned vehicles (on the same road, and same heading of the cars having detected a traffic jam) to receive and process the V2X decentralized environmental notification messages

  - Capabilities for all vehicles crossing the car signalling a traffic jam to store and forward received decentralized environmental notification messages according to their geocasting parameters

    - Communications requirements: performance

- Emergency electronic brake lights:

  - Minimum frequency of the periodic message: 10 Hz

  - Critical time (latency time less than 100 ms)

- Wrong way driving warning:

  - Minimum frequency of the periodic message: 10 Hz

  - Critical time (latency time less than 100 ms)

- Stationary vehicle warning:

  - Minimum frequency of the periodic message: 10 Hz

  - Critical time (latency time less than 100 ms)

- Traffic condition warning:

  - Minimum frequency of the periodic message: 1 Hz

- Signal violation warning:

  - Minimum frequency of C2X decentralized environmental notification messages: 10 Hz

  - Maximum latency time: 100 ms

- Roadwork warning:

  - Minimum frequency of the periodic message: 2 Hz

  - Maximum latency time: 100 ms

- Decentralized floating car data:

  - Minimum frequency of the periodic message: 1 Hz to 10 Hz according to considered event

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Co-operative traffic efficiency: Speed management

    - Description

The primary objective of applications in the traffic management class is the improvement of traffic fluidity. However it is recognized that in improving traffic management they may offer secondary benefits not directly associated with traffic management.

This application includes the following use cases:

- Regulatory / contextual speed limits notification

- Traffic light optimal speed advisory

    - Communications requirements: functional

- Regulatory / contextual speed limits notification:

  - Capability for a road side unit to gather regulatory/contextual speed limits and to broadcast them periodically in I2V co-operative awareness messages

  - Capability for a vehicle to receive and process I2V co-operative awareness messages containing regulatory/contextual speed limits

- Traffic light optimal speed advisory:

  - Capability for a road side unit (traffic light) to broadcast periodically the timing data associated its current state (using I2V co-operative awareness messages)

  - Detailed topological and geometrical information about the intersection

  - Capability for compliant vehicles to receive broadcasted I2V co-operative awareness messages and processes them

  - Minimum frequency of the periodic message: 2 Hz

  - Minimum duration of the Reception: according to the vehicle speed and transmission range

  - Maximum latency time: 100 ms

  - Minimum positioning accuracy: better than 5 m

    - Communications requirements: performance

- Regulatory / contextual speed limits notification:

  - Minimum frequency of the periodic message: 1 Hz to 10 Hz according to used broadcasting technology

- Traffic light optimal speed advisory:

  - Minimum frequency of the periodic message: 2 Hz

  - Minimum duration of the Reception: according to the vehicle speed and transmission range

  - Maximum latency time: 100 ms

  - Minimum positioning accuracy: better than 5 m

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Co-operative traffic efficiency: Co-operative navigation

    - Description

The primary objective of applications in the traffic management class is the improvement of traffic fluidity. However it is recognized that in improving traffic management they may offer secondary benefits not directly associated with traffic management.

This application includes the following use cases:

- Traffic information and recommended itinerary

- Enhanced route guidance and navigation

- Limited access warning and detour notification

- In-vehicle signage

    - Communications requirements: functional

- Traffic information and recommended itinerary:

  - Capability for a road side unit to broadcast periodically some local traffic information and provide circulation advices to reduce traffic jams

  - Capability for a vehicle to received and process broadcasted traffic information and establish an P2P unicast session in case of local map download

- Enhanced route guidance and navigation:

  - Internet access: IPv6 is required, the geonetworking protocol combined with IPv6 extends the range of the RSU

  - RSU broadcasts the capability to offer Internet access and the known navigation support servers (e.g. in CAM messages). The RSU can act as an IPv6 router or a network bridge

  - Vehicles configure an IPv6 address, connect to predefined or advertised servers and establish a session. By means of the geonetworking protocol, extend the communication range of the RSU

- Limited access warning and detour notification:

  - Capability for a road side unit to broadcast periodically messages providing road restricted access information and associated to restriction conditions, see providing detour advices

  - Capability for vehicles to received and process messages and inform driver of the conditions to access or not to the limited access road

- In-vehicle signage:

  - Capability for a road side unit to broadcast periodically messages providing the traffic sign type and status

  - Capability for vehicles to received and process messages and inform driver of the decoded traffic sign

    - Communications requirements: performance

- Traffic information and recommended itinerary:

  - Minimum frequency of the periodic message: 1 Hz to 10 Hz according to used broadcasting technology

  - Maximum latency time: 500 ms

- Enhanced route guidance and navigation:

  - I2V Network CAMs required minimum frequency: 1 Hz CAM can be aggregated with IPv6 standard signalling for address configuration (Router Advertisement)

  - I2V maximum latency time: 500 ms

- Limited access warning and detour notification:

  - Minimum frequency of the periodic message: 1 Hz to 10 Hz according to used broadcasting technology

  - Maximum latency time: 500 ms

- In-vehicle signage:

  - Minimum frequency of the periodic message: 1 Hz

  - Maximum latency time: 500 ms

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Co-operative local services: Location based services

    - Description

Applications in the co-operative local services and global internet services classes advertise and provide on-demand information to passing vehicles on either a commercial or non-commercial basis. These services may include Infotainment, comfort and vehicle or service life cycle management. Co-operative local services are provided from within the ITS network infrastructure.

This application includes the following use cases:

- Point of Interest notification

- Automatic access control and parking management

- ITS local electronic commerce

- Media downloading

    - Communications requirements: functional

- Point of Interest notification:

  - Capability for a road side unit to broadcast I2V co-operative awareness messages to announce some locally based services/points of Interest information. Capability for the road side unit to establish a P2P session with any requesting vehicle.

  - Capability for vehicles to received and process broadcasted I2V co-operative awareness messages including point of interest notification and establish with the road side unit a P2P session to get detailed POIs information

- Automatic access control and parking management:

  - Capability for a road side unit to broadcast periodically I2V co-operative awareness messages signalling its access control capabilities and its active state. Capability for the Road Side Unit to establish a P2P unicast session with a requesting vehicle

  - Capability for concerned vehicles to receive broadcasted I2V co-operative awareness messages, analyze them and if establish a P2P unicast session with the Road Side Unit to get access in the controlled area

- ITS local electronic commerce:

  - Capability for a road side unit to broadcast I2V co-operative awareness messages informing on its electronic commerce capabilities and active state. Capability for this RSU to establish a P2P unicast session with any requesting vehicle to process fully the electronic commerce transactions under its responsibility

  - Capability for a concerned vehicle to receive I2V co-operative awareness messages process them and establish an P2P unicast session with the road side unit to achieve completely an electronic commerce transaction

- Media downloading:

  - Internet access: IPv6 is required. For broadcast - based services, link - local addressing is enough. For unicast - based services IPv6 globally valid addresses need to be provided. If using ITS 5,9 GHz, the geonetworking protocol combined with IPv6 extends the range of the RSU

  - RSU: broadcast the capability to offer Internet access and the known application servers (e.g. In CAM messages). The RSU can act as an IPv6 router or a network bridge. Specific applications can run in the RSU or in application servers. These applications need to be able to inform vehicles about media downloading/purchasing capabilities. Purchasing and delivery need to be secured

  - Vehicle: Configure an IPv6 address, connect to predefined or announced servers and establish an unicast session. Complete the media purchasing/downloading transaction

    - Communications requirements: performance

- Point of Interest notification:

  - Minimum frequency of the periodic message: 1 Hz

  - Maximum latency time: 500 ms

- Automatic access control and parking management:

  - Minimum frequency of the periodic message: 1 Hz

  - Maximum latency time: 500 ms

- ITS local electronic commerce:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

- Media downloading:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

- ITS local electronic commerce:

  - Electronic commerce required security level

- Media downloading:

  - DRM required security level

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Global internet services: Communities services

    - Description

Applications in the co-operative local services and global internet services classes advertise and provide on-demand information to passing vehicles on either a commercial or non-commercial basis. These services may include Infotainment, comfort and vehicle or service life cycle management. Global internet services are acquired from providers in the wider internet.

This application includes the following use cases:

- Insurance and financial services

- Fleet management

- Loading zone management

    - Communications requirements: functional

- Insurance and financial services:

  - Internet access: IPv6 is required. For broadcast - based services, link - local addressing is enough. For unicast - based services IPv6 global valid addresses need to be provided. If using ITS 5,9 GHz, the geonetworking protocol combined with IPv6 extends the range of the RSU.

  - RSU: Broadcast the capability to offer the service and known application servers (e.g. in CAMs), inform on its capabilities to achieve some insurance and financial services. The RSU can act as an IPv6 router or a network bridge. Specific applications can run in the RSU or in application servers. These applications need to be able to establish a P2P session with any requesting vehicle to process fully the relevant insurance and financial services

  - Capability for concerned vehicles to receive I2V co-operative awareness messages process them, configure an IPv6 address, connect to predefined or announced servers and establish an unicast session to achieve completely the relevant insurance and financial services

- Fleet management:

  - Internet access: IPv6 is required. For broadcasting - based services, link - local addressing is enough. For unicast - based services IPv6 globally valid addresses need to be provided. If using the ITS 5,9 GHz, the geonetworking protocol combined with IPv6 extends the range of the RSU

  - RSU: Broadcast the capability to offer the service and known application servers (e.g. in CAMs), inform on its capabilities to achieve some fleet management services. The RSU can act as an IPv6 router or a network bridge. Specific applications can run in the RSU or in application servers. These applications need to be able to process fully the fleet management services when requested

  - Capability for concerned vehicles to receive I2V co-operative awareness messages process them, configure an IPv6 address, connect to predefined or announced servers and establish an unicast session to achieve completely the relevant fleet management services if necessary

- Loading zone management:

  - Capability for a road side unit to broadcast I2V Co-operative Awareness Messages informing on its capabilities to liaise with parking booking centre. Capability for this RSU to establish a P2P unicast session with any requesting vehicle to fully process the parking booking service

  - Capability for concerned vehicles to receive I2V Co-operative Awareness messages process them and establish any requested P2P unicast sessions with the local road side unit to fully process the parking booking service

    - Communications requirements: performance

- Insurance and financial services:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

- Fleet management:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

- Loading zone management:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

  - Global internet services: ITS station life cycle management

    - Description

Applications in the co-operative local services and global internet services classes advertise and provide on-demand information to passing vehicles on either a commercial or non-commercial basis. These services may include Infotainment, comfort and vehicle or service life cycle management. Global internet services are acquired from providers in the wider internet.

This application includes the following use cases:

- Vehicle software / data provisioning and update

- Vehicle and RSU data calibration

    - Communications requirements: functional

- Vehicle software / data provisioning and update:

  - Internet access: IPv6 is required. For broadcast - based services, link - local addressing is enough. For unicast - based services IPv6 globally valid addresses need to be provided. If using ITS 5,9 GHz, the geonetworking protocol combined with IPv6 extends the range of the RSU.

  - RSU: Broadcast the capability to offer the service and known application servers (e.g. in CAMs), inform on its capabilities to achieve some software/parameters provisioning services. The RSU can act as an IPv6 router or a network bridge. Specific applications can run in the RSU or in application servers. These applications need to be able to process fully the software/parameters provisioning services when requested

  - Capability for concerned vehicles to receive I2V co-operative awareness messages process them, configure an IPv6 address, connect to predefined or announced servers and establish an unicast session to achieve completely the relevant software/parameters provisioning services if necessary

- Vehicle and RSU data calibration:

  - Internet access: Any network having the required throughput capabilities. If using WAVE: SCH2 using P2P unicast and IPv6 protocol. Internet access using IPv6 protocol

  - Capability of a road side unit to broadcast I2V Co-operative Awareness messages

  - Capability for a concerned vehicle to receive I2V Co-operative Awareness messages process them and achieve completely the internet exchange with a selected internet navigation support server

    - Communications requirements: performance

- Vehicle software / data provisioning and update:

  - I2V minimum CAMs frequency: 1 Hz

  - Maximum latency time: 500 ms

- Vehicle and RSU data calibration:

  - Maximum latency time: 500 ms

  - Minimum frequency of V2V Co-operation Awareness messages: 1 Hz

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

- Vehicle software / data provisioning and update:

  - Specific security procedures to protect against mistakes or malicious software/parameters downloading

    - Security threats and risks

    - Communications standards

    - Communications standards gaps

    - Comments

5. Japan (TTC)

    - Contributors

- Yasubumi Chimura, TTC, s-car-chair@ttc.or.jp, Japan

  - Utilization of In-Vehicle Total Information & Communication Environment

    - Description

This application provides an information communication environment for travelers just like in a home space (or in an office space) in the vehicle.

The general information network (LAN) is built in the vehicle, and this information network is connected with the outside communication networks through the radio communication media.

This in-vehicle information network is connected with vehicle CAN bus through gateway (firewall), and exchanges the ITS information between outside communication networks.

    - Communications requirements: functional

- The connecting functions of the in-vehicle information network (LAN) and the outside communication networks

- The connecting function of the in-vehicle information network (LAN) and vehicle CAN bus

- The connecting functions of the in-vehicle information network (LAN) and information terminals (nomadic and mobile devices)

    - Communications requirements: performance

- For the communication of ITS information of the safety driving assistance functions, low latency and high quality transmission is required

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- The exclusion of the influence over the driving control function of vehicle by the computer virus etc.

    - Security threats and risks

    - Communications standards

- In-vehicle information network architecture;

- The interface specifications between mobile communication equipments (cellular phone, DSRC etc.) and the in-vehicle information network (LAN)

- The interface specifications of gateway between the in-vehicle information network (LAN) and vehicle CAN bus

- The interface specifications between in-vehicle information network (LAN) and information terminals (nomadic and mobile devices)

- The firewall function of the gateway between the in-vehicle information network (LAN) and vehicle CAN bus

- The communication address assignment method of the equipment connected with the in-vehicle information network (LAN)

    - Communications standards gaps

- ISO/TC204/WG17 is proceeding with the standardization work item of ISO/PWI 13185 “ITS/Telematics applications supported by nomadic and mobile devices and in vehicle gateway operate on a common software interface”. The scope of ISO/PWI 1318 is the one part of the in-Vehicle total Information & communication network. This application titled “Utilization of In-Vehicle Total Information & Communication Environment” is to provide an information communication environment such as at a home space (or at an office space) to travelers in the vehicle.

    - Comments

- Comments to the gaps in functional requirements:

  - To refine service features and requirements according to the different types of operating vehicles

  - To Identify the priority of different services and the scenario of multi-service concurrency

  - Refining the platform architecture and the relationship between each entities of the platform so as to elaborate the communication requirement

- Comments to the gaps in performance requirements:

  - To refine service requirements according to the different types of operating vehicles so as to elaborate the performance requirement

  - Ad-hoc network between vehicle to vehicle communications for disaster response and resilience

    - Description

In a disaster situation, the ad-hoc network consisting of vehicles with the communication capability provides appropriate emergency information such an order, route and area for evacuation.

    - Communications requirements: functional

- Routing Protocol between vehicle and vehicle

- Quality Control for some applications (ex. Emergency Call)

    - Communications requirements: performance

- TBD

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Encryption

- Virtual Private Network

    - Security threats and risks

- Tap

- DoS attack

    - Communications standards

- ISO TC204/CALM

- IEEE 802.11p

- WAVE

- DSRC

- ITS-F/IVC

- C2CCC

- ETSI TG37

    - Communications standards gaps

- New items

    - Comments

  - Disaster information

    - Description

During disaster, public authority transmits disaster and related information to public. The information distribution will be uni-direction from public agencies to public.

    - Communications requirements: functional

- Wide area transmission

- Both one way broadcast and two way communication will be used

    - Communications requirements: performance

- Robust communication

- Fast transmitting will be needed. (Ex. Tsunami or earthquake warning)

    - Security priority (1=lowest; 5=highest)

- 4

    - Communications requirements: security

- Counter measure for masquerade will be needed

    - Security threats and risks

- Masquerade

    - Communications standards

    - Communications standards gaps

    - Comments

  - Alive information (“I am alive” message)

    - Description

After disaster, people transmit their vital message including personal information, position and so on.

    - Communications requirements: functional

- Wide area transmission

- Both one way broadcast and two way communication will be used

    - Communications requirements: performance

- Robust communication

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Counter measure for masquerade and communication interruption will be needed

    - Security threats and risks

- Masquerade

- Interruption

    - Communications standards

    - Communications standards gaps

    - Comments

  - Person finder

    - Description

Searching the person with his/her name, address and physical features

    - Communications requirements: functional

- Two way communication will be used

    - Communications requirements: performance

- Robust communication however the high capacity will not be needed

    - Security priority (1=lowest; 5=highest)

- 5

    - Communications requirements: security

- Counter measure for masquerade and communication interruption will be needed

    - Security threats and risks

- Masquerade

- Interruption

    - Communications standards

    - Communications standards gaps

    - Comments

  - Evacuation route information

    - Description

After disaster, public authorities provide route information to refugee with safety environment.

    - Communications requirements: functional

- Two way communication will be used

    - Communications requirements: performance

- Robust communication however the high capacity will not be needed

    - Security priority (1=lowest; 5=highest)

- 2

    - Communications requirements: security

- General security will be needed

    - Security threats and risks

- General threats

    - Communications standards

    - Communications standards gaps

    - Comments

  - Refugee information

    - Description

Each refugee provides the current condition, logistics and relief supplies information.

    - Communications requirements: functional

- Both one-way broadcast and two way communication will be used

    - Communications requirements: performance

- High capacity communication will be needed for rich content

    - Security priority (1=lowest; 5=highest)

- 2

    - Communications requirements: security

- General security will be needed

    - Security threats and risks

- General threats

    - Communications standards

    - Communications standards gaps

    - Comments

  - Management of Electric Vehicles and Electric Vehicle charging spots

    - Description

To achieve efficiency in the electric power management and better experience of EV users, the following applications are assumed based on communication networks

- Notification of EV charging spots to EVs

- Authentication, billing and charging, and

- Management of EV charging spot

    - Communications requirements: functional

- Application interfaces to manage EV charging spots

- Notification of and/or navigation towards EV charging spot(s)

- Secure communications for user authentication

    - Communications requirements: performance

- TBD

    - Security priority (1=lowest; 5=highest)

- 3

    - Communications requirements: security

- Encryption

- Virtual Private Network

    - Security threats and risks

- Tap

- DoS attack

    - Communications standards

- ISO TC204/CALM

- IEEE 802.11p

- WAVE/DSRC

- 3GPP

- 3GPP2

- ETSI TC ITS

    - Communications standards gaps

- New items

    - Comments

6. United States of America (Department of Transportation)

    - Contributors

- Steve Sill, U.S. DoT

- Dick Schnacke, TransCore, ISO TC 204

Sources:

- Connected Vehicle Safety Pilot Program, http://www.its.dot.gov/research/safety_pilot_overview.htm

- National Highway Transportation Safety Administration (NHTSA), Vehicle Safety Communications – Applications (VSC-A), Final Report and Appendix 1-3, available at http://www.nhtsa.gov/Research/Crash%2BAvoidance/Office%2Bof%2BCrash%2BAvoidance%2BResearch%2BTechnical%2BPublications

Vehicle-to-Vehicle (V2V) Communications for Safety is the wireless exchange of data among vehicles traveling in the same vicinity which offers opportunities for significant safety improvements.

V2V is a key research program of the Intelligent Transportation Systems Joint Program Office (ITS JPO) within the U.S. Department of Transportation’s (U.S. DOT) Research and Innovative Technology Administration (RITA).

Through the multimodal program, the ITS JPO and the private sector are able to share information between vehicles to achieve transformative safety benefits to the multimodal transportation sector.

The vision for the U.S. DOT’s V2V Communications for Safety research program is that each vehicle on the roadway will eventually be able to communicate with other vehicles, and that this rich set of data and communications will support a new generation of active safety applications and systems.

Data and communication standards have been developed through this research effort including the SAE J2735 Basic Safety Message; a standard communications architecture/ platform communicating in the 5.9 GHz band of radio spectrum.

  - Forward-collision warning

    - Description

The FCW safety application issues a warning to the driver of the host vehicle (HV) in case of an impending rear-end collision with a vehicle ahead in traffic in the same lane and direction of travel.

    - Communications requirements: functional

The minimum operational speed range for FCW shall be 11.4 – 30 meters per second. The FCW zone is defined as: same direction traffic, ahead (same lane). The longitudinal detection distance is no greater than 300 meters from the front bumper of the HV.

The lateral detection distance is one-half of a lane width about each side of the HV’s center. The FCW zone shall follow the curvature of the HV’s forward road geometry in both lateral and longitudinal directions.

The remote vehicle(s) (RV(s)) shall periodically send out status as part of the over-the-air (OTA) messages. The system shall support periodic broadcast updates with a minimum update rate of 100 ms.

    - Communications requirements: performance

NLOS: FCW is intended to function even when the view to the threat vehicle(s) is blocked by other vehicles, inclement weather, or terrain conditions.

LOS: FCW shall function with at least one obstructing (equipped or unequipped) vehicle between the host and the remote threat vehicle. The dimensions of an obstructing vehicle that satisfies this minimum performance requirement shall be no greater than 7.3 meters long, 2.2 meters high, and 2.4 meters wide.

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See C-1-5 of Appendix 1, NHTSA Final Report

  - Emergency Brake-Light Warning / Emergency Electronic Brake Lights

    - Description

The EEBL safety application enables a HV to broadcast a self-generated emergency brake event to surrounding remote vehicles (RVs). Upon receiving such event information, the RV determines the relevance of the event and provides a warning or an advisory to the driver, if appropriate.

    - Communications requirements: functional

The minimum operational speed range for EEBL shall be 11.4 – 30 meters per second (25-67 mph). The RV shall send out a hard brake status as part of the OTA messages as soon as hard braking is detected. Hard braking is defined as deceleration greater than 0.4 g.

The EEBL zone is defined as: same direction traffic, ahead (same lane), ahead-left, and ahead-right (left and right immediate adjacent lanes). The longitudinal detection distance is no greater than 300 meters from the front bumper of the HV to the rear bumper of the RV. The EEBL zone shall follow the curvature of the HV’s forward road geometry in both lateral and longitudinal directions.

    - Communications requirements: performance

NLOS: EEBL is intended to function even when the view to the threat vehicles is blocked by other vehicles, inclement weather, or terrain conditions. EEBL shall function with at least one obstructing vehicle between the HV and the RV. The dimensions of an obstructing vehicle that satisfies this minimum performance requirement shall be no greater than 7.3 meters long, 2.2 meters high, and 2.4 meters wide.

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See C-1-3 of Appendix 1, NHTSA Final Report

  - Intersection Movement Assist

    - Description

The IMA safety application is intended to warn the driver of a HV when it is not safe to enter an intersection due to high collision probability with other RVs.

    - Communications requirements: functional

The minimum operational speed range for IMA shall be 7 to 30 meters per second for the RV. The minimum operational speed range for the HV is 0 to 30 meters per second.

The RV(s) shall periodically send out a status as part of the OTA messages. The system shall support periodic broadcast updates with a minimum of at least 100 ms.

The IMA zone is defined as intersecting left or intersecting right cross direction traffic out to a length of 300 meters from the projected Intersection Point. The detection distance is no greater than 300 meters for clear line-of-sight between the front bumpers of the HV and the RV. The width of the IMA zone is one half vehicle length to the left and right of the longitudinal axis through the center of the vehicle.

    - Communications requirements: performance

NLOS: IMA is intended to function even when the view to the threat vehicles is blocked by other vehicles, inclement weather, or terrain conditions.

LOS: IMA shall function with at least one obstructing (equipped or unequipped) vehicle between the HV and the remote-threat vehicle. The dimensions of an obstructing vehicle that satisfies this minimum performance requirement shall be no greater than 7.3 meters long, 2.2 meters high, and 2.4 meters wide.

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See C-1-12 of Appendix 1, NHTSA Final Report

  - Blind Spot Warning / Lane Change Warning

    - Description

The BSW + LCW safety application is intended to warn the driver of the HV during a lane change attempt if the blind-spot zone into which the HV intends to switch is, or will soon be, occupied by another vehicle traveling in the same direction. Moreover, the application provides advisory information that is intended to inform the driver of the HV that a vehicle in an adjacent lane is positioned in a blind-spot zone of the HV when a lane change is not being attempted.

    - Communications requirements: functional

The minimum operational speed range for BSW+LCW shall be 10 - 30 meters per second. Warnings will not be given for stationary vehicles.

The RV(s) will periodically send out status as part of the OTA messages. The system will support periodic broadcast updates up to a minimum of at least 100 ms.

    - Communications requirements: performance

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See C-1-7 of Appendix 1, NHTSA Final Report

  - Do Not Pass Warning

    - Description

The DNPW safety application is intended to warn the driver of the HV during a passing maneuver attempt when a slower moving vehicle, ahead and in the same lane, cannot be safely passed using a passing zone which is occupied by vehicles with the opposite direction of travel. In addition, the application provides advisory information that is intended to inform the driver of the HV that the passing zone is occupied when a vehicle is ahead and in the same lane and a passing maneuver is not being attempted.

    - Communications requirements: functional

The operational speed range for DNPW shall be 11.4 to 30 meters per second. The minimum operational delta speed range between the DNPW-equipped vehicle and an oncoming vehicle will be 11.4 to 60 meters per second. The threat vehicle speed is within +/- 5 mph of the HV speed, and the threat vehicle’s acceleration is negligible (+/- 0.1 m/s^2).

    - Communications requirements: performance

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See C-1-9 of Appendix 1, NHTSA Final Report

  - Left Turn Across Path / Opposite Direction

    - Description

Left Turn Across Path is very similar to Intersection Movement Assist (see above).  In simple terms LTAP is intended warn a driver when it is not safe to make a left turn due to a high collision with a vehicle coming from the opposite direction.

    - Communications requirements: functional

    - Communications requirements: performance

    - Security priority (1=lowest; 5=highest)

    - Communications requirements: security

    - Security threats and risks

    - Communications standards

- DSRC

    - Communications standards gaps

    - Comments

See IMA

http://www.its.dot.gov/factsheets/v2v_factsheet.htm