| Question(s): |  |  | Meeting, date: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: |  | Working Party: |  | Intended type of document (R-C-TD): |  |  |
| Source: | ITU-T Focus Group on Aviation Applications of Cloud Computing for Flight Data Monitoring |  |  |  |  |  |
| Title: | Deliverable 4 - Avionics and Aviation Communications Systems (Rev.1) |  |  |  |  |  |
| Contact: | ITU/TSB |  |  |  | Tel: Fax: Email: tsbfgac@itu.int |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

ITU-T Focus Group on Aviation Applications of Cloud Computing for Flight Data Monitoring

- Deliverable 1 - Existing and Emerging Technologies of Cloud Computing and Data Analytics;

- Deliverable 2/3 - Use Cases and Requirements;

- Deliverable 4 - Avionics and Aviation Communications Systems;

- Deliverable 5 - Key findings, recommendations for next steps and future work.

For more information see http://itu.int/en/ITU-T/focusgroups/ac/

Table of Contents

1.	Executive summary	6

2.	Background and context	7

3.	Structure of the report	8

4.	Relationship with other FG AC working groups	8

5.	Definitions	8

6.	Real time transmission	8

7.	Assumptions	8

7.1	Introduction	8

7.2	The Definition of “Real Time data”	9

7.3	Categories of “Flight Data”	9

8.	Real time data transmission performance	10

8.1	Introduction	10

8.2	Background	10

8.3	Data Streaming	10

8.4	Bandwidth Needs Analysis for Real Time Flight Data Transmission and Data Link Systems Performance Summary	11

8.4.1	Bandwidth Needs Analysis for Real Time Flight Data Transmission	11

8.4.2	Data Link Systems Performance	12

8.5	Conclusions	13

9.	Ground based systems and services infrastructure	15

9.1	Current Infrastructure	15

9.1.1	Introduction	15

9.1.2	Summary of Ground Based Infrastructure Capabilities	15

9.1.3	Flight Data Monitoring, Flight Tracking and Alerting Solutions and Services	15

9.2	Ground Based Infrastructure	16

9.2.1	Introduction	16

9.2.2	System Wide Information Management (SWIM)	16

9.2.3	Flight Data Sharing Programs	17

10.	On board infrastructure	19

10.1	On Board Information Systems Infrastructure	19

10.1.1	Introduction	19

10.1.2	High Level Summary	19

10.1.3	On-board Information Systems	19

10.1.4	Aircraft Flight Data Management and Recording Infrastructure	19

10.1.5	Flight Data Concentrator and Flight Data Recorder	20

10.1.6	Real-time Flight Data Analysis	20

10.1.7	Auxiliary Flight Data Recording	20

10.1.8	Aircraft Servers, IP Data Routing and Airport Surface Data Communications	21

10.1.9	Other Avionics and Electronics Systems	21

10.1.10	Conclusion	22

10.2	On board Aircraft Surveillance and Tracking Infrastructure	22

10.2.1	Introduction	22

10.2.2	ADS-B	23

10.2.3	Future Air Navigations Systems (FANS)	23

11.	On board data link infrastructure	24

11.1	Current	24

11.1.1	Introduction	24

11.1.2	On-board Datalink Systems Infrastructure – AIS Domain / Flight Deck Systems	24

11.1.3	On-board Data Link Systems Infrastructure – PIES Domain/Cabin Systems	26

11.1.4	Data Rates	26

11.1.5	Conclusion – On-board Data Link Infrastructure (Current)	27

11.2	On-board Data Link Infrastructure - Future	29

11.2.1	Introduction	29

11.2.2	Internet Protocol Suite and New Links for Future DataComm	29

11.2.3	Iridium NEXT / Certus	29

11.2.4	Conclusion – On-board Data link Infrastructure (Future)	30

12.	Issues and limitations	31

12.1	Introduction	31

12.2	Future Data Stream Solutions	31

12.3	Data Compression	31

12.4	Cyber Security	31

13.	Recommendations and conclusions	32

13.1	Recommendations	32

13.2	Conclusions	32

Appendix 1- Summary of Ground Based Infrastructure Capabilities	33

Appendix 2 – ADS-B Mandates	46

Appendix 3 – Summary of Data Link Systems Profiles and Performance	47

Appendix 4 – Analysis of Global Bandwidth and Cloud Storage Required to Support Black Box Streaming	47

Appendix 5 – WG 4 Composition	48

Version control

| Version history | Date | Comments |
| --- | --- | --- |
| V 8.1 | 06.02.15 | Presented to plenary Montreal (All) |
| V 1.1 | 01.03.15 | Started new numbering system to track draft versions as sub numbering of Version 1 |
| 1.1 | 01.03.15 | Noted space based vs. terrestrial technologies and the evolution of broadband |
| 1.1 | 01.03.15 | Cyber security, compression, encryption to be included, perhaps not in detail but to be identified as operational issues. Referenced issues including data limitations such as bandwidth, latency, compression, reliability, etc. |
| 1.2 | 15.04.15 | Aligned with relevant principles announced at the ICAO High Level Safety Conference (HLSC) and proposed SARPs |
| 1.2 | 15.04.15 | Ensure we highlight the difference between ATC/AOC/AAC/APC & ACD/AISD/PIESD/PODD |
| 1.3 – 1.5 | 03.05.15 | General format and content updates |
| 1.7 | 17.08.15 | Version presented to WG meeting 4 in Los Angeles |
| 1.8 | 19.08.15 | Formatting and editorial adjustments after WG feedback including improved referencing, linkage to WG 5B, clarity of objectives, formulation of recommendations. |
| 1.9 | 28.09.15 | Updated. Please for review and action also on pending tasks: Nelson, Stephen, Willie, Hannes and Ifeanyi |
| 1.10 | 20.10.15 | Updated Master Document |
| 1.14 | 15.11.15 | Updated Master Document |
| 1.15 | 15.12.15 | Updated Master Document |
| 1.16 | 15.01.16 | Rev1 |

# Deliverable 4 - Avionics and Aviation Communications Systems

# Executive summary

In accordance with the terms of reference of FGAC Working Group 4, this Report examines the feasibility of using recent developments in commercial broadband services, as well as reusing existing infrastructure, for real-time flight data streaming where appropriate.

There are a number of current and future infrastructure components and data link services which will satisfy the objectives of the Global Aeronautical Distress and Safety System (GADSS). These are examined in detail in this Report.

# Background and context

The Global Aviation Community in its quest for continuous and sustainable safety of air navigation shortly after the Malaysia Airline MH 370 disappearance at the behest of the Government of Malaysia held an Expert Dialogue Meeting in Kuala Lumpur that culminated in the setting up of the Focus Group on Aviation Applications of Cloud Computing for Flight Data Monitoring (FG AC) by the International Telecommunications Union.

Based on the above, the FG AC held its first meeting in Kuala Lumpur, Malaysia, 1-3 December 2014, during which four sub-working groups were established. Since then four other meetings were held: February 2015 in Montreal, Canada (ICAO HQ), May 2015 in Geneva, Switzerland (ITU HQ), August 2015 in Los Angeles, USA (Teledyne Controls), and December 2015 in Frankfurt, Germany (Deutsche Lufthansa HQ).

The terms of reference of the WG 4:

“The deliverable examines the feasibility of using recent developments in commercial broadband services, as well as reusing existing infrastructure, for real-time flight data streaming where appropriate.”

The following input contributions were received for Deliverable 4:

- AC-I-018 - Implementation Considerations for Real-Time Flight Data Monitoring by Teledyne Controls, United States.

- AC-I-017 – Broadband Services for Flight Data Monitoring by Inmarsat, United King-dom.

- AC-I-013 – Input to Deliverable 4 by Intelsat, Luxembourg

- SITA Aviation Cloud

- Further to the above, additional inputs have come from group members, ICAO, ITU, and RTCA SC –206 DO-349 Appendix C published in 2014, groups, and during plenary sessions and meetings as well as other Stakeholders.

This Report is based on inputs received from FG AC participants.

The following were areas of focus in this work:

- Ground Based Infrastructure;

- On board information systems Infrastructure; and

- On board Data links Infrastructure.

Other considerations were capability limitations, cyber security and International Civil Aviation Organization (ICAO) Standards and Recommended Practices (SARPs).

In the course of WG 4 deliberations additional experts contributed. The full list is contained in Appendix 5.

# Structure of the report

The Report covers two major areas as indicated below:

- The feasibility of using recent developments in commercial aeronautical data link services: this covers recent developments from various commercial broadband technologies and services for the Aeronautical environment.

- Reusing existing Infrastructure for real-time flight data streaming where appropriate: this covers the various existing aviation satellite technologies and services (safety and non-safety purposes) as being provided currently to the Aviation community and its potential to support real-time flight data streaming.

# Relationship with other FG AC working groups

In accomplishing its tasks WG 4 took into account relevant inputs from the other working groups.

# Definitions

A central consolidation of acronyms and definitions has been produced (see WG 5 deliverable).

# Real time transmission

Real-time transmission of various data from the aircraft has become a significant focus for global aviation safety authorities.  The ability to transmit relevant operational and safety data from aircraft operating in all regions of the globe is seen as an important factor and referenced in the ICAO Global Aeronautical Distress and Safety System (GADSS) report.

This Report examines the feasibility of using recent developments in commercial aeronautical data link services, as well as reusing existing infrastructure, for real-time flight data streaming where appropriate. The Report examines in detail the combination of airborne systems, ground systems and/or associated services that supports the generation, collection, analysis, transmission, storage and sharing of flight data.

# Assumptions

## Introduction

Fundamental assumptions in relation to the use cases were made, as much of the required information is either proprietary or not available at all. Wherever this is the case, assumptions were made based on industry knowledge and experience (see Appendix 4 for the data volumes associated with flight data recording standards).

A detailed description of the use cases are found in Deliverable 2/3.

The following examples of use-cases were considered:

- Flight tracking for safety and security (e.g. Search And Rescue, border protection);

- Flight tracking for route planning and optimization (e.g. crew scheduling and fuel optimization);

- ATM (e.g. ATC including ground movement and airspace optimization);

- Predictive maintenance;

- Inflight and post-flight trouble-shooting;

- Reliability;

- Accident investigation;

- Flight crew techniques;

- Approach statistics;

- OEMs – Air framers and engines;

- Meteorological purposes;

- Cargo information;

- Environmental efficiency;

- R&D information;

- Information for regulatory purposes

It is assumed that these use cases remain valid for the foreseeable future. In accordance with its terms of reference WG 4 focused on flight data monitoring for safety and security.

## The Definition of “Real Time data”

For the purposes of this report, “real time data” is defined as data with adequate update rate and latency to meet the operational requirement.

## Categories of “Flight Data”

The following categories of flight data parameters were considered:

- Navigational and trajectory data (e.g. position, altitude, speed, climb rate, attitude etc.);

- Engineering data (e.g. N1, EGT, hydraulic line pressures, error codes etc.);

- Mission planning and identity information (e.g. call sign, flight number, flight plan, passenger lists and cargo manifests etc.).

# Real time data transmission performance

## Introduction

There is a need to ensure consistent definition and use of data communication capabilities to apply the required communication performance for a global data communications.  This section provides a description of real time data is and supporting data transmission performance.  The material in this section referenced relevant ICAO document 9869 AN/462 MANUAL ON REQUIRED COMMUNICATION PERFORMANCE (RCP).  This report has drawn on this manual to set a baseline of possible real time data communication performance.

This section examines examples of current communication performance standards relevant to navigation and surveillance and explores the data volumes and bandwidth requirements associated with real time flight data transmission that may meet GADSS flight data recovery objectives. The purpose of this report, “real time data” is defined as data with adequate update rate and latency to meet the operational requirement.

## Background

Data communication capabilities provide for the integration of capabilities to exchange information between ground based operations and aircraft.  To establish more contexts the following describes some of primary parameters which are considered:

- Communication transaction time - The maximum time for the completion of the operational communication transaction after which the initiator should revert to an alternative procedure.

- Continuity - The probability that an operational communication transaction can be completed within the communication transaction time.

- Availability - The probability that an operational communication transaction can be initiated when needed.

- Integrity - The probability that communication transactions are completed within the communication transaction time with undetected error.

- Further definitions with regard to current communication standards are:

- RCP 240 would be used for controller intervention capability supporting separation assurance in a 30/30 separation environment.

- RCP 400 would be used for controller intervention capability supporting separation assurance in current environments where separations are greater than 30/30 and alternative technologies are planned

## Data Streaming

Data streaming can and will be used for a variety of purposes. Its application may range from search and rescue, accident investigation to aircraft and engine maintenance management.  The performance requirements will vary depending on the application. Further definitional work will be required to set out what will be the required performance for real time data streaming based on the expected application.  It is anticipated that real time data streaming performance values or standards are likely to be selected based on the anticipated ICAO SARPs for GADSS.

## Bandwidth Needs Analysis for Real Time Flight Data Transmission and Data Link Systems Performance Summary

A study of the bandwidth needs for real time flight data streaming and resulting data volumes generated as well as a survey of various terrestrial and satellite data link systems in use on aircraft today are provided in Appendix 4 and Appendix 3, respectively, and are summarized below.

### Bandwidth Needs Analysis for Real Time Flight Data Transmission

There are two possible modes of real-time flight data transmission that may be considered.

- The first mode is continuous real-time flight data streaming at all times even during normal operations;

- The second mode is for triggered transmission of flight data which involves manual or automated activation of flight data streaming when a distress situation is encountered.

Performing routine and continuous real-time flight data streaming on aircraft generates a relatively low bandwidth requirement per aircraft but generates the largest global requirement.

Relevant studies, including the report published by the BEA after the 2009 Air France Flight 447 accident and an NTSB Safety Recommendation letter published January 22, 2015, recommend that solutions enabling Triggered Transmission of Fight Data (TTFD) are employed for aircraft used on Extended Overwater Operations (EOO).

The NTSB proposes that “(flight) data should be captured (and transmitted) from a triggering event until the end of the flight and for as long as a time period before the triggering event as possible.” Performing triggered transmission of flight data in this manner introduces a higher bandwidth requirement for an aircraft in distress and the bandwidth need increases the closer to the end of flight and the longer the time period before the end of flight. However with a low number of distress situations the global bandwidth needs will be a fraction of that from continuous routine real-time data streaming.

An analysis illustrating the data transmission bandwidth performance needs for both continuous routine black box streaming and TTFD modes of flight data transmission is provided in Appendix 4. The appendix has two sets of tables. The first set of tables describes the global bandwidth need and the global data volumes generated if up to 20,000 aircraft were to be simultaneously streaming flight data. 3 sets of values are provided illustrating the data volumes and bandwidth needs associated with 3 example flight data Black Box recording rates:

- Aircraft Position Data Recording only

- 64 Word Per Second standard Flight Data Recording (circa 1995 common standard)

- 1024 Word Per Second standard Flight Data Recording (circa 2015 common standard)

| FDR Standard | Aircraft Position only | 64 wps FDR | 1024 wps FDR |
| --- | --- | --- | --- |
| Bandwidth needed for routine continuous FDR streaming | 72 bps per (1) aircraft | 768 bps per (1) aircraft | 12.3 Kbps per (1) aircraft |
| Global bandwidth needed | 690 Kbps for 10,000 aircraft | 7.32 Mbps for 10,000 aircraft | 117 Mbps for 10,000 aircraft |
| Global FDR Data Volume | 130 GB per month for 10,000 aircraft | 1.4 TB per month for 10,000 aircraft | 22 TB per month for 10,000 aircraft |

The 1024 wps FDR bandwidth analysis is really a worst case analysis and the overall global bandwidth needs are likely to be significantly less than illustrated. This is because the analysis assumes no data compression is achieved and the FDR standards and actual data volumes are expected to be much less on most aircraft in service. While many newer aircraft record flight data at the 1024 wps standard, the most common standards in use are 256 wps or less for narrow body aircraft and 512 wps or less for wide body aircraft.

Appendix 4 provides various TTFD analysis illustrating how many hours of flight data could be transmitted through 432 Kbps bandwidth based on a triggering event occurring at various times from 1 to 15 minutes prior to the end of flight. Calculations are provided for 1024 wps, 512 wps, 256 wps and 64 wps FDR standards and some extracted results of how much accumulated data could be streamed are shown below.

| FDR Standard | Time of Triggering Event |  |  |
| --- | --- | --- | --- |
|  | 2 minutes before end of flight | 5 minutes before end of flight | 10 minutes before end of flight |
| 1024 wps | 1 Flight Hour of data sent | 2 Hours of data sent | 5 Hours of data sent |
| 512 wps | 2 Hours of data sent | 5 Hours of data sent | 11 Hours of data sent |
| 256 wps | 4 Hours of data sent | 11 Hours of data sent | 23 Hours of data sent |
| 64 wps | 18 Hours of data sent | 45 Hours of data sent | 99 Hours of data sent |

### Data Link Systems Performance

Information relating to the capabilities and bandwidth of various terrestrial and satellite data link technologies are defined in Appendix 3. The Appendix includes two tables; one with terrestrial data link characteristics for VDL Mode 0/A, VDL Mode 2, HF Data Link, VDL Mode 4, UAT/978, 1090ES, GBAS/GRAS VDB and ATG using EVDO and LTE technologies and one with satellite data link characteristics for L band GEO Equatorial of various generations (I3, I4), L band LEO, Ku band GEO and Ka Band GEO technologies.

The appendix provides information for each technology including example providers, link use mode (air-ground, ground-air, air-air), altitude restrictions, geographic coverage, frequency band, data rate, safety classification and latency. The data rates associated with each link are extracted and provided in the tables below.

| Satellite Technology | L Band GEO |  |  |
| --- | --- | --- | --- |
|  | Classic Aero H / H+ | Swift64 | SwiftBroadband |
| Data Rate (from aircraft) | 0.6 –10.5 Kbps | 64 Kbps | 432 Kbps |

| Satellite Technology | L band LEO | Ku Band GEO | Ka Band GEO |
| --- | --- | --- | --- |
| Data Rate (from aircraft) | 2.4 Kbps | 1Mbps | 5Mbps |

| Terrestrial Technology | VDL 0/A | VDL 2 | HF DL | VDL 4 | UAT/978 |
| --- | --- | --- | --- | --- | --- |
| Data Rate (from aircraft) | 2.4 Kbps | 31.5 Kbps | 0.3 – 1.8 bps | 19.2 Kbps | 1 Mbps |

| Terrestrial Technology | 1090 ES | GBAS/GRAS VDB | ATG EvDO Rev.A | ATG EvDO Rev.B | ATG LTE |
| --- | --- | --- | --- | --- | --- |
| Data Rate (from aircraft) | 0.695 kbps | 31.5 Kbps | 1.8Mbps | 3.6 Mbps | TBD |

## Conclusions

- The total data volume associated with flight data recording at the latest common FDR standard of 1024 wps is considerably less than might be expected (Less than 22 TB for 10,000 aircraft)

- The total bandwidth requirements to routinely transmit flight data at 1024 wps in real-time (less than 117 Mbps total for 10,000 aircraft) is considerably less than might be expected

- Many narrow band data link systems have the potential to be used to stream basic flight data since only 72 bps is required to continuously stream aircraft position data from any aircraft.

- Terrestrial data links cannot support Extended Overwater Operations (EOO) which is a primary focus for GADSS

- Existing Ku band and Ka band satellite data link systems have significant bandwidth enough to support both routine flight data streaming and Triggered Transmission of Flight Data

- Classic Aero (over the I3, I4 and MTSAT system) provide near global coverage, has had safety classification for many years and has sufficient bandwidth to achieve some forms of limited data streaming

- SwiftBroadband provides near global coverage, is expected to have safety classification in the near term and provides enough bandwidth to support both routine flight data streaming and Triggered Transmission of Flight Data

- Iridium provides 100% global coverage and has safety classification but does not have sufficient bandwidth today to support streaming of most commonly used flight data (FDR) standards such as 256 wps or 512 wps. Iridium NEXT will have sufficient bandwidth.

# Ground based systems and services infrastructure

## Current Infrastructure

### Introduction

This section explores using existing ground based infrastructure and services for real-time flight data streaming where appropriate. This section explores current computing capabilities and provides a high level summary of each technology. This section is supported by Appendix 1 – Summary of ground based infrastructure capabilities.

Infrastructure that can be used to support real-time flight data streaming can be broken into several components of technology, products and services. It is important to note that the content in Appendix 1 is limited to available information from those organizations who participated or contributed to the work of WG 4.

### Summary of Ground Based Infrastructure Capabilities

The table in Appendix 1 provides an overview of different communication service providers (CSPs) that could potentially provide real time flight data streaming solution. While numerous factors will influence final market outcomes, it is probable that any real time flight data streaming solution may require regulation based on the anticipated ICAO SARPs for GADSS.

In addition, this Report is based on knowledge of existing operations and as such the data does not reflect future equipage, commercial or technology changes.

### Flight Data Monitoring, Flight Tracking and Alerting Solutions and Services

Every airline should have a Flight Data Monitoring application utilized for post flight data analysis. Although not designed for real-time flight data monitoring, these systems may be adapted for real-time flight data monitoring use cases. Examples of FDM software and services providers include:

- Teledyne Controls

- Airbus

- Sagem

- Aerobytes

- GE Aviation (former Austin Digital)

Airlines may utilize a cloud service for FDM hosted by another party. It is worth noting that ICAO Annex 6 does make provision for airlines to outsource their FDM activities should they choose to.

There are also other flight data solutions that may be cloud based, which are used for flight tracking that may also support real-time flight data monitoring, reporting and alerting. Examples of these systems include:

- FlightWatching

- SITA OnAir’s AIRCOM ® Flight Tracker

- Data Centres (e.g. Google, Microsoft, SAP, Oracle)

- Flight Radar 24

- Rockwell Collins MultiLinkSM

## Ground Based Infrastructure

### Introduction

The aviation industry is now focused on interoperability and seamless air traffic management practices.  This section explores some of the concepts that deliver a global approach to data management and sharing.

### System Wide Information Management (SWIM)

Currently, there is no efficient or effective ground-air/air-ground mechanism for the data management, exchange, and sharing of aeronautical information.

The Aircraft Access to SWIM (AAtS) initiative is the effort that will define how and what is necessary to connect aircraft to SWIM infrastructure during all phases of flight. It is important to realize that the AAtS initiative will not implement a specific infrastructure to create the actual link to the aircraft, but it will define a set of operational and technical requirements that will be used to drive that infrastructure. This infrastructure will create a full data information exchange (i.e., uplink/downlink) capability.

Figure 1: Diagram illustrating Aircraft Access to SWIM (AAtS) (picture courtesy of SESAR Joint Undertaking)

AAtS will provide aircraft with guidance on how to connect to a common collection of aeronautical services provided from multiple sources. Example sources include services from the FAA, Department of Homeland Security (DHS), airports and other information sources publishing to the SWIM platform. Using FAA SWIM services and a standards-based approach this will create a globally interoperable and shared aviation information environment. System Wide Information Management (SWIM) is an advanced technology program designed to facilitate greater sharing of Air Traffic Management (ATM) system information, such as airport operational status, weather information, flight data, status of special use airspace, and daily ATM operational limitations. SWIM is designed to support current and future ATM programs by providing a flexible and secure information management architecture for sharing ATM information.

The SWIM concept will be an important and influencing element in facilitating the streaming of real time flight data.  Major global programs such as NextGen and SESAR regard SWIM as central to delivering their programs.

https://www.faa.gov/nextgen/programs/swim/qanda/media/swim_service.png

### Flight Data Sharing Programs

There are several multi-airline and multi-national data sharing programs that exist today that involve the centralizing airline flight data storage. IATA’s Flight Data eXchange (FDX) program and the FAA’s Aviation Safety Information Analysis and Sharing (ASIAS) system are two examples.

- International Air Transport Association’s (IATA) Global Aviation Data Management (GADM)

Techniques to improve aviation safety have moved beyond the analyses of isolated accidents to data-driven analyses of trends and the interaction between the links in the air transport chain.

This approach is supported by the Global Aviation Data Management (GADM) program. The GADM, evolving from GSIC (Global Safety Information Centre), is becoming a broader data management platform, aiming at integrating all sources of operational data received from various channels and IATA unique programs, such as Flight Operations, Infrastructure, IATA audits, etc. into a common and interlinked database structure.

With GADM, IATA will be in a position to provide the industry with comprehensive, cross-database analysis and with this, to support a proactive data-driven approach for advanced trend analysis and predictive risk mitigation.

Pulling from all areas of operations sources, GADM will be the most comprehensive airline operational database available. These sources include the IATA accident database, the Safety Trend Evaluation Analysis and Data Exchange System (STEADES) database, IOSA and ISAGO audit findings, the Flight Data eXchange (FDX), the Ground Damage Database (GDDB), maintenance-related and other operational databases.

More than 470 organizations around the globe submit their data to the GADM. Over 90% of IATA member carriers are participating.

- Federal Aviation Administration (FAA)

The Federal Aviation Administration (FAA) promotes the open exchange of safety information in order to continuously improve aviation safety. To further this basic objective, the FAA developed the Aviation Safety Information Analysis and Sharing (ASIAS) system. The ASIAS system enables users to perform integrated queries across multiple databases, search an extensive warehouse of safety data, and display pertinent elements in an array of useful formats.

A phased approach continues to be followed in the construction of this system. Additional data sources and capabilities will be available as the system evolves in response both to expanded access to shared data and to technological innovation.

Systems that support data sharing and offer data protection to airlines may be suitable platforms to support centralized “escrow” services for hosting airline streamed black box data.

# On board infrastructure

## On Board Information Systems Infrastructure

### Introduction

This section explores the feasibility of using existing information and data systems infrastructure on-board aircraft that could be used to support real-time flight data transmission and data streaming.

The section is structured around specific and current avionics and electronics systems that are often standard and are widely installed and utilized for normal airline operations. On-board information systems infrastructures that are already installed on aircraft that could possibly be used to support real-time flight data transmission or streaming can be broken into several groups of avionics and electronics systems. Aircraft data links systems which transmit data off the aircraft are covered in a later section and are not described in this section which focuses on the systems that generate and provide flight information and data.

This section is supported by Appendix 2 -ADS-B- Mandates.

### High Level Summary

The following is a high level summary of each avionics and electronics system that may be considered a data source that could support flight data transmission or streaming.

### On-board Information Systems

On-board information systems can be grouped as follows for the purposes of considering them for their suitability for streaming aircraft information and flight data:

- Aircraft Flight Data Management and Recording Systems

- Other Avionics and Electronic Systems

### Aircraft Flight Data Management and Recording Infrastructure

These are the systems on-board today that are used to collect, process, analysis, store and for-ward flight data via available off-board data links such as ACARS data links and other data link systems in the flight deck or cabin. Aircraft are also increasingly equipped with airport surface datalinks that are used to transmit recorded flight data that is equivalent to Black Box data. Most airlines and half of aircraft in the world are already equipped and are routing flight data this way post flight on a routine basis for safety and maintenance applications. Small packages of data from onboard Flight Data Management systems are sent via short text messages using ACARS on a large majority of aircraft in the global fleet.

The diagram below illustrates typical Aircraft Flight Data Management and Recording infrastructure that is present on virtually every large passenger and cargo aircraft built since the late 1990s which represents the majority of aircraft in service today. While the names of the units on various aircraft types vary the functionality provided is the same. In the diagram below generic terms are used for the various functions.

### Flight Data Concentrator and Flight Data Recorder

There are typically between 20 and 50 avionics Line Replaceable Units (LRUs) on the aircraft and on the engines that collectively have access often to thousands of flight data parameters. A select sub-set of these data parameters are collected together in real time in a Flight Data Concentrator which in turn packages the incoming data into a stream which feeds into the crash survivable Flight Data Recorder (FDR) more commonly known as the Black Box. The data stored in the flight recorder is utilized for accident and incident investigation purposes.

### Real-time Flight Data Analysis

The flight data acquired by the Flight Data Concentrator is also made available to a real-time analysis function on the aircraft which is most often known as the Aircraft Condition Monitoring System (ACMS) which also has been a standard feature in aircraft since the late 1990s. The real-time analysis function enables various aircraft systems and the engines to be monitored continuously and based on certain triggers or conditions small packages of flight data are sent to airline operations and maintenance through a short text message router (ACARS) which has been commonly used by airlines on most aircraft for more than twenty years. The real-time analysis function also independently sends flight data that can be equivalent to or greater than the Black Box recording to an auxiliary recorder function on the aircraft.

### Auxiliary Flight Data Recording

Since the advent of ICAO Annex 6 Part 1 requirements in 2005 virtually every airline in the world has had a need to routinely collect recorded flight data from the aircraft and perform post-flight Flight Data Analysis for flight operations safety monitoring and improvement purposes. Many airlines were already performing Flight Data Analysis not only for safety benefits but also to realize maintenance and operational efficiency improvements and the industry had already developed several auxiliary flight data recorders. Auxiliary flight data recorder functions (QAR, DAR, SAR) are now also standard on most aircraft since they make it easier to routinely harvest flight data rather than accessing and downloading data from the Black Box (FDR). Auxiliary flight data recorder technology has moved from magnet tape, to Magneto-Optical disk to PCMCA and other solid state cards and increasingly today the auxiliary recorder function is connected with or hosted on a networked system on the aircraft.

### Aircraft Servers, IP Data Routing and Airport Surface Data Communications

Over the last five years aircraft increasingly are installed with a network server or other IP data routing capability and an airport surface data communication capability that features IEEE 802.11 (Wi-Fi) or 2G, 3G or 4G cellular technologies. With all these technologies coming together over 170 airlines and around 8,000 aircraft today are routinely transmitting auxiliary recorder flight data post flight while on the ground at the airport.

### Other Avionics and Electronics Systems

Other systems that generate and collect data that may be suitable or relevant for transmitting aircraft data in flight include the following:

- Flight Management System (FMS). The FMS is an important source and destination for aircraft information. The ACARS system is the data communication system available to the FMS but working together the FMS and the ACARS system enable the important application ADS-C and FANS utilized on many long haul aircraft operations. The FMS also is connected to and outputs flight data parametric data to the Flight Data Concentrator and Real-time Flight Data analysis (ACMS) systems.

- Centralized Aircraft Fault Monitoring or Maintenance Computers. These systems include CMC, CFDIU, ECAM and others. Most avionics units and systems are required to monitor themselves and report any fault conditions and codes in a standard format. The CMC, CFDIU, ECAM or similar systems centralize all the fault information from all the avionics systems on the aircraft. The fault information is made available for download and the most important information that is critical to aircraft maintenance and troubleshooting is relayed to the airline’s maintenance provider via ACARS data links.

- Other ACARS peripherals and end systems. There are many other avionics units that typically have dedicated applications that are also connected to ACARS and are therefore able to send short text message data via ACARS data links. As the FMS, and maintenance computers provide data to the ACMS and FDR system so do most avionics systems also provide flight data to the ACMS and FDR.

- Aeronautical Operation Control (AOC) System. This system is typically resident inside the same unit that is also the ACARS Router. The AOC is used to send short text message operational reports such as start of flight, end of flight, take-off and landing (OOOI) reports.

- Aircraft Interface Devices (AID). Aircraft Interface Devices (AID) are discrete devices or avionics interface functions hosted in other avionics systems that are designed to safely provide flight data and connectivity services to other less critical or non-certified systems such as installed or portable Electronic Flight Bags (EFBs). ARINC 834 defines an Aircraft Data Interface Function (ADIF). Although EFBs were the intended clients for ADIF flight data feeds, it is worth considering that fielded AID ADIF functionality may be re-purposed to support real-time transmission of flight data parameters. Although AIDs have not seen widespread deployment yet on new aircraft from aircraft manufacturers it is expected AID and ADIF functionality will become widespread due to the increasing use of tablets by airline crew on board. It is therefore worth considering connecting AID’s ADIF function via aircraft data links to provide real-time data off board.

### Conclusion

Of all the on-board information systems, ACMS has access to the richest source of data on all aircraft types. ACMS is connected with ACARS and can use all the data links available to the ACARS router. ACMS also provides much larger data volumes to aircraft servers and some QAR units that also function as IP data routers transmitting flight data post flight. These Routers if they are connected with and/or integrated with ACMS are well placed to provide flight data for in-flight streaming. ACMS can support triggering and sending anything from small amounts of data up to full black box data or more and because it is classified as User Modifiable Software (UMS), ACMS can be easily changed and deployed on in service aircraft without need for costly aircraft re-certification.

All the other on-board information systems listed can send data via ACARS but they can’t support flight data streaming. They are not easily connected to Satcom datalinks and it is not easy to change triggering or data content sent on all these systems. ACARS AOC has a UMS reprogrammable capability but is has very limited access to aircraft flight data parameters compared to ACMS.

## On board Aircraft Surveillance and Tracking Infrastructure

### Introduction

Aircraft surveillance is considered an Air Traffic Control function. Primary radar was and is used to track aircraft and it does not require any avionics equipment on the aircraft. Secondary Surveillance Radar (SSR) was introduced to expand surveillance to provide additional information related to the aircraft. SSR technology requires ATC Transponders (Transmitter/Responders) avionics on-board the aircraft. Initially Mode A and Mode C was used for commercial transport but today aircraft utilize Mode S which is an enhanced SSR mode with selective interrogation capabilities. ATC or Mode S transponders ignore interrogations not addressed with their unique identity code, reducing channel congestion. SSR is now being phased out in favour of ADS-B but avionic-wise is an extension of ATC Mode S transponders.

For surveillance needs over oceanic and remote regions which are beyond the reach of terrestrial SSR, VHF and ADS-B technologies, there are two main approaches. The first approach is ADS-C.  This is position report (and other avionics data) which is obtained by the ATC Flight Data Processing (FDP) system setting up a ‘contract’ for information from its peer aircraft avionics ADS-C function (this can be in the FMS on a Boeing aircraft or the Air Traffic Service Unit (ATSU) on an Airbus aircraft). This utilizes and the ACARS data link system for communication. ADS-C is the only solution available to ATC today. The second approach, which will be available in the near future, is space based ADS-B which is enabled by new ADS-B payloads deployed on satellite constellations ‘listening’ to ADS-B ‘broadcast’ positional data and then relay to the ground. The same Mode S transponders that are used in terrestrial ADS-B are planned to be used to support space based ADS-B.

### ADS-B

ADS-B is a well-established cooperative surveillance technology and data broadcast standard which has been used for surveillance for more than ten years primarily over land masses. Space Based ADS-B will enable global surveillance, including oceanic flight operations, when it becomes operational in 2018.  Appendix 2 summarizes the existing or planned ADS-B equipage mandates which will enable maximum operational benefit to be obtained.

The projected performance of Space Based ADS-B is consistent with that of terrestrial ADS-B and fully supports the flight tracking recommendations made by the IATA Aircraft Tracking Task Force (ATTF) in December 2014 and ICAO’s GADSS.

### Future Air Navigations Systems (FANS)

The FANS messages are sent over the ACARS data links and networks. FANS applications include:

- ADS-C. Automatic dependent surveillance-contract (ADS-C) is an existing technology with regulatory approval globally and already provides a two-way communications function between ATC ground systems and aircraft which can be transmitted automatically without pilot action. This is important as it maximizes the utilization of existing certified aircraft tracking ADS-C is an important building block as it currently fully supports the conclusions of the Aircraft Tracking Task Force (ATTF) that a near term goal of global tracking of airline flights should be pursued as a matter of priority.  It is also consistent with the findings from the draft ICAO Global Aeronautical Distress & Safety System (GADSS) Concept of Operation.

# On board data link infrastructure

## Current

### Introduction

This section describes existing data link avionics system infrastructure available on aircraft today. Appendix 3 provides more details. On-board data link systems are typically divided as follows categories. Systems that are a part of and support:

- The flight deck - the Aircraft Control Domain (ACD)

- The Aircraft Information Systems (AIS) data domain

- Data Link systems that are a part of and support the cabin or the Passenger Information & Entertainment Systems (PIES) data domain.

- Data Link systems that are limited to ground use only. Also known as Airport Surface Data Communications systems that include Wi-Fi (GateLink) and Cellular technologies, these systems are not considered further in this Report since they are never used in flight and therefore cannot support flight tracking or real-time in-flight data streaming.

Data Link systems that are required for critical required data communications between air crew and air traffic control and airline operations control are described as supporting Safety Services . For example, aircraft separation through the use of ADS-C is described as a datalink safety service.

For a data link system to be accepted and qualified as suitable for Safety Services the communications avionics and the associated data link services must meet stringent performance requirements.  These avionics systems typically take years to specify, develop and then qualify before they undergo months of flight trials in order to demonstrate the required level of dependability needed for Safety Services. Aeronautical Mobile-Satellite (R) Service (AMS(R)S) is designated by the ICAO and ITU for two-way communications via satellite(s) pertaining to the safety and regularity of flight along national or international civil air routes. To date Inmarsat I-3 (Classic Aero) and 1-4 Classic Aero service are approved for Safety Services. Iridium is now being used for Safety Services and Inmarsat I-4 (SwiftBroadband) is also now undergoing FANS over SwiftBroadband evaluation for Safety Services.

Aeronautical Mobile (Route) Service (AM(R)S) is designated by ITU for two-way communications pertaining to the safety and regularity of flight. To date VHF Data Link including VDL Mode 2 is the only terrestrial data link approved and used for safety services.

Air-to-Ground (ATG) cellular, Ku band and Ka band data link systems are not approved for safety services.

### On-board Datalink Systems Infrastructure – AIS Domain / Flight Deck Systems

Most data link systems for flight deck and avionics use are associated with the ACARS system which is available and used onboard most aircraft today, especially for long haul trans-oceanic aircraft. There is some use of other airborne data links for flight deck use but this is rather limited compared to the use of data links associated with ACARS. ACARS systems and associated Data Links shall be considered first followed by a discussion on other data links utilized for the flight deck.

#### ACARS – Aircraft Communication Addressing & Reporting System (ACARS)

ACARS character-oriented protocol has been in use since the late 1970s, having been designed for transmission over narrow bandwidth pipes such as VHF radios. Linked to this is ground networks hosted by Rockwell Collins Information Systems (ARINC) and SITA, allowing aircraft to send reports of up to 220 characters in length either automatically or upon request. This allows aircraft and airline operations centers to exchange information such as equipment health and maintenance data, flight relevant events such as OOOI (Out, Off, On, In) status, or other en-route flight data such as engine performance, speed, altitude, flight plans, numbers and city pair destinations.

The ACARS unit or function is not a data link system in itself, that processes the character-oriented messages on board the aircraft, but rather a short text message router that uses available data link systems that may be installed and connected. These links include:

- VHF Data Link or VHF Digital Link (VDL Mode 2);

- HF Data Link;

- Inmarsat Classic Aero Satcom systems;

- Iridium Satcom.

These links all are narrow band. HF provides 600 bps, while VDL Mode 2 provides 31.5 Kbps and Analog VHF Data and Satcom links are limited to only 2.4 Kbps when used for ACARS. The actual throughput data rate for VDL Mode 2 is less than 20 Kbps. This means these ACARS data links are suited to sending short character oriented messages as they were designed for, but they are not suited for streaming full Black Box data from modern aircraft generating over 5 MB per flight hour. It is feasible and it has been demonstrated that flight data parameters can be streamed over VDL Mode 2 and Iridium at a lesser rate that matches older Black Box data standard recording rates.

VHF or VDL Mode 2 is the most widely used over land while Classic Aero Satcom is the most widely used on oceanic routes. HF data link is used to a much lesser extent and Iridium is increasingly being used too. Typically airline’s will configure their ACARS systems to utilize the lowest cost link when available which is usually VDL Mode2, then Satcom, , then HF data link but the airline preferences may vary based on their negotiated data services costs.

The diagram also illustrates that many avionics systems are connected to the ACARS router as clients or “end-system” peripherals onboard the aircraft. Systems such as the flight management system (FMS), aircraft condition monitoring (ACMS) and maintenance and fault monitoring (CMC) as well as many other avionics are connected. The ACARS unit itself also includes an Aeronautical Operational Control (AOC) application and the ACARS system is the core messaging protocol for FANS, CPDLC and ADS-C air traffic applications.

#### Other Data Link Systems Used for Flight Deck Applications

There are several systems that are designed for Flight Deck and avionics data communications that utilize Iridium that are not linked with the ACARS system. These include the following systems:

- Panasonic (formerly Airdat) FlightLink Weather Data Link system;

- STAR Navigation’s In-flight Safety Monitoring System (Star-ISMS);

- FLYHT’s Automated Flight Information & Reporting System (AFIRS).

### On-board Data Link Systems Infrastructure – PIES Domain/Cabin Systems

Over the last 5 years there has been more and more broadband data link systems installed in the cabin on many airlines’ aircraft. In the US there have been a large number of Air-to-Ground (ATG) cellular systems installed by GoGo. Elsewhere in the world, airlines have installed SITA OnAir and Aeromobile systems which mostly use Inmarsat SwiftBroadband to bring connectivity to passengers on a global basis. Panasonic, Global Eagle Entertainment (formerly Row44) and Thales (formerly LiveTV) have collectively installed Ku and Ka band satcom systems on significant numbers of aircraft.

### Data Rates

The data rates of the cabin broadband links are high compared to flight deck ACARS links (see Appendix 3):

- SwiftBroadband data link supports up to 432 Kbps per channel

- GoGo’s ATG-3 can provide 1.8 Mbps off the aircraft and 3.1 Mbps to the aircraft

- GoGo’s ATG-4 can provide 3.6 Mbps off the aircraft and 9.8 Mbps to the aircraft

- Ku band offers 1 Mbps off the aircraft and 50 Mbps or more to the aircraft

- Ka band offers 5 Mbps off the aircraft and 50 Mbps or more to the aircraft

All of these systems provide relatively fast data rates off the aircraft compared to ACARS data links i.e. between 432 Kbps and 5 Mbps which is many times more than what would is needed to support Black Box flight data streaming. These cabin links also much less expensive per MB to use so are also well suited to transfer non-safety service, non-ATC ACARS traffic.

Iridium has been installed by some airlines supporting cabin operations but due to the narrow bandwidth (2.4 Kbps) the applications are relatively limited for example to live credit card validation or telemedicine.

### Conclusion – On-board Data Link Infrastructure (Current)

- Flight deck ACARS data link systems are already used to perform flight tracking. Together with the FMS, ACARS enables ADS-C. Since FMS, ACMS and AOC capabilities are all integrated with ACARS, these may be used to expand flight tracking without installing additional equipment on the aircraft. With ACMS and AOC being User Modifiable Software (UMS) they are particularly well suited to hosting trigger algorithms that could be used to implement abnormal and autonomous distress tracking. With the fullest access to flight data parameters ACMS is most likely the best suited and could be used for abnormal and autonomous distress tracking. The probable down side of using ACARS data links is their high transmission cost, but depending on the type of transmission /streaming/function this should be expected to be low, this may not be a major concern.

- Current flight deck data link systems are not suited to full flight data streaming due to the narrow bandwidth and high transmission costs of these data links and due to the fact that flight deck communications are not IP based today but are really designed around messaging using special ARINC protocols.

- Cabin data link systems such as Ku-band, Ka-band and L-band Inmarsat SwiftBroadband where approved do provide very high bandwidth and low cost data transfer that supports routine tracking, distress tracking and even full flight Black Box streaming. ATG links since they operate only over land are not suited for trans-oceanic operations. Cabin broadband Satcom data link systems, although they do not have the same current equipage rates as flight deck data link systems, they are increasingly being installed to provide passenger Internet access and this is forecasted to continue at a high installation growth rate.

- An apparent limitation of cabin data links is that they do not have native access to flight data system sources on board. There are network enabled IP data routing systems that have access to flight data that could be connected with the cabin broadband data link systems; and with time most of the Ku-band and Ka-band services will cover more and more of flight routes. Cabin data links also have the issue that they are within the PIES domain on the aircraft which means there are additional security measures that may be needed to protect AIS domain systems from potential attack from the cabin. However the industry is already working on security solutions to enable AIS and PIES domains to be connected.

- The diagram below illustrates how on board information systems as described in Section 14.1 may be connected with broadband data link systems to enable real-time data transmission.

- If cabin data link systems can be securely connected to AIS domain flight data information infrastructure on board such as IP data routers that already have access to flight data then this combination would be very well suited to performing flight data streaming in support of GADSS flight data recovery requirements. Airlines are already downloading aircraft flight data post flight over airport surface data links. Re-use of these systems to re-direct the data transmission over broadband links is logical. Only after ICAO establishes performance standards can it be ascertained which data links can be used to meet the requirements.

- Since ICAO guidelines are that the solution for data streaming shall be performance based and be the responsibility of air carriers, and shall not be prescriptive it will be possible for airlines and/or aircraft manufacturers to select from the combinations of available data acquisition, processing and routing systems and available datalink systems to build a solution that meets the SARPs.

- In view of the above, further considerations on frequency spectrum allocations and bandwidth requirements may be envisaged in order to properly examine the feasibility of reusing existing infrastructure to support real-time flight data streaming, which covers the various existing aviation satellite technologies and services (safety and non-safety purposes) as currently being provided to the aviation community throughout the world.

## On-board Data Link Infrastructure - Future

### Introduction

This section explores the feasibility of using recent developments in on-board data link infrastructure. Future on-board data link systems may also be divided into three categories:

- Data Link systems that are a part of and support the Aircraft Control Domain (ACD) or the Aircraft Information Systems (AIS) aircraft data domain. Planned developments here include the approval of Inmarsat SwiftBroadband for Safety Services use and introduction of a new Internet Protocol Suite (IPS) that will utilize SwiftBroadband, VDL Mode 2 as well as new data links such as L-band Digital Aeronautical Communications System (LDACS) terrestrial data link, Iridium Certus and other “Future Satcom” technologies for flight deck use.

- New Data Link systems that are a part of and support the cabin or the Passenger Information & Entertainment Systems (PIES) data domain. Future developments include the introduction of Ku band and Ka band systems using High Throughput Satellites (HTS) and dual channel Ku band systems such as GoGo’s 2KU system. Also there are new ATG networks and systems planned including an LTE based ATG planned by Inmarsat for the European region.

- Data Link systems that are limited to ground use only. Future Airport Surface Data Communications systems developments expected are introduction of equipment utilizing commercial LTE as well as AeroMACS which uses a dedicated 5.1 GHz band allocated for aviation use by the ITU. These airport surface data systems are not considered further in this report since they will not be used in flight and therefore cannot support flight tracking or real-time in-flight data streaming.

### Internet Protocol Suite and New Links for Future DataComm

New network infrastructure for safety services based on the modern Internet Protocol Suite (IPS) is planned to meet future SESAR/FAA NextGen future DataComm needs. The airline and manufacturer industry body SAE-ITC Airlines Electronics Engineering Committee (AEEC) is considering beginning work to create a detailed technical definition of IPS for aeronautical safety services in a new ARINC Standard. This specification is to be based on the ICAO Doc 9896 IPS definition and on prevalent commercial IP network technology (e.g., IETF RFC 2460 for IPv6) with the modifications necessary to support aeronautical safety services. It is anticipated that IPS will use multiple line-of-sight and beyond-line-of-sight subnetworks that operate in ‘protected’ spectrum allocated by ITU and ICAO for safety services, including Inmarsat SwiftBroadband, Iridium Certus, AeroMACS, future Satcom and LDACS systems, and possibly VDL Mode 2.

### Iridium NEXT / Certus

Iridium will begin the replacement of the entire Iridium satellite constellation of 66 low Earth Orbit satellites including 6 in-orbit spares. This replacement network is called Iridium NEXT and will begin in 2016 and will complete by late 2017. First aircraft equipage and regulatory operational assessments will take place in early 2017 for inclusion in testing and development of the ICAO GADSS program. With the increased capacity and much greater bandwidth (up to 1.4 Mbps), Iridium will continue to provide safety voice and data communications in addition to an entire new capability of safety and non-safety services including Flight Data Recorder (FDR) download and other utilizing secure IP streaming capability.

### Conclusion – On-board Data link Infrastructure (Future)

Due to the long-time scales involved in developing new avionics data link systems and equipping significant number of aircraft already in service the future onboard data link systems described above may not be suitable in the near term. In the long term for 2020 and beyond use of these data links systems could be considered.

In view of the above, further considerations on frequency spectrum allocations and bandwidth requirements may be envisaged in order to properly examine the feasibility of using future data link systems and recent developments in commercial aeronautical datalink services, which covers the latest developments from various commercial broadband technologies and services for the aeronautical environment throughout the world.

# Issues and limitations

## Introduction

There are a range of strategic and technical issues which must be explored across the work of the entire sub working groups.  To ensure that these issues are documented the following have been identified to date.

## Future Data Stream Solutions

Given the limited time to devote to this report it has not been possible to define or develop future solutions for data streaming which could reduce the consequences associated with aircraft operating in abnormal circumstances.  There is an opportunity to progress this future design work using this report as the baseline of existing capabilities.

## Data Compression

It involves encoding information using fewer bits than the original representation. It is useful because it helps reduce resource usage, such as data storage space or transmission capacity. Lossless (no information is lost) compression reduces bits by identifying and eliminating statistical redundancy and involves trade-offs among various factors, including the degree of compression, space–time complexity and the computational resources required.

In order to make an efficient use of the frequency spectrum and to make best use of available bandwidth, data compression is a must. There are plenty of mechanisms already developed and tested that might be ready to implement, for example, Recommendation ITU-T V.44 (11/00) offers a compression ratio of 6:1 (for pure text).

## Cyber Security

Cyber security is the process of applying security measures to ensure confidentiality, integrity, and availability of data by which digital equipment, information and services are protected from unintended or unauthorized access, change or destruction. The goal is to protect data both in transit and at rest and includes, but not limited to encryption, integrity and authentication methods. Countermeasures can be put in place in order to ensure security of data. Some of these measures include, but are not limited to, access control, awareness training, audit and accountability, risk assessment, penetration testing, vulnerability management, and security assessment and authorization.

# Recommendations and conclusions

## Recommendations

The following recommendations are proposed for ITU consideration:

- That there are a range of existing technologies and infrastructure which can support the establishment of real time data streaming capabilities from operating aircraft.

- Note that this Report contains a significant amount of material that can be considered under the responsibility of the Radiocommunication Sector (ITU-R) and is indeed currently being studied in ITU-R Study Groups 4 and 5.

- Note that this report represents a valuable baseline of real time data streaming capabilities and the content is relevant to many aspects of current safety improvements associated with flight tracking and real time data streaming.

- Ensure that the various related working group committees are supplied with a copy of this report to support the various aspects related to improving aviation safety.

- Once GADSS performance based requirements are defined for flight data streaming, further work will be required regarding the assessment of aircraft types and current equipage levels, what level of global service coverage is needed, what data volumes may be sent and what bandwidth is needed – and assess worst case needs (i.e., the bandwidth needed).

- Explore the significant range of operational, regulatory, technology and commercial aspects of the findings documented.  This is work which could be conducted subject to the views of the ITU.

- Commence work to define or develop future solutions for data streaming which could reduce the consequences associated with aircraft operating in abnormal circumstances, using this report as the baseline of existing capabilities.

- Consider the material contained in this Report in further developing related activities and relevant Reports/Recommendations under the scope of concerned ITU study groups.

- Further work is required to establish real time data streaming performance parameters or standards and these values or parameters are likely to be selected based on the anticipated ICAO SARPs for GADSS.

## Conclusions

This Report examines the feasibility of using recent developments in commercial aeronautical datalink services, as well as reusing existing infrastructure, for real-time flight data streaming where appropriate.

The findings are that there are a range of existing technology capabilities that can be utilized that have existing avionic and regulatory approval and are consistent with the findings of the ATTF and GADSS. In addition there is a commercial evolution path with new technologies that are being progressed which also are consistent with the ATTF and GADSS concepts.

The analysis conducted also suggests that the original concept of black box in the cloud is a limiting term in that real time streaming has a broader relevance and meaning as there are a variety of technology solutions that could be implemented.

The report also concludes that while this is a valuable source document there are a number of actions which could be progressed and these are outlines in the recommendations section above.

# Appendix 1- Summary of Ground Based Infrastructure Capabilities

The following table provides a brief overview of different communication service providers (CSPs) that could potentially provide or support the development of real time flight data transmission.

Note: It is important to note that the content in the table is limited to those organizations who participated or contributed to the work of the Working Group.

| Service Provider | Firms | Datalink Provider | Downstream Data from the flight deck | Send messages and control flight deck | Is the datalink certified for safety communications? | Experience with "event-triggered" systems | Current Safety Connectivity Products | Equipage rates |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Legacy Cockpit CSPs | SITA On Air, | Inmarsat & Iridium |  |  |  |  |  |  |
|  | ARINC Rockwell Collins, Satcom Direct |  |  | Yes. | Yes. | Yes. | - Electronic Flight Bags |  |
|  |  |  |  |  |  |  | - Electronic Flight Instruments | - 80% of wide bodies are equipped. |
|  |  |  |  | ARINC and SITA are the leading CSPs for safety critical communications. Their services (from onboard the aircraft to the ground infrastructure to the software programs) are certified to transmit safety critical data between aircraft and air traffic controllers. | ARINC and SITA both use Inmarsat L-band and a series of VHF/HF ground networks. | ARINC and SITA specialize in software employed by ATC/AOC ground units, which includes programs designed to interpret aircraft flight data in real time. | - Flight Data Computer Systems |  |
|  |  |  |  |  |  |  | - Cockpit Communications Hardware/Interfaces | - 4,200 satellite connected aircraft (20% of global fleet). |
|  |  |  | Yes as Datalink Providers | This allows their onboard network technologies full access to communicate with flight computers. |  | They are safety certified for bidirectional communication with the aircraft flight computers, and can run an event identification/alert system outside of the aircraft. | - Datalink and Satcom System services |  |
|  |  |  |  |  |  |  | - large scale global AOC/ATC service capabilities | - Extensive communications technology infrastructure on the ground. |
|  |  |  |  |  |  |  |  | - Extensive product range dealing with ATC/AOC software programs. |
|  |  |  |  |  |  |  |  | - Satellite over remote areas/VHF over major landmass areas. |
|  | Flyht, STAR Navigation, Blue Sky Navigation, Spidertracks | Iridium |  |  |  | Yes. |  |  |
|  |  |  | Yes. | Yes but are not approved for safety services as per the ICAO GOLD manual. | Not all. |  | - Electronic Flight Bags | - Only datalink provider with polar coverage. |
|  |  |  |  |  |  |  | - Electronic Flight Instruments |  |
|  |  |  | Just as ARINC and SITA, these companies tend to provide a bundled service that includes flight deck applications, datalink provisioning, air to ground and software to access and manipulate flight data on the ground. |  | For example Iridium L-band is certified for safety operations. |  | - Flight Data Computer Systems | - Extensive experience, certification in safety and cockpit communications. |
|  |  |  |  |  |  |  | - Cockpit Communications Hardware/Interfaces |  |
|  |  |  | Flyht in particular has developed extensive ability to harvest, package, and transmit different types of data. |  |  |  | - Datalink and Satcom System services |  |
|  |  |  |  |  |  |  | - AOC/ATC tailored products |  |
| Cabin oriented CSPs | Gogo, Panasonic, Global Eagle, Viasat, Global Xpress (Inmarsat) | Ku and Ka band Satellite operators |  |  |  |  |  |  |
|  |  |  | Yes. | No. | No. | No. |  | - Over 4,000 aircraft online today, at least 12,000 by 2023. |
|  |  |  | While unable to send commands to the flight computer, the onboard equipment can stream flight data off of the flight computers. This could then be transmitted off the aircraft to 3rd parties. | Cabin oriented CSPs currently use onboard network technology that is not certified to send communications to cockpit flight computers. | The Ku and Ka bands are not approved for safety communications due to risk of link failure (rain fade, skew angle degradation, etc.) | Cabin oriented CSPs would be obliged to incorporate AIDs to merge their onboard networks with flight deck avionics. | - Some EFB and limited cockpit/crew applications. | - Increased throughput |
|  |  |  |  |  |  |  | - Limited data streaming to Airline Operations. |  |
|  |  |  |  |  | Today, only the L-band is approved under the ARINC standards |  |  | - Reduced cost per MB |
| Equipment/Airborne Infrastructure Providers | Teledyne Controls, Arconics, Lufthansa Systems, UTC Aerospace, DAC International, navAreo, Astronautics, CMC Electronics, Flyht, Cobham | Access to multiple data links | Yes | Unknown, but will likely be subject to strict regulations once BBiC standards are established. | N/A | Yes. |  | - Ability to aggregate data from various parts of aircraft, including different software platforms, into one format |
|  |  |  |  |  |  |  | - Electronic Flight Bags |  |
|  |  |  |  |  |  | The majority of these companies have "smart data" capability, as well as the ability to access specific data types from the flight deck. | - Aircraft Interface Devices | - AIDs allow non SOS-certified hardware (i.e. EFBs) to have bidirectional communications with Safety avionics. |
|  |  |  |  |  |  |  | - Electronic Flight Instruments |  |
|  |  |  |  |  |  |  | - Various AOC services such as flight tracking, terrestrial data streaming, etc... | - Ability to convert ACARS messages into IP data packets to be sent over broadband links |
| Flight Data Monitoring | FDSL, Teledyne, Sagem, Aerobytes, Airbus, GE Aviation | N/A | N/A | N/A | N/A | N/A | N/A | performed by virtually all airlines |
| Air Traffic Service tracking providers | Multiple ATC service providers such as FAA, Air services Australia, Airways New Zealand, NAMA Nigeria, ATNS South Africa, Euro control. | No | Yes | Yes depending on type of technology | Yes subject to type of technology | Yes subject to type of technology | N/A | Unknown and subject to type of technology |

# Appendix 2 – ADS-B Mandates

The following table summarizes planned or existing ADS-B mandates globally.

| Region | Published Material and Equipage Mandates |
| --- | --- |
| Europe | - The European Commission has enacted an Implementing Regulation laying down requirements for the performance and the interoperability of surveillance for the Single European Sky ((EU) No 1207/2011). This was recently updated by Commission Implementing Regulation (EU) No 1028/2014 of 26 September 2014 amending Implementing Regulation (EU) No 1207/2011 which mandates specific ADS-B equipage after 7 June 2020. |
| United States | - In 2010, the FAA issued a new rule contained in Title 14 of the Code of Federal Regulations (14 CFR) part 91, §§ 91.225 and 91.227. This rule requires ADS-B (Out) performance when operating in designated classes of airspace within the NAS after 1 January 2020. |
| Canada | - Transport Canada Advisory Circular (AC) No. 700-009 - Issue 2 EASA AMC 20-24 |
| Australia | - Guidance material: CAO 20.18, Amend Order No. 3, dated December 2009 - Mandates ADS-B Out for upper airspace (≥FL290) in December 2013 |
| Hong Kong | - After 31 December 2014 for aircraft flying within Hong Kong FIR between FL290 and FL410 - Must meet DO-260 (Version 0) requirements of ICAO Annex 10 and ICAO Doc 9871 Chapter 2, or DO-260A (Version 1) requirements of ICAO Doc 9871 Chapter 3 - Means of compliance per EASA AMC 20-24 or CASA CAO 20.18 Appendix XI |
| Singapore | - Guidance material: CAAS AIC 14, 28 December 2010 - Implement the use of ADS-B Out after 12 December 2013 within certain parts of the Singapore FIR (≥FL290) - EASA AMC 20-24 or CASA CAO 20.18 Appendix XI, otherwise must fly at <FL290 |
| Other Asia Pacific | - Expected to follow ADS-B Avionics Requirements template per APANPIRG Conclusion 21/39 - EASA AMC 20-24 or CASA CAO 20.18 Appendix XI |

# Appendix 3 – Summary of Data Link Systems Profiles and Performance

# Appendix 4 – Analysis of Global Bandwidth and Cloud Storage Required to Support Black Box Streaming

Worst case scenarios for continuous streaming and triggered streaming are provided in the spreadsheet below. The triggered streaming are for various word per second with associated Kbps from 64 wps – 1024 wps.

# Appendix 5 – WG 4 Composition

i)	Ifeanyi Frank Ogochukwu - Debbie Mishael Consulting, Nigeria (Group Leader)

ii)	Stephen Angus - Inmarsat, UK

iii)	Matt De Ris - Panasonic Avionics Corporation, USA

iv)	William Cecil - Teledyne Controls, USA

v)	Hannes-Stephan Griebel - Thales Alenia Space, Germany

vi)	Juan Pablo Martin - Universidad Tecnológica Nacional, Argentina

vii)	Nelson Malaguti - International Telecommunication Union, Switzerland

viii)	Maiwada Abdulaziz - Nigerian Airspace Management Agency, Nigeria

ix)	Olumuyiwa Adegorite - Nigerian Airspace Management Agency, Nigeria

x)	Rachel Donald - Inmarsat Aviation, Switzerland

xi)	Carlos Flores - Federal Communications Commission, USA

xii)	Loftur Jonasson - International Civil Aviation Organization, Canada

xiii)	Paul Najarian - Department of State, USA

xiv)	Ken McLean - Aireon LCC, Australia

xv)	Michael Hooper - Iridium, USA