| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 100-E June 1998 Original: English |

Question: 9/11

Texte disponible seulement enText available only inE

STUDY GROUP 11 – REPORT R 100

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (05/98) FOR RECOMMENDATION Q.50 (03/93)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.50 (03/93)

## 1.2	Background

This guide is an accumulation of amendments to the 1993 edition of the ITU-T Recommendation Q.50.

It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

–	editorial errors;

–	technical errors, such as omissions, inconsistencies, etc.; and

–	ambiguities.

## 1.4	Contacts

| Editor: | Peter Goldstein Siemens Ltd Switzerland | Tel: +41 1 495 3884 Fax: +41 1 495 4053 Email: peter.goldstein@itu.ch |
| --- | --- | --- |

## 1.5	Document history

| Version | Summary |
| --- | --- |
| 05/98 | New Implementors' Guide |

# 2	Editorial Errors

## 2.1	5.3 Change of connection type after call set-up

Error: The first sentence incorrectly references "simultaneous" call set up, instead of "demand" call set up.

Correction: In the first sentence replace "simultaneous" with "demand".

Error: The statement "The procedure to support changes of the connection types is for further study" is inappropriate, since the protocols in Annexes A and B do not support the change of connection type.

Correction: Add: In an implementation using either Annex A or B, the change of connection type cannot be indicated to the DCME. Therefore the "request for 64 kbit/s" sent at the beginning of the call from the ISC towards the DCME will not be changed until the call is released. This aligns the current signalling capability defined in Annexes A and B with Multi-Use Bearer Service.

Delete: "The procedure to support such changes of the connection type is for further study."

## 2.2	Annex B, B.1 General

Error: The information in this section apparently was taken unchanged from a previous contribution when Annex B was generated. However, the signalling interface in Annex B is not intended for the exclusive use to connect "Deutsche Bundespost exchanges to the TAT-8 cable", but rather is for universal use.

Correction: Delete the first and second sentences of B.1 General:

–	The interface between ISC and DCME ... onward.

–	Appropriate test equipment ... end of 1996.

Replace: "The mentioned interface ..." with "The signalling interface ...".

## 2.3	Annex B, B.6 DCME load tests

Error: The information in this section apparently was taken from a previous contribution explaining some possible test scenarios. The section however does not contain protocol relevant information.

Correction: Delete the section.

# 3	Technical Errors

None.

# 4	Ambiguities

None.

________________