| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 102-E June 1998 Original: English |

Question: 12/11

Texte disponible seulement enText available only inE

STUDY GROUP 11 – REPORT R 102

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (05/98) FOR RECOMMENDATION Q.732.2-5 (07/96)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.732.2-5 (07/96)

[2]	ITU-T Recommendation Q.115 (01/97)

[3]	ITU-T Recommendation Q.764 (09/97)

## 1.2	Background

This guide is a compilation of amendments to the 1996 edition of the ITU-T Recommendation Q.732.2-5 [1]. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

•	editorial errors;

•	technical errors, such as omissions, inconsistencies, etc.; and

•	ambiguities.

## 1.4	Contacts

Editor:	Pieter Veenstra				Tel:	+31 70 3439121
	KPN Telecom B.V.				Fax:	+31 70 3437303
	P.O. Box 30150				Email:	p.k.veenstra@kpn-telecom.nl
	2500 GD The Hague
	The Netherlands

## 1.5	Document history

| Version | Summary |
| --- | --- |
| 05/98 | New Implementors' Guide |

# 2	Editorial errors

None.

# 3	Technical errors

## 3.1	Echo control-related procedures

Synopsis

New echo control logic procedures have been specified in ITU-T Recommendation Q.115 [2] in conjunction with new echo control signalling procedures in ITU-T Recommendation Q.764 [3]. As a result, the echo control-related statements in the procedures for Call Diversion services have become outdated. As a consequence, the echo control-related text phrases in § 2.5.2.5.1.2 c) and § 2.5.2.5.1.2 e) have to be replaced by new text.

Please replace the following text phrases in ITU-T Recommendation Q.732.2-5 [1]:

2.5.2.5.1.2 c)	Diversion procedures at the diverting exchange

...

If during the establishment of the forwarded leg the total propagation delay of the connection exceeds the value where echo control is necessary, echo control devices should be included. The echo control procedures as defined in the basic call control procedures apply.

...

2.5.2.5.1.2 e)	Handling of messages and parameters in a diverting exchange

...

TABLE 2-4/Q.732

Mapping of information received in an ACM

| ACM | CPG |
| --- | --- |
| ... | ... |
| Other parameters are transferred transparently (NOTE) |  |
| NOTE - The echo control information parameter is not transferred into the CPG message. The procedures are according to 2.7.2.2.2.1/Q.764 of the basic call. |  |

..

by:

2.5.2.5.1.2 c)	Diversion procedures at the diverting exchange

...

The echo control logic as defined in ITU-T Recommendation Q.115 (01/97) shall apply. The exchange shall also have the capability of storing propagation delay information received in the Initial Address message.

...

2.5.2.5.1.2 e)	Handling of messages and parameters in a diverting exchange

...

TABLE 2-4/Q.732

Mapping of information received in an ACM

| ACM | CPG |
| --- | --- |
| ... | ... |
| Other parameters are transferred transparently (NOTE) |  |
| NOTE - The contents of the echo control information parameter may be changed by the echo control logic as defined in ITU-T Recommendation Q.115 (01/97). |  |

...

# 4	Ambiguities

None.

__________________