| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 210-E February 2000 Original: English |

Question: 2/11

Texte disponible seulement enText available only inE

STUDY GROUP 11 - REPORT R 210

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (12/99) FOR RECOMMENDATION Q.754 (06/97)

________

Summary

This document contains the enhancements to MRVT parameter values to give additional diagnostics when a loop is detected in the MTP routing tables during an MTP Routing Verification Test. A parameter has been added for a current route indication. Some errors in the typing of the OMASE ASN.1 are also corrected from 06/97 Q.754. The complete ASN.1 OMASE definition is included to enable compilation.

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.754 (06/97) - Signalling System No. 7 management Application Service Element (ASE) definitions.

## 1.2	Background

This guide is a compilation of reported defects, their resolutions and two minor upgrades to the 1997 edition of ITU-T Recommendation Q.754. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following category:

•	editorial errors.

In addition, this guide records two minor enhancements to the Recommendation in the following category:

•	increased interoperability.

## 1.4	Contacts

| Rapporteur Q.2/11: | Clemens Suerbaum Siemens AG ICN M CS 22 Machtlfingerstr. 1 D-81359 Munich Germany | Tel: +49 89 722 42418 Fax: +49 89 722 29098 E-mail: clemens.suerbaum@icn.siemens.de |
| --- | --- | --- |

## 1.5	Document history

| Version | Summary |
| --- | --- |
| 05/98 | New Implementors' Guide |
| 12/99 | Addition of currentRouteList. BIT STRING ellipsis replaced by SIZE(32) constraint. |

# 2	Q.754 changes for increased interoperability

## 2.1	Change subclause 2.1.1.1.6 to:

##### 2.1.1.1.6	infoRequest

This optional parameter inserted, if at all, only by the SP initiating the test, indicates that the initiator can recognize MRVR messages occasioned by the routeTraceNew event type. The infoRequest parameter indicates which information is requested if an MRVR message should be sent to the initiator. It also can indicate which parameters should be updated as the MRVT messages traverse the network. Current values can be pointCode (bit 0 = 1), and/or pointCodeList (bit 1), and/or routePriorityList (bit 2), and/or currentRouteIndicatorList (bit 3).

| Parameter | Code |
| --- | --- |
| InfoRequest | 10001101 |
| Contents |  |
| Bit string containing any or all of the indicated values. |  |

## 2.2	Add a new subclause 2.1.1.1.9:

#### 2.1.1.1.9	currentRouteIndicatorList

If the infoRequest parameter is included and requests it, as each SP is traversed it adds the currentRouteIndicator of the route to the next SP into the currentRouteIndicatorList.

| Parameter | Code |
| --- | --- |
| currentRouteIndicatorList | 10110001 |
| Contents |  |
| Sequence of CurrentRouteIndicator, tagged as "CurrentRouteIndicator" with the contents indicating unknown, or currentRoute, or one of the reasons for possibleRoute viz: possibleRouteSignRouteSetLocked, possibleRouteLinkSetUnavailable, possibleRouteHigherPriorityRouteAvailable, or the general value possibleRouteNoFurtherAnalysisPerformed. |  |

## 2.3	Section 2.1.2.1.2 becomes:

##### 2.1.2.1.2	detectedLoop

When a loop is detected, the trace of Point Codes of the crossed SPs augmented in order by the Point Code of the SP detecting the loop and the Point Code of the SP completing the loop are included.

| Parameter | Code |
| --- | --- |
| detectedLoop | 10100001 |

## 2.4	Section 2.1.3 becomes:

### 2.1.3	routeTraceNew

This report is used if the prompting testRoute action contained an infoRequest parameter.

| routeTraceNew EVENT | Timer  0 | Class  4 | Code  00000100 |
| --- | --- | --- | --- |

#### 2.1.3.1	Event information

##### 2.1.3.1.1	success

On successful completion, the trace of the Point Codes (one or more) of the SPs crossed are included in pointCodeList (copied from the pointCodesTraversed parameter of the testRoute action).

routePriorityList is copied from the testRoute action, if present and requested in the infoRequest parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

currentRouteIndicatorList is copied (and possibly filled up to the correct length) from the testRoute action, if present and requested in the infoRequest parameter.

##### 2.1.3.1.2	detectedLoop

If a loop is detected, the trace of Point Codes of the crossed SPs augmented in order by the Point Code of the SP detecting the loop and the Point Code of the SP completing the loop is included in pointCodeList.

routePriorityList is copied from the testRoute action, if present and requested in the infoRequest parameter, augmented by the priority of the route to the test destination via the SP completing the loop.

currentRouteIndicatorList is copied (and possibly filled up to the correct length) from the testRoute action, if present and requested in the infoRequest parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.3	excessiveLengthRoute

If this error occurs, the entire route is copied from the testRoute action pointCodesTraversed parameter into the pointCodeList parameter.

routePriorityList is copied from the testRoute action, if present and requested in the infoRequest parameter.

currentRouteIndicatorList is copied (and possibly filled up to the correct length) from the testRoute action, if present and requested in the infoRequest parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.4	unknownDestination

This is equivalent to unknownDestination of 2.1.2.1.4. If the infoRequest parameter of the prompting testRoute action requested it, the pointCodesTraversed parameter of the testRoute action is copied into pointCodeList.

routePriorityList is copied from the testRoute action, if present and requested in the infoRequest parameter.

currentRouteIndicatorList is copied (and possibly filled up to the correct length) from the testRoute action, if present and requested in the infoRequest parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.5	routeInaccessible

If this event is reporting just one inaccessible SP, its point code is placed in pointCode.

If the event is reporting more than one inaccessible SP (and hence the prompting testRoute action indicated that the originator could accept it), the list of all such inaccessible SPs is put into pointCodeList.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.6	processingFailure

If the test cannot be run due to local conditions, the event reports a processing failure. This includes rejection of the testRoute action by SCCP or TC at a remote SP.

If the testRoute action infoRequest parameter was present, and had bit 0 set to 1, the point code of the SP where the test failed is put into the pointCode parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.7	unknownInitiatingSP

The point code of the SP detecting the unknown initiator is returned in the pointCode parameter.

If the prompting testRoute result contained a copyData parameter, this is copied into the routeTraceNew copyData parameter.

##### 2.1.3.1.8	timerExpired

The point codes of the SPs from which no result of the testRoute actions were received are included into pointCodeList.

##### 2.1.3.1.9	sPNotAnSTP

This error occurs if the intermediate SP does not have the STP function, or it is known that it is not authorized to transfer messages from the test initiator to the test destination.

The pointCodesTraversed parameter of the prompting testRoute action is copied into pointCodeList.

routePriorityList is copied from the testRoute action, if present and requested in the infoRequest parameter.

currentRouteIndicatorList is copied (and possibly filled up to the correct length) from the testRoute action, if present and requested in the infoRequest parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.10	indirectRoute

This report is used if the direct route test was requested and the SP receiving the MRVT message did not have a route to the test initiator directly through the MRVT message sender. The identity of the prompting MRVT message sender is included in the pointCode parameter, the identity of the SP detecting no direct route is the OPC in the MRVR message's MTP label.

##### 2.1.3.1.11	maxNrMRVTestsAlready

This report is used by the signalling point receiving the MRVT message if the maximum number of MRV Tests nT are already running at the SP.

If the testRoute action infoRequest parameter was present, and had bit 0 set to 1, the point code of the SP where the test failed is put into the pointCode parameter.

If there are parameters in the prompting testRoute action that are not understood, and testRoute contains a returnUnknownParams parameter which requests them, they are copied into the copyData parameter.

##### 2.1.3.1.12	unknownParameter

When a node receives an MRVT message containing one or more unrecognized parameters, the Point Code of the node not recognizing the parameter is included, plus, if their return was requested in the testRoute returnUnknownParams parameter, a copyData parameter containing those unrecognized parameters.

# 3	Q.754 changes due to editorial errors and MRVT enhancements

Section 6.3 becomes:

-- OMASE  protocol --
OMASE { itu-t(0) recommendation q 754 omase(0) version2(2) }

DEFINITIONS EXPLICIT TAGS::=
BEGIN
-- TCAP definitions --  
EXPORTS EVERYTHING;

-- the OPERATION and ERROR information objects defined here are equivalent to the respective MACROs in
-- TCAPMessages {ccitt recommendation q 773 modules(2) messages(1) version2(2) } of Rec. Q.773 (1993) --

OPERATION ::= CLASS
{
		&ArgumentType		OPTIONAL,
		&ResultType		OPTIONAL,
		&Errors			ERROR OPTIONAL,
		&Linked			OPERATION OPTIONAL,
		&operationCode		Code UNIQUE OPTIONAL
}
WITH SYNTAX
{
		[PARAMETER	&ArgumentType]
		[RESULT		&ResultType]
		[ERRORS		&Errors]
		[LINKED		&Linked]
		[CODE		&operationCode]
}

ERROR ::= CLASS
{
		&ParameterType		OPTIONAL,
		&errorCode			Code UNIQUE OPTIONAL
}
WITH SYNTAX
{
		[PARAMETER	&ParameterType]
		[CODE		&errorCode]
}

Code ::= CHOICE
{
		localValue		INTEGER.
		GlobalValue		OBJECT IDENTIFIER
}

Figure 3/Q.754 (sheet 1 of 11) – Formal syntax of OMASE services

-- OMASE operators --

eventReport OPERATION ::=
{
PARAMETER eventReportArgument		EventReportArgument

CODE	localValue:0
}

confirmedAction OPERATION ::=
{
PARAMETER actionArgument		ActionArgument

RESULT 	actionResult		ActionResult
ERRORS	{ accessDenied | invalidArgumentValue |
		noSuchAction | noSuchArgument |
		noSuchObjectClass | noSuchObjectInstance |
		processingFailure }

CODE	localValue:7
}

Figure 3/Q.754 (sheet 2 of 11) – Formal syntax of OMASE services

-- The om-service error definitions are as follows:

noSuchObjectClass		ERROR ::=
	{
	PARAMETER	ObjectClass
	CODE		localValue:0
	}

noSuchObjectInstance	ERROR ::=
	{
	PARAMETER	ObjectInstance
	CODE		localValue:1
	}

accessDenied			ERROR ::=
	{
	CODE		localValue:2
	}

noSuchAction		ERROR ::=
	{
	PARAMETER	NoSuchAction
	CODE		localValue:9
	}

processingFailure		ERROR ::=
	{
	PARAMETER	ProcessingFailure -- optional --
	CODE		localValue:10
	}

noSuchArgument		ERROR ::=
	{
	PARAMETER	NoSuchArgument
	CODE		localValue:14
	}

invalidArgumentValue	ERROR ::=
	{
	PARAMETER	InvalidArgumentValue
	CODE		localValue:15
	}

Figure 3/Q.754 (sheet 3 of 11) – Formal syntax of OMASE services

-- The following gives the supporting type definitions: --

ActionArgument		::= SEQUENCE	{
					COMPONENTS OF		BaseManagedObjectId,
					accessControl			[5] AccessControl OPTIONAL,
					actionInfo				[12] IMPLICIT ActionInfo }

ActionInfo			::= SEQUENCE	{
					actionType		[3] IMPLICIT CNF-ACTION.&operationCode,
					actionInfoArg	[4] CNF-ACTION. &ActionArgType(@actionType)
								OPTIONAL}

ActionResult			::= SEQUENCE	{
					managedObjectClass		ObjectClass OPTIONAL,
					managedObjectInstance		ObjectInstance OPTIONAL,
					currentTime				[5] IMPLICIT GeneralizedTime OPTIONAL,
					actionReply				[6] IMPLICIT ActionReply OPTIONAL }

ActionTypeId		::= CHOICE		{
					-- globalForm... --
					localForm				[3] IMPLICIT CNF-ACTION }

BaseManagedObjectId	::= SEQUENCE	{
					baseManagedObjectClass		ObjectClass,
					baseManagedObjectInstance	ObjectInstance }

EventReportArgument	::= SEQUENCE	{
					managedObjectClass		ObjectClass,
					managedObjectInstance		ObjectInstance,
					eventTime				[5] IMPLICIT GeneralizedTime OPTIONAL,
					eventType				[7] IMPLICIT EVENT. &operationCode
					eventInfo				[8] EVENT. &EventInfo Type (@eventType)
										OPTIONAL}

EventTypeId			::= CHOICE		{
					-- globalForm... --
					localForm				[7] IMPLICIT EVENT }

ActionReply			::= SEQUENCE	{
					actionType				[3] IMPLICIT CNF-ACTION.
										&operationCode,
					actionReplyInfo			[4]
								CNF-ACTION.&ActionResultType(@actionType)
					}

AccessControl		::= EXTERNAL
-- AccessControl syntax is to be compatible with that defined in CMIP coded X.209 --

Figure 3/Q.754 (sheet 4 of 11) – Formal syntax of OMASE services

InvalidArgumentValue	::= CHOICE {
					actionValue		[0] IMPLICIT ActionInfo,
					eventValue		[1] IMPLICIT SEQUENCE {
					eventType		[7] IMPLICIT EVENT.&operationCode,
					eventInfo		[8] EVENT.&EventInfoType(@eventType)
									OPTIONAL }}

NoSuchAction		::= SEQUENCE	{ managedObjectClass	ObjectClass,
							actionType				ActionTypeId }

NoSuchArgument		::= CHOICE		{
					actionId						[0] IMPLICIT SEQUENCE {
						managedObjectClass			ObjectClass OPTIONAL,
						actionType					ActionTypeId },
					eventId						[1] IMPLICIT SEQUENCE {
						managedObjectClass			ObjectClass OPTIONAL,
						eventType					EventTypeID }}

ObjectClass			::= CHOICE		{
					globalForm						[0] IMPLICIT OBJECT
												IDENTIFIER,
					-- ... --
					}

ObjectInstance		::= CHOICE		{
					-- ... --
					nonSpecificForm					[3] IMPLICIT OCTET STRING,
					-- ... --
					}

ProcessingFailure		::= SEQUENCE	{
					managedObjectClass				ObjectClass OPTIONAL,
					managedObjectInstance				ObjectInstance OPTIONAL,
					specificErrorInfo					[5] IMPLICIT SpecificErrorInfo }

SpecificError		::= INTEGER	-- defined by object class --

SpecificErrorInfo		::= SEQUENCE	{
					errorType				[0] IMPLICIT SpecificError.&errorCode,
					errorParm				[1] IMPLICIT SPECIFIC-ERROR. &ProcessingError
										ParmType(@errorType) OPTIONAL }

Timer				::= INTEGER	-- seconds –

Figure 3/Q.754 (sheet 5 of 11) – Formal syntax of OMASE services

-- Specific event reports are categorized by object class. The protocol uses may be described

-- by the EVENT MACRO below. --

EVENT ::= CLASS

{
		&EventInfoType		OPTIONAL,

&operationCode		INTEGER UNIQUE OPTIONAL

}
	WITH SYNTAX
	{
		[EVENTINFO		&EventInfoType]
		[CODE			&operationCode]
	}

-- Specific Actions are categorized by object class. The protocol uses may be described

-- by the CNF-ACTION INFORMATION OBJECT below. --

CNF-ACTION ::= CLASS

{

&ActionArgType			OPTIONAL,

&ActionResultType		OPTIONAL,

&SpecificErrors			SPECIFIC-ERROR OPTIONAL,

&operationCode			INTEGER UNIQUE OPTIONAL

}
	WITH SYNTAX
	{
		[ACTIONARG			&ActionArgType]
		[ACTIONRESULT			&ActionResultType]
		[SPECIFICERRORS		&SpecificErrors]
		[CODE				&operationCode]
	}

-- Errors that are action or event specific are defined using the SPECIFIC-ERROR macro below. --

SPECIFIC-ERROR ::= CLASS

{

&ProcessingErrorParmType		OPTIONAL,

&errorCode					INTEGER UNIQUE OPTIONAL

}
		WITH SYNTAX
		{
			[PARAMETER		&ProcessingErrorParmType]
			[CODE			&errorCode]
		}

Figure 3/Q.754 (sheet 6 of 11) – Formal syntax of OMASE services

-- specific OMASE constructs follow --

testRoute CNF-ACTION ::=
	{
		ACTIONARG SEQUENCE{
						initiating SP			[0] IMPLICIT PointCode,
						traceRequested		[1] IMPLICIT BOOLEAN,
						threshold			[2] IMPLICIT INTEGER,
						pointCodesTraversed	[3] IMPLICIT PointCodeList,
						formIndicator		[4] IMPLICIT FormIndicator OPTIONAL,
-- formIndicator is required in SRVT, but not used in MRVT --

mtpBackwardRoutingRequested
										[5] IMPLICIT BOOLEAN OPTIONAL,
-- mtpBackwardRoutingRequested is required in SRVT, but not in MRTV --

testInitiatorGT		[6] IMPLICIT GlobalTitle OPTIONAL,
						destinationPC		[7] IMPLICIT PointCode OPTIONAL,
						destinationSSN		[8] IMPLICIT SubsystemNumber 												OPTIONAL,
						backupDPC			[9] IMPLICIT PointCode OPTIONAL,
						backupSSN			[10] IMPLICIT SubsystemNumber 											OPTIONAL,
						originalGT			[11] IMPLICIT GlobalTitle OPTIONAL,
						inputGT			[16] IMPLICIT GlobalTitle OPTIONAL,
-- parameters with tags 4 through 12 can only be used in SRVT, not MRVT --

routePriorityList		[12] IMPLICIT RoutePriorityList
												OPTIONAL,
-- routePriorityList can only be used in MRVT, and only if the infoRequest parameter is present. --

infoRequest			[13] IMPLICIT BIT STRING {
										pointCode(0),
										pointCodeList(1),
										routePriorityList(2),
										currentRouteIndicatorList(3)
										} (SIZE(32)) OPTIONAL,
-- infoRequest is used to indicate that the test initiator node can accept a route TraceNew
-- RVR message, and also asks for particular parameters to be returned in it, if it is sent. This
-- parameter can only be inserted at the initiator node, but it can be copied into regenerated MRVTs. --

returnUnknownParams	[14] IMPLICIT BIT STRING {
										tag15(0),
										tag16(1),

tag17(2)
										} (SIZE(32)) OPTIONAL,
-- returnUnknownParams is used to indicate which parameters that a node does not understand
-- should be returned in an RVR if one is sent (or in an RVA message in the copyData field
-- if the test initiator is unknown). Bit 0 represents an RVT parameter with tag value 15, bit 1
-- an RVT parameter with tag value 16, etc. To avoid confusion in the copyData field, when
-- defining a new parameter in the RVR message, the tag should have the same value as it has
-- in the RVT message. This parameter can only be present if infoRequest is present. --

directRouteCheck		[15] IMPLICIT BOOLEAN OPTIONAL,
-- directRouteCheck can only be used in MRVT. --
						currentRouteIndicatorList [17] IMPLICIT BOOLEAN OPTIONAL,
-- currentRouteIndicatorList can only be used in MRVT. --

… }

Figure 3/Q.754 (sheet 7 of 11) – Formal syntax of OMASE services

SPECIFICERRORS		{ failure | partialSuccess }

CODE				1

}

-- TC timer = T1 for MRVT, = T2 for SRVT, Class = 1--

CurrentRouteIndicatorList::= SEQUENCE OF CurrentRouteIndicator

CurrentRouteIndicator ::= INTEGER{
				unknown 		(0),
				currentRoute 	(1),
				possibleRouteNoFurtherAnalysisPerformed 	(2),

possibleRouteSignRouteSetLocked	(3),
possibleRouteLinkSetUnavailable	(4),

possibleRouteTfpReceived	(5),

possibleRouteHigherpriorityRouteAvailable	(6)
								…} (0..255)

PointCode ::= OCTET STRING

PointCodeList ::= SEQUENCE OF PointCode

RoutePriorityList ::= SEQUENCE OF Priority

Priority ::= INTEGER{

unknown(0),
				firstChoice(1),
				secondChoice(2),
				thirdChoice(3),
				…} (0..255)

FormIndicator ::= INTEGER
		{ compare (0),
		noCompare (1) } (0..1)

GlobalTitle ::= OCTET STRING
		-- the GlobalTitle here consists of the SCCP GTI + GT, the GTI should be encoded exactly as in
		-- 3.4.1/Q.713, and the GT as in 3.4.2.1 to 3.4.2.4/Q.713 as appropriate. --

SubsystemNumber ::= OCTET STRING

failure		SPECIFIC-ERROR ::=
		{
		PARAMETER SEQUENCE		{failureType		[0] IMPLICIT FailureString,
								traceSent		[1] IMPLICIT BOOLEAN,
								copyData		[2] IMPLICIT CopyData OPTIONAL,
		-- copyData might be present if failureType is unknownInitiatingSp, traceSent is FALSE,
		-- and the prompting RVT message contained a requestInfo parameter, or returnUnknownParams
		-- was in the RVT message. --

… }
		CODE	1
		}

Figure 3/Q.754 (sheet 8 of 11) – Formal syntax of OMASE services

FailureString ::= BIT STRING
			{  detectedLoop(0),
			excessiveLengthRoute(1),
			unknownDestination(2),
			routeInaccessible(3),
			processingFailure(4),
			unknownInitiatingSP(5),
			timerExpired(6),
			sPNotAnSTP(7),
		-- wrongSp is a synonym, used in SRVT, of sPNotAnSTP. --

incorrectTranslation-Primary (8),
			incorrectTranslation-Secondary (9),
			incorrectTranslation-Intermediate (10),
			notPrimaryDestination (11),
			notSecondaryDestination (12),
			notRecognizedPrimary (13),
			notRecognizedSecondary (14),
			routingProblem (15),
		-- bits 8 through 15 might only be set in SRVT, not MRVT. --

maxNrMRVTestsAlready(16),
		-- maxNrSRVTestsAlready is a synonym, used in SRVT, of maxNrMRVTestsAlready. --

indirectRoute(17),

-- indirectRoute might only be set in MRVT, not SRVT. --

unknownParameter(18)
		} (SIZE(32))

CopyData ::= OCTET STRING

partialSuccess	SPECIFIC-ERROR ::=
	{
		PARAMETER SEQUENCE	{failureType		[0] IMPLICIT FailureString,
							traceSent		[1] IMPLICIT BOOLEAN,
							copyData		[4] IMPLICIT CopyData OPTIONAL,

-- copyData might be present if failureType is unknownInitiatingSP, traceSent is FALSE,
	-- and the prompting RVT message contained a requestInfo parameter,
	-- or returnUnknownParams was in the RVT message. --

… }
			CODE	2
	}

Figure 3/Q.754 (sheet 9 of 11) – Formal syntax of OMASE services

routeTrace		EVENT ::=
		{
		EVENTINFO CHOICE {
				success			[0] IMPLICIT PointCodeList,
				detectedLoop		[1] IMPLICIT PointCodeList,
				excessiveLengthRoute	[2] IMPLICIT PointCodeList,
				unknownDestination	[3] IMPLICIT NULL,
				routeInaccessible		[4] IMPLICIT PointCode,
				processingFailure		[5] IMPLICIT NULL,
				unknownInitiatingSP	[6] IMPLICIT PointCode,
				timerExpired		[7] IMPLICIT PointCodeList,
				sPNotAnSTP			[8] IMPLICIT PointCodeList,
		-- wrongSP is a synonym, used in SRVT, for sPNotAnSTP. --

incorrectTranslation-Primary		[9] IMPLICIT PointCodeList,
		incorrectTranslation-Secondary		[10] IMPLICIT PointCodeList,
		incorrectTranslation-Intermediate	[11] IMPLICIT PointCodeList,
		notPrimaryDestination			[12] IMPLICIT PointCodeList,
		notSecondaryDestination			[13] IMPLICIT PointCodeList,
		notRecognizedPrimary			[14] IMPLICIT PointCodeList,
		notRecognizedSecondary			[15] IMPLICIT PointCodeList,
			routingProblem			[16] IMPLICIT PointCodeList
		-- the choices with tags 9 through 16 can only be used in SRVT. --

}

CODE	2
		}

-- TC Timer = 0, Class = 4

routeTraceNew EVENT ::=
	{
		EVENTINFO SEQUENCE {
			result					[0] IMPLICIT ErrorTag,
			pointCode				[1] IMPLICIT PointCode OPTIONAL,
			pointCodeList			[2] IMPLICIT PointCodeList OPTIONAL,
			routePriorityList			[3] IMPLICIT RoutePriorityList OPTIONAL,
			copyData				[4] IMPLICIT CopyData OPTIONAL,

-- copyData allows any parameters included in an RVA message, when the
	-- test initiator is unknown, to be copied into the RVR, without enhancing it. It also
	-- allows new OPTIONAL RVT parameters not understood by the node generating the
	-- RVR from the RVA message to be returned, when requested by the test initiator.
	-- Note that a new parameter defined in routeTraceNew should, if it is also defined in
	-- testRoute, have the same tag value as in testRoute.
	-- One RVR message should be sent for each error detected (no error diagnostics should be
	-- "or'd" together). –

currentRouteIndicatorList	[5] IMPLICIT CurrentRouteIndicatorList OPTIONAL,

… }

CODE	4

}

-- TC Timer = 0, Class = 4

Figure 3/Q.754 (sheet 10 of 11) – Formal syntax of OMASE services

ErrorTag ::=INTEGER {
		success(0),
		detectedLoop(1),
		excessiveLengthRoute(2),
		unknownDestination(3),
		routeInaccessible(4)
		processingFailure(5)
		unknownInitiatingS(6),
		timerExpired(7),

-- wrongSP is a synonym, used in SRVT, of sPNotAnSTP. --

incorrectTranslation-Primary(9),
	incorrectTranslation-Secondary(10),
	incorrectTranslation-Intermediate(11),
	notPrimaryDestination(12),
	notSecondaryDestination(13),
	notRecognizedPrimary(14),
	notRecognizedSecondary(15),
		routingProblem(16)
	-- values 9 through 16 are applicable only in SRVT, not in MRVT. --

maxNrMRVTestsAlready(17),
	-- maxNrSRVTestsAlready is a synonym, used in SRVT, of maxNrMRVTestsAlready. --

indirectRoute(18),
	-- value 18 is applicable only in MRVT, not SRVT. --

unknownParameter(19),

… } (0..255)

END -- OMASE protocol –

Figure 3/Q.754 (sheet 11 of 11) – Formal syntax of OMASE services

____________