| Agenda item 1.13 - proposed modification to footnote No. S5.502 INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 56-E |
|  |  |  | 30 March 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | PLENARY MEETING |
| Australia |  |  |  |
| ProPOsal for the work of the Conference |  |  |  |

# 1	Introduction

The band 13.75-14 GHz is allocated on a co-primary basis to the fixed-satellite service and the radiolocation service.

Radio Regulation No. S5.502, prescribes sharing conditions based on a minimum e.i.r.p. requirement of +68 dBW for fixed-satellite service earth stations coupled with the maximum e.i.r.p. restriction on the radiolocation service.

Some systems operating in the fixed-satellite service may be able to share successfully with the radiolocation service whilst using an e.i.r.p. of less than the +68 dBW required by No. S5.502. In order to facilitate this sharing and yet ensure that the radiolocation service is protected against claims by the fixed-satellite service that interference is caused by the radiolocation service it is proposed:

•	to suppress the minimum e.i.r.p. restriction prescribed in No. S5.502, but

•	to retain the limitation of the minimum antenna diameter, and

•	to prescribe that systems operating in the fixed-satellite service with an e.i.r.p. of less than +68 dBW be unable to claim protection from interference by the radiolocation service.

# 2	Proposal

Australia proposes the following specific modification of No. S5.502:

MOD	AUS/56/1

S5.502	In the band 13.75-14 GHz, the e.i.r.p. of any emission from an earth station in the fixed-satellite service shallshould be at least 68 dBW, and should not exceed 85 dBW,. The earth station shall transmit with a minimum antenna diameter of 4.5 m. The fixed-satellite service shall not claim protection against harmful interference caused by the radiolocation service where the fixed-satellite service uses an e.i.r.p. of less than +68 dBW. In addition the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services towards the geostationary-satellite orbit shall not exceed 59 dBW.

Reasons:	A minimum e.i.r.p. restriction is overly prescriptive for some fixed-satellite service systems and its removal could lead to more efficient frequency spectrum utilization. However the radiolocation service requires ongoing protection from claims of interference to systems in the fixed-satellite service that use less than +68 dBW. Australia believes that the above proposal could satisfy the requirement of all the services allocated to this frequency band. Australia also believes that further studies are required before any further changes are made to the sharing criteria.

__________