| 1 Modifications to Article S5 INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 307(Rev.1)-E |
|  |  |  | 24 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | WORKING GROUP 5A |
| Chairperson, Sub-Working Group 5A-1 |  |  |  |
| CONCLUSIONS RELATING TO AGENDA ITEM 1.6.1 ON THE USE OF HAPS IN IMT-2000 |  |  |  |

MOD

1 710-2 170 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 710-1 930 FIXED MOBILE S5.380 ADD S5.BBB S5.149 S5.341 S5.385 S5.386 S5.387 S5.388 |  |  |
| 1 930-1 970 FIXED MOBILE ADD S5.BBB | 1 930-1 970 FIXED MOBILE ADD S5.BBB Mobile-satellite (Earth-to-space) | 1 930-1 970 FIXED MOBILE ADD S5.BBB |
| S5.388 | S5.388 | S5.388 |
| 1 970-1 980 FIXED MOBILE ADD S5.BBB S5.388 |  |  |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) S5.388 S5.389A S5.389B S5.389F |  |  |
| 2 010-2 025 FIXED MOBILE ADD S5.BBB | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) | 2 010-2 025 FIXED MOBILE ADD S5.BBB |
| S5.388 | S5.388 S5.389C S5.389D S5.389E S5.390 | S5.388 |

| 2 025-2 110 SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE S5.391 SPACE RESEARCH (Earth-to-space) (space-to-space) S5.392 |  |  |
| --- | --- | --- |
| 2 110-2 120 FIXED MOBILE ADD S5.BBB SPACE RESEARCH (deep space) (Earth-to-space) S5.388 |  |  |
| 2 120-2 160 FIXED MOBILE ADD S5.BBB | 2 120-2 160 FIXED MOBILE ADD S5.BBB Mobile-satellite (space-to-Earth) | 2 120-2 160 FIXED MOBILE ADD S5.BBB |
| S5.388 | S5.388 | S5.388 |
| 2 160-2 170 FIXED MOBILE ADD S5.BBB | 2 160-2 170 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) | 2 160-2 170 FIXED MOBILE ADD S5.BBB |
| S5.388 S5.392A | S5.388 S5.389C S5.389D S5.389E S5.390 | S5.388 |

ADD

S5.BBB	In Regions 1 and 3, the bands 1 885-1 980 MHz , 2 010-2 025 MHz and 2 110-2 170 MHz, and in Region 2, the bands 1 885-1 980 and 2 110-2 160 MHz, may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications 2000 (IMT-2000) [in accordance with Resolution [COM5/13]]. [(See Resolution [COM5/13].)] [Such use shall comply with the provisions of Resolution [COM5/13].] [These bands are allocated to the fixed, mobile and mobile-satellite services, and the use by IMT-2000 applications using high altitude platform stations as an IMT-2000 base station in these bands is based on the equality of rights between all allocated radio services and does not establish priority of assignments in these bands among stations of the primary services to which they are allocated.]

# 2	Modifications to Article S11

ADD

S11.8A	g)	if that assignment is to be used for a high altitude platform station.

# 3	New resolution

ADD

RESOLUTION  [COM5/13]  (WRC-2000)

Use of high altitude platform stations providing IMT-2000 in the bands 
1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 
and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2

The World Radiocommunication Conference 2000 (Istanbul, 2000),

considering

a)	that the bands 1 885-2 025 MHz and 2 110-2 200 MHz, are identified in No. S5.388 as intended for use on a worldwide basis for IMT-2000, including the bands 1 980-2 010 MHz and 2 170-2 200 MHz for the satellite component of IMT-2000;

b)	that a high altitude platform station is defined in No. S1.66A as “a station located on an object at an altitude of 20 to 50 km and at a specified, nominal, fixed point relative to the Earth”;

- that high altitude platform stations may offer a new means of providing IMT-2000 services with minimal network build out as it is capable of providing service to a large footprint together with a dense coverage;

- that the use of high altitude platform stations as base stations of terrestrial IMT-2000 is optional for administrations and that such use should not have any priority over other terrestrial IMT-2000 use;

- that, in accordance with [No. MOD S5.388 and Resolution IMT (WRC-2000)] [No. S5.388 and Resolution 212 (Rev.WRC-97)], administrations may use the bands identified for IMT-2000, including the bands noted herein, for stations of other primary services to which they were allocated;

- that these bands are allocated to the fixed and mobile services on a co-primary basis;

- that ITU-R has studied sharing and coordination between high altitude platform stations and other stations within IMT-2000, has considered compatibility of high altitude platform stations within IMT-2000 with some services allocated in the adjacent bands, and has established Recommendation ITU-R M.1456;

- that ITU-R did not address sharing and coordination between high altitude platform stations and some existing systems, particularly PCS (Personal Communications Service), MMDS (Multichannel Multipoint Distribution Service), and systems in the fixed service, which are currently operating in some administrations in the bands 1 885-2 025 MHz and 2 110-2 200 MHz;

i)	that in accordance with No. S5.BBB, high altitude platform stations may be used as base stations of terrestrial IMT-2000 in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2 the use by high altitude platform stations as an IMT-2000 base station in these bands is based on the equality of rights between all allocated radio services and does not establish priority of assignments in these bands among stations of the primary services to which they are allocated,

recognizing

that the value in resolves 1 may not be appropriate for the protection of some stations operating in these bands in the fixed and mobile services,

resolves

1	that:

a)	for the purpose of protecting certain stations operating within IMT-2000 in neighbouring administrations from co-channel interference, a high altitude platform station operating as a base station to provide IMT-2000 shall not exceed a provisional value of co-channel power flux-density (pfd) level –121.5 dB (W/(m2/MHz)) on the Earth’s surface outside an administration’s borders unless agreed otherwise with the affected neighbouring administration;

b)	a high altitude platform station operating as a base station to provide IMT-2000, in order to protect fixed stations from interference, shall not exceed a provisional value of out-of-band pfd level on the Earth’s surface in the bands 2 025-2 110 MHz of:

–	–165 dB(W/(m2/MHz)) for angles of arrival (q) less than 5° above the horizontal plane;

–	–165 + 1.75 (q – 5) dB (W/(m2/MHz)) for angles of arrival between 5° and 25° above the horizontal plane; and

–	–130 dB(W/(m2/MHz)) for angles of arrival between 25° and 90° above the horizontal plane,

2	that such a high altitude platform station shall, as of the end of WRC-03, operate only in accordance with such limits as are confirmed or, if appropriate, revised by WRC-03, irrespective of the date of bringing into use;

3	that administrations wishing to implement high altitude platform stations within a terrestrial IMT-2000 system shall conform with the following:

a)	that for the purpose of protecting certain stations operating within ITM-2000 in neighbouring administrations from co-channel interference, administrations using high altitude platform stations as base stations to IMT-2000 shall use antennas that comply with the following antenna pattern:

G(y) = Gm - 3(y/yb)2 		dBi	for		0° £ y £ y1

G(y) = Gm + LN			dBi	for		y1 < y £ y2

G(y) = X - 60log (y)		dBi	for		y2 < y £ y3

G(y) = LF				dBi	for		y3 < y £ 90°

where:

G(y):	gain at the angle y from the main beam direction (dBi)

Gm:	maximum gain in the main lobe (dBi)

yb: 	one-half of the 3 dB beamwidth in the plane of interest (3 dB below Gm) (degrees)

LN: 	near-in-side-lobe-level in dB relative to the peak gain required by the system design, and has a maximum value of –25 dB

LF:	Gm - 73 dBi far side-lobe level (dBi)

y1 = yb −LN3	degrees

y2 = 3.745 yb    	degrees

X = Gm + LN + 60log (y2)             dB

y3 = 10(X−LF)60                          degrees

The 3 dB beamwidth (2yb) is again estimated by:

(yb)2 = 7442/(100.1Gm) (in degrees2)

where Gm is the peak aperture gain (dBi);

b)	that a high altitude platform station operating as a base station to provide IMT-2000, in order to protect mobile earth stations of the satellite component of IMT-2000 from interference, shall not exceed an out-of-band pfd level of –165 dB (W/(m2/4 kHz)) on the Earth’s surface in the bands 2 160-2 200 MHz in Region 2 and 2 170-2 200 MHz in Regions 1 and 3;

4	that administrations wishing to implement high altitude platform stations within a terrestrial IMT-2000 system shall, prior to notification under Article S11, take into account in their bilateral coordination with administrations of neighbouring countries, the operation and growth of existing and planned systems in the fixed and the mobile service allocated on a primary basis;

5	that administrations wishing to implement high altitude platform stations within a terrestrial IMT-2000 system shall, pending the review by WRC-02/03 of the studies mentioned below, for the purpose of protecting fixed service stations operating in neighbouring administrations from co-channel interference, take full account of the relevant ITU-R Recommendations relating to protection values for fixed stations (see Recommendation ITU-R F.758),

invites ITU-R

to complete, as a matter of urgency, additional studies of high altitude platform stations sharing criteria with, between and into other systems in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 in Region 2, and in adjacent bands, and to report on the results of these studies on time for consideration of WRC-02/03 to allow revision of the values in resolves 1.

___________