| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-2000 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 399-E |
|  |  |  | 26 May 2000 |
|  |  |  | Original: English |
| ISTANBUL, 8 MAY – 2 JUNE 2000 |  |  |  |
|  |  |  | COMMITTEE 4 |
| Algeria (People's Democratic Republic of), Saudi Arabia (Kingdom of), Bahrain (State of), Bulgaria (Republic of), United Arab Emirates, Jordan (Hashemite Kingdom of), Lao People's Democratic Republic, Lebanon, Libya (Socialist People's Libyan Arab Jamahiriya), Malaysia, Morocco (Kingdom of), Oman (Sultanate of), Qatar (State of), Syrian Arab Republic, Tonga (Kingdom of), Viet Nam (Socialist Republic of), Yemen (Republic of) |  |  |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |  |  |
| ISSUES RELATED TO THE SHARING SITUATION IN THE FREQUENCY BAND 13.75-14 GHz |  |  |  |

MOD	ALG/ARS/BHR/BUL/UAE/JOR/LAO/LBN/LBY/MLA/MRC/OMA/QAT/SYR/TON/
VTN/YEM/399/1

S5.502	In the band 13.75-14 GHz, the earth station in the fixed-satellite service shall1 have a minimum antenna diameter of 4.5 m and the e.i.r.p. of any emission should be at least 68 dBW and should not exceed 85 dBW. In addition the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW. The protection of assignments to receiving space stations in the fixed-satellite service operating with earth stations that, individually, have an e.i.r.p. of less than 68 dBW shall not impose constraints on the operation of the radiolocation and radionavigation stations operating in accordance with the Radio Regulations. S5.43 does not apply. See Resolution [COM5/10].

ADD	ALG/ARS/BHR/BUL/UAE/JOR/LAO/LBN/LBY/MLA/MRC/OMA/QAT/SYR/TON/
VTN/YEM/399/2

_______________

1	S5.502.1	In the band 13.75-14 GHz, administrations may authorize operation of earth stations in the fixed-satellite service with antenna diameter of less than 4.5 m, subject to ensuring that such operation shall not cause unacceptable interference to co-frequency radiolocation or radionavigation services operating in the territories of neighbouring administrations.

Reasons:	To allow administrations to make flexible and efficient use of the band 13.75-14 GHz, through the use of fixed-satellite service earth stations of diameter less than 4.5 m, whilst ensuring the protection of co-primary services.

__________