MOD	IAP/5/127

5.537A	In Bhutan, Indonesia, Iran (Islamic Republic of), Japan, Maldives, Mongolia, Myanmar, Pakistan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 27.5-28.35 GHz may also be used by high altitude platform stations (HAPS). The use of HAPS within the band 27.5-28.35 GHz is limited within the territory of each administration to a single 300 MHz sub-band. Such use of 300 MHz of the fixed service allocation by HAPS in the above countries is further limited to operation in the HAPS-to-ground direction and shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems or other co-primary services. See Resolution [HAPS 28-31] (WRC-03).     (WRC-03)

Reasons:	The identification of 300 MHz of spectrum within the band 27.5-28.35 GHz, along with the 300 MHz at 31-31.3 GHz, is intended to be an alternative for the 2  300 MHz that is problematic in the specified countries due to excessive rain attenuation at 47 GHz. This proposal implements that intent by limiting use of HAPS in the named administrations to 300 MHz within the identified 850 MHz frequency range identified at WRC-2000. The second 300 MHz band is found at 31-31.3 GHz (see IAP/5/115 below). This proposal also incorporates the provisions of new Resolution HAPS 28-31 (WRC-03) into the Radio Regulations.