MOD	IAP/5/128

5.543A	In Bhutan, Indonesia, Iran (Islamic Republic of), Japan, Maldives, Mongolia, Myanmar, Pakistan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 31-31.3 GHz may also be used by high altitude platform stations (HAPS) in the ground-to-HAPS direction. The use of the band 31-31.3 GHz by systems using HAPS shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems or other co-primary services, taking into account No. 5.545. The use of HAPS in the band 31-31.3 GHz shall not cause harmful interference to the passive services having a primary allocation in the band 31.3-31.8 GHz, taking into account the interference criteria given in Recommendations ITU-R SA.1029 and ITU-R RA.769 and the unwanted emission levels given in Recommendation ITU-R F.1570. See Resolution [HAPS28-31] (WRC-03).     (WRC-03)

Reasons:	ITU-R studies conducted to date have demonstrated that certain HAPS system designs, operating with certain constraints, could operate on a non-interference basis and appropriately protect other systems and services. In addition, these studies have shown that HAPS can operate without the need for claiming protection. Given that HAPS applications in the FS and the adjacent services are of different status and operate in different bands, the results of studies should remain within the ITU-R and not result in specific limits within No. 5.543A. There is, however, a need to modify No. 5.543A to incorporate new Resolution [HAPS 28-31] (WRC-03) into the Radio Regulations. ITU-R has agreed to the interference levels for passive services in the referenced Recommendation.