MOD	IAP/5/144

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) m) p) | 4 172.5 to 4 181.5 18 f. 0.5 kHz | 6 263 to 6 275.5 25 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 181.75 | 6 275.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 280.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) m) p) |  | 6 281 to 6 284.5 8 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) h) p) | 4 187 to 4 202 31 f. 0.5 kHz | 6 285 to 6 300 31 f. 0.5 kHz | 8 342 to 8 365.5 48 f. 0.5 kHz | 12 422 to 12 476.5 110 f. 0.5 kHz | 16 619 to 16 683 129 f. 0.5 kHz |  | 22 242 to 22 279 75 f. 0.5 kHz | 25 161.5 to 25 171 20 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 365.75 | 12 476.75 | 16 683.25 | 18 870 | 22 279.25 | 25 171.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 370.75 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) p) |  |  | 8 371 to 8 376 11 f. 0.5 kHz |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 376.25 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) m) p) |  |  | 8 376.5 to 8 396 40 f. 0.5 kHz | 12 477 to 12 549.5 146 f. 0.5 kHz | 16 683.5 to 16 733.5 101 f. 0.5 kHz | 18 870.5 to 18 892.5 45 f. 0.5 kHz | 22 284.5 to 22 351.5 135 f. 0.5 kHz | 25 173 to 25 192.5 40 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 549.75 | 16 733.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 554.75 | 16 738.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) m) p) |  |  |  | 12 555 to 12 559.5 10 f. 0.5 kHz | 16 739 to 16 784.5 92 f. 0.5 kHz |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (non paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK and for A1A or A1B Morse telegraphy (working) b) p) | 4 202.5 to 4 207 10 f. 0.5 kHz | 6 300.5 to 6 311.5 23 f. 0.5 kHz | 8 396.5 to 8 414 36 f. 0.5 kHz | 12 560 to 12 576.5 34 f. 0.5 kHz | 16 785 to 16 804 39 f. 0.5 kHz | 18 893 to 18 898 11 f. 0.5 kHz | 22 352 to 22 374 45 f. 0.5 kHz | 25 193 to 25 208 31 f. 0.5 kHz |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414.25 | 12 576.75 | 16 804.25 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) n) o) p) | 4 209.5 to 4 219 20 f. 0.5 kHz | 6 314 to 6 330.5 34 f. 0.5 kHz | 8 416.5 to 8 436 40 f. 0.5 kHz | 12 579 to 12 656.5 156 f. 0.5 kHz | 16 806.5 to 16 902.5 193 f. 0.5 kHz | 19 680.5 to 19 703 46 f. 0.5 kHz | 22 376 to 22 443.5 136 f. 0.5 kHz | 26 100.5 to 26 120.5 41 f. 0.5 kHz |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| ... |  |  |  |  |  |  |  |  |