MOD	IAP/5/147

5.443B	Additional allocation: The band 5 010-5 030 MHz is also allocated to the radionavigation-satellite service (space-to-Earth) (space-to-space) on a primary basis. In order not to cause harmful interference to the microwave landing system operating above 5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz by all the space stations within any radionavigation-satellite service system (space-to-Earth) operating in the band 5 010-5 030 MHz shall not exceed –124.5 dB(W/m2) in a 150 kHz band. In order not to cause interference to the radio astronomy service in the band 4 990-5 000 MHz,- RNSS systems operating in the band 5 010-5 030 MHz shall comply with the limits in the band 4 990-5 000 MHz defined in Resolution [RNSS 1.2.3-1] (WRC-03).     (WRC-03)

NOTE – The following Resolution is intended to clarify the application of the pfd/epfd limits, and to define the assumptions to be made for epfd calculations.