ADD	IAP/5/153

draft  resolution  [rnss-1]  (wrc-03)

Protection of aeronautical radionavigation service systems from the equivalent power flux-density produced by radionavigation-satellite service networks 
and systems in the 1 164-1 215 MHz frequency band

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the band 960-1 215 MHz is allocated on a primary basis to the aeronautical radionavigation service (ARNS) in all Regions;

b)	that the band 1 164-1 215 MHz is also allocated on a primary basis to the radionavigation-satellite service (RNSS), subject to the condition in No. 5.328A that operation of RNSS systems shall be in accordance with this Resolution;

c)	that this Conference has determined that protection of the ARNS from harmful interference can be achieved if the value of the equivalent power flux-density (epfd) produced by all the space stations of all RNSS (space-to-Earth) systems in the 1 164-1 215 MHz band does not exceed the level of –121.5 dB(W/m2) in any 1 MHz band;

d)	that WRC-2000 adopted Resolution 605 (WRC-2000) to provide for implementation of a provisional aggregate power flux-density limit during the period between WRC-2000 and WRC-03, and requested ITU-R studies on the need for an aggregate pfd limit, and revision, if necessary, of the provisional pfd limit given in No. 5.328A;

e)	that only a limited number of RNSS systems are expected to be deployed in the 1 164-1 215 MHz band, and only a few of these systems at most would have overlapping frequencies;

f)	that ARNS systems can be protected without placing undue constraints on the development and operation of RNSS systems in this band;

g)	that to achieve the objectives in considering f), administrations operating RNSS systems will need to agree cooperatively through consultation meetings to equitably share the aggregate epfd in a manner to achieve the level of protection for ARNS systems that is stated in considering c);

h)	that it may be appropriate for representatives of administrations operating ARNS systems to be involved in determinations made pursuant to considering g),

noting

a)	that WRC-2000 invited ITU-R to conduct, the appropriate technical, operational and regulatory studies on the overall compatibility between the radionavigation-satellite service and the aeronautical radionavigation service in the band 960-1 215 MHz;

b)	that WRC-2000 resolved to recommend that WRC-03 review the results of the studies,

resolves

1	that, in order to protect ARNS systems, administrations shall ensure, pursuant to this Resolution, that the equivalent pfd level produced by all space stations of all RNSS systems does not exceed the level –121.5 dB(W/m2) in any 1 MHz band;

2	that administrations operating or planning to operate in the 1 164-1 215 MHz frequency band RNSS systems or networks for which complete coordination or notification information, as appropriate, was received by Radiocommunication Bureau after 2 June 2000, in collaboration, shall take all necessary steps, including by means of appropriate modifications to their systems or networks, to ensure that the aggregate interference into ARNS systems caused by such RNSS systems or networks operating co-frequency in these frequency bands is shared equitably among the systems identified in resolves 3 and does not exceed the level of the aggregate protection criterion given in resolves 1 above;

3	that administrations, in carrying out their obligations under resolves 1 and 2 above, shall take into account only those RNSS systems with frequency assignments in the band 1 164-1 215 MHz that have met the milestones listed in the Annex to this Resolution through appropriate information provided to the consultation meetings referred to in considering g);

4	that no single RNSS system shall be permitted to use up the entire interference allowance specified in resolves 1 above in any 1 MHz of the 1 164-1 215 MHz band;

5	that administrations shall communicate to the Bureau the results of any aggregate sharing determinations made in application of resolves 2 above, without regard to whether such determinations result in any modifications to the published characteristics of their respective systems or networks;

6	that administrations operating ARNS systems in the 1 164-1 215 MHz band should participate, as appropriate, in discussions and determinations relating to the resolves above;

7	that the methodology contained in draft new Recommendations ITU-R M.[RNSS2] (8D/TEMP/185(Rev.1)) shall be used by administrations for calculating the aggregate epfd produced by all the space stations within all RNSS systems in the band 1 164-1 215 MHz,

urges administrations

to deal with RNSS inter-system matters as required, as early as possible.

ANNEX  TO  draft  RESOLUTION  [RNSS-1]  (WRC-03)

Milestone criteria for application of Resolution RNSS

1	Submission of appropriate ITU Advance Publication, and Coordination or Notification documentation.

2	Entry into satellite manufacturing or procurement agreement, and entry into satellite launch agreement:

–	The RNSS system or network operator should possess: i) clear evidence of a binding agreement for the manufacture or procurement of its satellites, and ii) clear evidence of a binding agreement to launch its satellites. The manufacturing or procurement agreement should identify the contract milestones leading to the completion of manufacture or procurement of satellites required for the service provision, and the launch agreement should identify the launch date, launch site and launch service provider. The notifying administration is responsible for authenticating the evidence of agreement and providing such evidence to other interested administrations in furtherance of its obligations under this Resolution.

3	In lieu of satellite manufacturing and launch agreements, clear evidence of irreversible funding arrangements for the project would be accepted. The notifying administration is responsible for authenticating the evidence of the financial commitment and for providing such evidence to other interested administrations in furtherance of its obligations under this Resolution.

Reasons:	This Resolution and Annex, along with incorporating provisions in Articles 5 (MOD 5.328A) and 21 (ADD Section VI), provides the mechanism by which administrations operating or planning to operate RNSS systems, all of which also operate co-frequency ARNS systems, will undertake the responsibility for ensuring the protection of ARNS systems. The Resolution recognizes that there is a need for discussions between and among administrations operating RNSS systems to ensure compliance with the obligation to protect ARNS systems, and that such discussions may involve administrations operating ARNS systems. Resolution RNSS-1 thus provides a basis for managing the total aggregate interference caused to ARNS systems by real RNSS systems.