MOD	IAP/5/196

# 1	Threshold values for determining when coordination is required between transmitting space stations in the fixed-satellite service or the broadcasting-satellite service and a receiving space station in the feeder-link Plans in the frequency bands 17.3-18.1 GHz (Regions 1 and 3) and 17.3-17.8 GHz (Region 2)

With respect to § 7.1, Article 7, coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service with a receiving space station in a broadcasting-satellite service feeder link in the Regions 1 and 3 feeder-link Plan or the Region 2 feeder-link Plan is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite service feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of Ts / Ts corresponding to 6%. Ts / Ts is calculated in accordance with Case II of the method given in Appendix 8. For feeder-link assignments in the Region 2 Plan (RARC-Sat 83) which have not been subsequently modified through successful application of Article 4 of Appendix 30A, the value for Ts, the satellite system noise temperature, to be used in determining  Ts / Ts, shall be 600˚ K.

Reasons:	Harmonization of the interregional feeder-link T/T interference criteria by adopting a common value of 6%.