MOD	IAP/5/241

TABLE  21-4 (end)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –120 10 | –120 + 0.75( – 5) 10 |  | –105 10 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |