ADD	IAP/5/34

ANNEX  TO  draft  RECOMMENDATION  [DAB]  (WRC-03)

RF protection ratio values

1	In accordance with Resolution 517 (Rev.WRC-03) digital modulation may be used in any of the HF bands allocated exclusively to the broadcasting service. This accommodation has to be made with the appropriate amounts of protection given to both analogue and digital emissions. RF protection ratios are part of the overall regulation of these emissions. Their values appear in Table 1 in this Annex.

2	Table 1 consists of RF protection ratios for co-channel and adjacent channel conditions. The independent variable in Table 1 is the centre frequency separation in kHz of any pair of emissions, wanted versus unwanted. Table 1 provides the required relative protection ratios for the DRM mode (Mode B3) that will be used extensively for HF skywave broadcasting in 10 kHz channels. The ratio data are in decibels.

3	The digital modulation governing these protection ratios is that which appears in summary in Part C of Appendix 11, as revised at this Conference, and the analogue modulation is double-sideband modulation as summarized in Part A of the same Appendix.

Table 1

Relative RF protection ratios (dB) between broadcasting systems below 30 MHz, and digital (64-QAM, protection level No. 1) interfered with by digital (identical robustness
modes and spectrum occupancy types)

| Wanted signal | Unwanted signal | Frequency separation f unwanted – f wanted (kHz) |  |  |  |  |  |  |  |  | Parameters |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | BDRM (kHz) | S/I (dB) |
|  |  | –20 | –15 | –10 | –5 | 0 | 5 | 10 | 15 | 20 |  |  |
| AM | DRM_B3 | –47 | –42 | –32 | 3 | 6 | 3 | –32 | –42 | –47 | 10 | – |
| DRM_B3 | AM | –54 | –48 | –40 | –3 | 0 | –3 | –40 | –48 | –54 | 10 | 7 |
| DRM_B3 | DRM_B3 | –53 | –47 | –38 | –3 | 0 | –3 | –38 | –47 | –53 | 10 | 16 |
| AM: DSB AM signal. DRM_B3: DRM signal, robustness mode B, spectrum occupancy type 3. BDRM: Nominal bandwidth of DRM signal. S/I: Signal-to-interference ratio for a BER of 104. |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE – For more complete and accurate information draft new Recommendation ITU-R BS.[Doc. 6/324] should be used.

Reasons:	This Annex provides the appropriate amounts of protection given to both analogue and digital emissions and supports the text of draft Recommendation [DAB] (WRC-03).