MOD	IAP/5/44

Methodology and data

The software should use:

–	Recommendation ITU-R BS.705 for the calculation of antenna patterns;

–	Recommendation ITU-R P.533 for the prediction of wanted field-strength values;

–	Recommendation ITU-R P.842 for the calculation of reliability values.

The set of 911 test points (agreed at WARC HFBC-87) should be used, supplemented where necessary with test points based on a geographic grid.

The software should calculate the field strength values and the fading margins at each test point inside the required service area for each of the frequency bands declared to be available, taking account of the relevant transmitting antenna characteristics for each frequency band. The desired RF signal-to-noise ratio should be user selectable with a default value of 34 dB in the case of DSB and as provided in the current ITU-R Recommendation1 in the case of digital emissions.

The dates for which calculations are made should be user selectable, the default values being:

–	0.5 month after the start of the season;

–	mid-point of the season;

–	0.5 month before the end of the season.

The times for which calculations are made should be user selectable, the default values being:

–	30 min past the hour in which the requirement starts;

–	30 min past each successive hour until the hour in which the requirement stops.

DESCRIPTION  3

Specification of input data for a requirement

1	Draft new Recommendation ITU-R BS.[Doc. 6/324].