| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 11 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 11 |  |

Agenda item 1.11 – AMSS (Earth-to-space) secondary allocation at 14 GHz

Introduction

Agenda item 1.11 requests the conference to consider the possible extension of the allocation to the mobile-satellite service (MSS) (Earth-to-space) on a secondary basis in the band 14-14.5 GHz to permit operation of the aeronautical mobile-satellite service (AMSS).

All or parts of the band 14-14.5 GHz are allocated on a primary basis to the fixed-satellite service (Earth-to-space), radionavigation, fixed and mobile services (except aeronautical mobile service). Secondary services allocated in all or parts of the band include: mobile-satellite service (Earth-to-space) (except AMSS), space research, radio astronomy and radionavigation-satellite service.

MSS systems have been in operation for many years offering land and maritime mobile-satellite services with a proven ability to coexist with all other services having primary or secondary allocation in the same band. The AMSS in the 14-14.5 GHz band is being proposed to meet a growing demand for two-way broadband communications by passengers and operators of commercial aircraft.

Resolution 216 (Rev.WRC-2000) called for inter alia studies to assess the feasibility of sharing the band 14-14.5 GHz between primary services in the band and the AMSS, with the latter service on a secondary basis. Studies were performed with all services including those on a secondary basis (where operating parameters were available). These studies determined that these services will be adequately protected provided that the AMSS operates within a defined operating characteristics, as included in the DNR ITU-R M.[AMSS].

Europe supports the extension of the allocation to the MSS uplink on a secondary basis in the band 14.0-14.5 GHz for the aeronautical mobile-satellite service (Earth-to-space) provided that the technical and operational limitations specified in the DNR ITU-R M.[AMSS] are applied to the AMSS in order to provide adequate protection of the terrestrial, space services and the radioastronomy service in the band from harmful interference due to stations operating in the aeronautical mobile-satellite service.

In addition, Europe recognizes that some CEPT Administrations operate FS or radio astronomy systems in the 14.25-14.5 GHz band and accepts that some of these Administrations will propose to WRC-03, on a national basis, country footnotes that would specify the necessary protection in the RR as detailed in DNR ITU-R M.[AMSS] in order to make the technical and operational constraints imposed to AMSS stations that could impact the stations on their territory.

Europe therefore proposes:

1	To extend the secondary allocation to include the aeronautical mobile-satellite service (Earth-to-Space) in the 14-14.5 GHz band by removing the phrase “except aeronautical mobile-satellite” from the allocation.

2	To suppress Resolution 216 (WRC-2000).

Proposals

ARTICLE  5

Frequency allocations

MOD	EUR/13A11/1

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) Space research 5.505 |  |  |

Reasons:	Conclusion has been reached that the exception concerning the aeronautical mobile satellite is no longer needed.

MOD	EUR/13A11/2

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) Space research 5.505 5.508 5.509 |  |  |

| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radionavigation-satellite |
| --- | --- | --- |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radio astronomy 5.149 |  |  |

Reasons:	Conclusion has been reached that the exception concerning the aeronautical mobile satellite is no longer needed.

SUP	EUR/13A11/3

RESOLUTION  216  (Rev.WRC-2000)

Possible broadening of the secondary allocation to the mobile-satellite
service (Earth-to-space) in the band 14-14.5 GHz to
cover aeronautical applications

Reasons:	The studies called for in this Resolution have been completed.

_____________