MOD	EUR/13A13/6

RESOLUTION  122  (Rev.WRC-03)

Use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platform stations (HAPS) in the fixed service and by other services

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the band 47.2-50.2 GHz is allocated to the fixed, mobile and fixed-satellite services on a co-primary basis;

b)	that WRC-97 made provision for operation of HAPS, also known as stratospheric repeaters, within the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

c)	that ITU has among its purposes “to promote the extension of the benefits of the new telecommunication technologies to all the world’s inhabitants” (No. 6 of the Constitution);

d)	that systems based on new technologies using high altitude platforms will be able to provide high-capacity, competitive services to urban and rural areas;

e)	that the development of any service requires major investment and that manufacturers and operators should be given the confidence to make the necessary investment;

f)	that high altitude platform systems are in an advanced stage of development and some countries have notified such systems to ITU in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

g)	that WRC-97 adopted a definition of HAPS in Article 1, modified No. 11.24 and added No. 11.26 providing for notices relating to assignments for HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz and that the Radio Regulations Board issued a provisional rule of procedure concerning notification periods in No. 11.24/1228 in February 1997;

h)	that in spite of the urgency attached to the development of such systems, technical, sharing and regulatory issues should be further studied in order to achieve the most efficient use of the spectrum available for these systems;

i)	that while the decision to deploy HAPS can be taken on a national basis, such deployment may affect neighbouring administrations, particularly in small countries;

j)	that technical studies have been undertaken on the characteristics of a system using HAPS in the frequency bands 47.2-47.5 GHz and 47.9-48.2 GHz and on the coordination and sharing requirements between systems using HAPS and systems in the conventional fixed service, radio astronomy and in other services, but that further studies are still in progress on the potential for interference between such systems;

k)	that the radio astronomy service has primary allocations in the bands 42.5-43.5 GHz and 48.94-49.04 GHz;

l)	that results of ITU-R studies have been presented which indicate that in WRC-97 designated bands at 47.2-47.5 GHz and 47.9-48.2 GHz, sharing between fixed-service systems using HAPS and other conventional fixed-service systems in the same area will require appropriate interference mitigation techniques to be developed and implemented;

m)	that No. 5.552 urges administrations to reserve fixed-satellite service (FSS) use of the band 47.2-49.2 GHz for feeder links for the broadcasting-satellite service, and that ITU-R studies indicate that HAPS in the fixed service may share with broadcasting-satellite feeder links;

n)	that ITU-R studies in the bands 47.2-47.5 GHz and 47.9-48.2 GHz indicate that sharing between fixed-service systems using HAPS and the FSS could be feasible under certain limitations, such as geographical separation between HAPS-based systems and FSS earth stations,

resolves

1	to urge administrations to facilitate coordination between HAPS in the fixed service operating in the bands 47.2-47.5 GHz and 47.9-48.2 GHz and other co-primary services in their territory and adjacent territories;

2	that, on a provisional basis, the procedures of Article 9 shall be used for coordination between satellite systems and systems using HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz,

requests ITU-R

1	to study the regulatory provisions that might be needed in order to address those cases where the deployment of HAPS in the territory of one administration may affect neighbouring administrations;

2	to complete studies on the appropriate technical sharing criteria for the situations referred to in considering j) above,

--

instructs the Director of the Radiocommunication Bureau

1	that notices concerning HAPS that were received by the Bureau prior to 22 November 1997, and provisionally recorded in the Master International Frequency Register in accordance with the provisional rule of procedure issued by the Board, shall be maintained;

2	that from 22 November 1997, and pending review of the sharing studies in considering j) and review of the notification process by WRC-03, the Bureau shall accept notices in the bands 47.2-47.5 GHz and 47.9-48.2 GHz only for HAPS in the fixed service and for feeder links for the broadcasting-satellite service, shall continue to process notices for FSS networks (except for feeder links for the broadcasting-satellite service) for which complete information for advance publication has been received prior to 27 October 1997, and shall inform the notifying administrations accordingly.

Reasons:	The sharing studies in the 18-32 GHz range are considered completed and hence the references to this frequency range can be removed from the Resolution. The provisions relative to the 48 GHz band of instructs the Director of the Radiocommunication Bureau 2 of Resolution 122 should also be extended until WRC-07.