MOD	EUR/13A15/2

890-1 260 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 164-1 215 AERONAUTICAL RADIONAVIGATION 5.328 RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) MOD 5.328A ADD 5.XXX |  |  |

Reasons:	The term “additional allocation” is defined in No. 5.35 for allocations “in an area smaller than a Region or in a particular country”. Since the RNSS allocation in 1 164-1 215 MHz is worldwide, the term additional allocation is not adequate, and the allocation should appear clearly in the Table of Frequency Allocations. The modification to the Table of Frequency Allocations for RNSS is proposed on the basis that modification to RR 5.328A states: “Stations in the RNSS shall not claim protection from stations of the aeronautical radionavigation service in the band 960-1 215 MHz”.