MOD	EUR/13A2/13

DESCRIPTION  3

Specification of input data for a requirement

The fields needed for a given requirement and their specifications are:

–	frequency in kHz, up to 5-digit integer;

–	start time, as 4-digit integer;

–	stop time, as 4-digit integer;

–	target service area, as a set of up to 12 CIRAF zones and quadrants up to a maximum of 30 characters;

–	site code, a 3-character code from a list of codes, or a site name and its geographic coordinates;

–	power in kW, up to 4-digit integer;

–	azimuth of maximum radiation;

–	slew angle, up to 2-digit integer representing the difference between the azimuth of maximum radiation and the direction of unslewed radiation;

–	antenna code, up to 3-digit integer from a list of values, or a full antenna description, as given in Recommendation ITU-R BS.705;

–	days of operation;

–	start date, in the case that the requirement starts after the start of the schedule;

–	stop date, in the case that the requirement stops before the end of the schedule;

–	modulation choice, to specify if the requirement is to use double-side band (DSB) or single-side band (SSB) (see Recommendation ITU-R BS.640) or digital emission (see Recommendation ITU-R BS.1514). This field may be used to identify any other type of modulation when this has been defined for use by HFBC in an ITU-R Recommendation;

–	administration code;

–	broadcasting organization code;

–	identification number;

–	identification of synchronization with other requirements.

DESCRIPTION  4

Compatibility analysis