MOD	EUR/13A25/25

TABLE  22-4B     (WRC-03)

Operational limits to the epfd radiated by non-geostationary-satellite
systems in the fixed-satellite service in certain frequency bands20, 24

| Reasons: These changes are intended to quantify No. 22.2. Moreover, as far as Table 22-3 is concerned, the modification proposed also aims at protecting the feeder links operating in accordance with the Plans of Appendix 30A (see No. 22.5F.2). The extension of the limits to the 17.7-17.8 GHz band in Regions 1 and 3 aims at keeping the consistency in the whole 17.3-18.6 GHz band. With the new allocation of FSS (space-to-Earth), the regulatory situation with respect to FSS (space-to-Earth) and BSS would be identical in the whole 17.3-18.6 GHz band: there would be, then, no justification to have no limit only in the 17.7-17.8 GHz band. Frequency band (GHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Geostationary-satellite system receive earth station antenna gain (dBi) | Orbital inclination of geostationary satellite (degrees) |
| --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |
| 17.3-17.8 in Regions 1 and 3 | –164 –162 | 100 100 | 40 40 |  49  49 |  2.5 > 2.5 and  4.5 |
| 17.3-17.8 in Regions 1 and 3 | –150 –148 | 100 100 | 1 000 1 000 |  49  49 |  2.5 > 2.5 and  4.5 |