MOD	EUR/13A25/28

7.3	Administrations planning to implement assignments for receiving earth stations in all Regions in the band 17.3-18.1 GHz in the fixed-satellite service (space-to-Earth) or in the band 17.3-17.8 GHz in the broadcasting-satellite service should evaluate the level of interference, assessed on the basis of coordination contours calculated in accordance with Appendix 7, which might be caused by a feeder-link earth station located on the territory of another administration and included in the service area of an assignment to a broadcasting-satellite service feeder-link space station which is in conformity with the appropriate Regional feeder-link Plan. Should the administration planning receiving earth stations find that interference may be caused by such a feeder-link earth station, it may request the administration responsible for the feeder-link earth station to indicate the geographical coordinates, the antenna characteristics and the elevation angle of the horizon around its existing and planned feeder-link earth stations.

Reasons:	The current available procedures applicable to the 17.7-18.1 GHz band to BSS and FSS (space-to-Earth) have to be extended to the proposed allocation of the band 17.3-17.7 GHz to the FSS (space-to-Earth).