MOD	EUR/13A27/3

# 3	Limits to the change in the power flux-density to protect the broadcasting-satellite service in Regions 1 and 2 in the band 12.2-12.5 GHz and in Region 3 in the band 12.5-12.7 GHz

With respect to § 4.1.1 c) of Article 4, an administration in Region 2 is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in exceeding the power flux-densities given below, at any test point in the service area of its overlapping frequency assignments.

With respect to § 4.2.3 a), 4.2.3 b) or 4.2.3 f) of Article 4, as appropriate, an administration in Region 1 or 3 is considered as being affected if the proposed modification to the Region 2 Plan would result in exceeding the power flux-densities given below, at any test point in the service area of its overlapping frequency assignments.

–147 dB(W/(m2  27 MHz))	for  0° 	  < 0.23°

–135.7 + 17.74 log   dB(W/(m2  27 MHz))	for  0.23° 	  < 2.0°

–136.7 + 1.66 2 dB(W/(m2  27 MHz))	for  2.0°	  < 3.59°

–129.2 + 25 log dB(W/(m2  27 MHz))	for  3.59°	  < 10.57°

–103.6 dB(W/(m2  27 MHz))	for  10.57°	 

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

Reasons:	Europe considers that 60 cm should be taken as the minimum diameter to be protected in order to ensure a more efficient use of the spectrum/orbit resources and facilitate access to these resources by future users. Therefore, it is proposed to adopt the pfd mask given in Section 3.2.2.3 a) of the CPM Report.