MOD	EUR/13A30/31

TABLE  5-1     (WRC-2000)

Technical conditions for coordination
(see Article 9)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz 7 025-7 075 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) and any associated space operation functions with a space station within an orbital arc of  10° of the nominal orbital position of a proposed network in the FSS |  | With respect to the FSS or BSS in the bands in 1), 2) and 3), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used. With respect to the FSS or BSS in the bands in 1), 2) and 3), an administration may request, pursuant to No. 9.41, that an administration be |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS, or BSS, not subject to a Plan, and any associated space operation functions with a space station within an orbital arc of  9° of the nominal orbital position of a proposed network in the FSS or BSS not subject to a Plan |  |  |

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 3) - -All bands above 17.3 GHz 4) All frequency bands, other than those in § 1), 2) and 3), allocated to a space service, and the bands in § 1), 2) and 3) where the radio service of the proposed network or affected networks is other than the services mentioned in the threshold/condition column, or in the case of coordination of space stations operating in the opposite direction of transmission | i) Bandwidth overlap, and ii) any network in the FSS, or BSS, not subject to a Plan, and any associated space operation functions with a space station within an orbital arc of  8° of the nominal orbital position of a proposed network in the FSS or BSS not subject to a Plan Value of T/T exceeds 6% | Appendix 8 | excluded from requests for coordination, giving as reason that the network of this administration will not be affected because value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 do not exceed 6%. When the Bureau, at the request of an administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |