MOD	EUR/13A30/5

2	For the application of No. 9.21, the agreement of an administration may be required with respect to the frequency assignments in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the band is allocated with equal rights or a higher category of allocation, which may affect or be affected, as appropriate, and:

a)	in cases involving a station in a space radiocommunication service with respect to any other station or involving a terrestrial radiocommunication station with respect to an earth station or a space station:

i)	which are in conformity with No. 11.31, and comply with the relevant conditions listed in § 1 b) to 1 g); or

ii)	for which the procedure under No. 9.21 has been initiated, with effect from the date of receipt by the Bureau, in accordance with No. 9.34, of the basic characteristics specified in Appendix 4, pending examination under No. 11.31 of that assignment at the notification stage;

Reasons:

•	To cover the case of coordination of a terrestrial station with respect to a space station.

•	To clarify that 2 a) ii) applies to provide only provisional protection to an assignment for which No. 9.21 has been applied, until the agreements required have been examined under No. 11.31 at the notification stage.

Sub-part 30D

Protection of assignments recorded under No. 11.41

Proposal

APPENDIX  5  (WRC-2000)

Identification of administrations with which coordination is to be effected or agreement sought under the provisions of Article 9