MOD	EUR/13A31/14

9.17A	m)	for any specific earth station, in respect of other earth stations operating in the opposite direction of transmission or for any typical mobile earth station in respect of specific earth stations operating in the opposite direction of transmission, in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission and where the coordination area of the earth station includes the territory of another country or the earth station is located within the coordination area of another earth station, with the exception of the coordination under No. 9.19;     (WRC-03)

Reasons:	No. 9.17A provides for coordination between earth stations operating in opposite directions of transmission. However, No. 9.17A is currently limited to specific earth stations. For coordination of MESs (Earth-to-space) with respect to MetSat earth stations (space-to-Earth), it will be necessary to coordinate typical MESs.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section II  –  Power limits for terrestrial stations