ADD	EUR/13A32/2

5.551AAA	While addressing the sharing conditions with the fixed service in the bands 37.5-40 GHz and 40.5-42.5 GHz, the power flux-density at the Earth’s surface from any FSS satellite should be at the level(s) required to meet the FSS link availability and performance objectives of the subject applications. In any case, these levels shall not exceed the applicable power flux-density limits in Table 21-4.

Reasons:	Consequential to the ITU-R global agreement.