MOD	EUR/13A35/4

Col. 17	Remarks.

9A.2	TEXT  FOR  NOTES  IN  THE  REMARKS  COLUMN  OF  THE
	REGIONS  1  AND  3  FEEDER-LINK  PLAN

1	(Not used.)

2	(Not used.)

3	Before an administration notifies to the Bureau or brings into use this frequency assignment to a transmitting feeder-link earth station in the band 17.7-18.1 GHz, it shall effect coordination of this assignment, using the method described in Appendix 7, in respect of any assignment to a specific earth station in the fixed-satellite service (space-to-Earth) in the band 17.7-18.1 GHz (see also Note to § 9A.2) and which is:

a)	in conformity with No. 11.31, and

b)	either recorded in the Master Register prior to 3 June 2000 with a favourable finding; or

c)	for which a notice is received by the Bureau prior to 3 June 2000 for recording in the Master Register, but has not yet been processed, and which subsequently receives a favourable finding based on the Plan as it existed on 3 June 2000.

4	Before an administration notifies to the Bureau or brings into use this frequency assignment to a transmitting feeder-link earth station in the bands 14.5-14.8 GHz or 17.7-18.1 GHz, it shall effect coordination of this assignment with any administration whose territory lies wholly or partly within the coordination area of the feeder-link earth station, using the method described in Appendix 7, in respect of any assignment to a station of the fixed or mobile service in the bands 14.5-14.8 GHz and 17.7-18.1 GHz (see also Note to § 9A.2) which is:

a)	in conformity with No. 11.31, and

b)	either recorded in the Master Register prior to 3 June 2000 with a favourable finding; or

c)	for which a notice is received by the Bureau prior to 3 June 2000 for recording in the Master Register, but has not yet been processed, and which subsequently receives a favourable finding based on the WRC-2000 Regions 1 and 3 feeder-link Plan as it existed on 3 June 2000.

5	This assignment shall be brought into use only when the limits given in § 5 of Annex 1 are not exceeded, or with the agreement of administrations identified in Table 1A, whose networks/beams listed in this Table may be affected with respect to assignments which are in conformity with the Region 2 feeder-link Plan on 12 May 2000 (see also Note to § 9A.2).

6	This assignment shall not claim protection from interference caused by the assignments which pertain to networks/beams identified in Table 1B which are in conformity with the Region 2 feeder-link Plan on 12 May 2000 (see also Note to § 9A.2).

7	This assignment shall not claim protection from interference caused by the assignments which pertain to networks/beams identified in Table 1C which are recorded in the Master Register with a favourable finding prior to 12 May 2000 (see also Note to § 9A.2).

The methodology and criteria for this analysis shall be those contained in § 1 of Annex 4, modified to take into consideration the system noise temperature of the received space station to be 600 K and to apply a T/T criterion of 6%.

8	Provisional beam. These assignments have been included in the Regions 1 and 3 feeder-link Plan by WRC-97. These assignments are for exclusive use by Palestine, subject to the Israeli-Palestinian Interim Agreement of 28 September 1995, Resolution 741 of the Council notwithstanding and Resolution 99 (Minneapolis, 1998) of the Plenipotentiary Conference.

9	(Not used.)

10	Provisional beam. These assignments have been included in the Regions 1 and 3 feeder-link Plan by WRC-2000. These assignments are for exclusive use by East Timor.

NOTE – In cases where assignments from the WRC-97 Plans without Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna characteristics, the coordination status afforded by the WRC-97 Plans shall be preserved.

In cases where assignments from the WRC-97 Plans with Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the compatibility will be reassessed using the revised criteria and methodology of WRC-2000 and the Remarks of the WRC-97 Plans assignment will either be maintained or reduced on the basis of the results of this analysis.

In other cases, the methodology described in Notes 3 to 7 shall be applied.