ADD	ASP/25/139

DRAFT  RESOLUTION  [RNSS 1.2.3-1]  (WRC-03)

Protection of the radio astronomy service in the band 4 990-5 000 MHz from 
unwanted emissions of the radionavigation-satellite service (space-to-Earth)
operating in the frequency band 5 010-5 030 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from space stations of the radionavigation-satellite service (RNSS) operating in the frequency band 5 010-5 030 MHz may cause interference to the radio astronomy service (RAS) in the band 4 990-5 000 MHz;

b)	that WRC-2000 (Istanbul) decided to introduce a provisional power flux-density (pfd) limit in the band 4 990-5 000 MHz to protect the RAS, and invited ITU-R to conduct studies to review this limit;

c)	that protection requirements for the RAS are given in Recommendations ITU-R RA.769-1 and ITU-R RA.1513, and are different for GSO and non-GSO satellite systems,

noting

a)	that ITU-R has developed Recommendation ITU-R M.1583 providing a methodology for interference calculations between non-GSO MSS or RNSS satellite systems and radio astronomy telescope sites, and containing in its Annex 2 a model of radiotelescope antenna pattern;

b)	that all studies related to the protection of the radio astronomy service in the band 4 990-5 000 MHz are made using a reference radiotelescope antenna with a maximum gain of 74 dBi (a diameter of 100 m),

resolves

1	that in order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, the pfd produced in this band by any GSO RNSS network operating in the 5 010-5 030 MHz band shall not exceed 171 dB(W/m2) in a 10 MHz band at any radio astronomy station;

2	that in order not to cause interference to the radio astronomy service in the band 4 990-5 000 MHz, over the sky, for elevations higher than the operating elevation angle qmin specified for the radiotelescope, the equivalent power flux-density (epfd) produced in this band by all space stations within any non-GSO RNSS system operating in the 5 010-5 030 MHz band shall not exceed 245 dB(W/m2) in a 10 MHz band at any radio astronomy station for more than 2% of the time, using a reference antenna with one of the beam patterns described in Annex 2 of Recommendation ITU-R M.1583 and a maximum gain of 74 dBi;

3	that administrations planning to operate a GSO or a non-GSO RNSS system in the band 5 010-5 030 MHz, for which complete coordination or notification information, as appropriate, has been received by the Bureau after 2 June 2000, shall send to the Radiocommunication Bureau the value of the maximum level of pfd as referred to in resolves 1 or the value of the maximum level of epfd as referred to in resolves 2, as appropriate,

instructs the Radiocommunication Bureau

as of the end of WRC-03, to review all RNSS systems for which complete coordination or notification information, as appropriate, has been received by the Bureau before the end of WRC-03 for the band 5 010-5 030 MHz, and, if appropriate, to revise its findings regarding compliance with No. 5.443B, taking into account additional information received under resolves 3.

Reasons:	Incorporation with a new Resolution [RNSS 1.2.3-1] (WRC-03) in the footnote No. 5.443B is to clarify the application of the pfd/epfd limits.

Until adoption of a definition of min by ITU, and publication of notified radio astronomy observatory data, a value of 5° may be assumed in appropriate calculations.