MOD	ASP/25/18

RECOMMENDATION  517  (Rev.WRC-03)

RF protection ratio values for single-sideband (SSB) 
emissions in the HF bands allocated exclusively 
to the broadcasting service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-97 adopted Article 12 as the seasonal planning procedure for the HF bands allocated exclusively to the broadcasting service;

b)	that this procedure is based principally on the use of double-sideband (DSB) emissions;

c)	that the RF co-channel protection ratio is one of the fundamental planning parameters;

d)	that this Conference has adopted Resolution 517 (Rev.WRC-03) relating to the introduction of digitally modulated and SSB emissions in the HF bands allocated exclusively to the broadcasting service;

e)	that the SSB system characteristics for HF broadcasting are contained in Appendix 11;

f)	that studies have shown that SSB emissions may require a lower RF co-channel protection ratio for the same reception quality,

recommends

that, in the preparation of the relevant Rules of Procedure for the application of Article 12, the Bureau should use the values of RF protection ratio given in the Annex to this Recommendation relating to SSB and DSB emissions in the HF bands allocated exclusively to the broadcasting service.

ANNEX  TO  RECOMMENDATION  517  (Rev.WRC-03)

RF protection ratio values

1	The values of RF protection ratio given in the table should be used whenever SSB emissions in conformity with the specification in Appendix 11 are involved in the use of the HF bands allocated exclusively to the broadcasting service.

2	For the reception of DSB and SSB (6 dB carrier reduction relative to peak envelope power) wanted signals, a conventional DSB receiver with envelope detection designed for a channel spacing of 10 kHz is assumed.

3	For the reception of an SSB wanted signal (12 dB carrier reduction relative to peak envelope power), the reference receiver as specified in Appendix 11, Part B, § 3, is assumed.

4	SSB signals with 6 dB carrier reduction relative to peak envelope power assume equivalent sideband power as specified in Appendix 11, Part B, § 1.2.

5	The figures for case 2 in the following Table relate to a situation where the centre frequency of the intermediate frequency pass-band of the DSB receiver is tuned to the carrier frequency of the wanted SSB signal. If this is not the case, the value for a difference of +5 kHz may increase to –1 dB.

RF protection ratio values with reference to the co-channel RF protection ratio for DSB wanted and 
unwanted signals (dB)1 for use in the HF bands allocated exclusively to the broadcasting service

| 1 Frequency separation f less than –20 kHz, as well as f greater than 20 kHz, need not be considered. | Wanted signal | Unwanted signal | Carrier frequency separation f unwanted – f wanted, f (kHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | –20 | –15 | –10 | –5 | 0 | +5 | +10 | +15 | +20 |
| 1 | DSB | SSB (6 dB carrier reduction relative to p.e.p.) | –51 | –46 | –32 | +1 | 3 | –2 | –32 | –46 | –51 |
| 2 | SSB (6 dB carrier reduction relative to p.e.p.) | DSB | –54 | –49 | –35 | –3 | 0 | –3 | –35 | –49 | –54 |
| 3 | SSB (6 dB carrier reduction relative to p.e.p.) | SSB (6 dB carrier reduction relative to p.e.p.) | –51 | –46 | –32 | +1 | 0 | –2 | –32 | –46 | –51 |
| 4 | SSB (12 dB carrier reduction relative to p.e.p.) | SSB (12 dB carrier reduction relative to p.e.p.) | –57 | –57 | –57 | –45 | 0 | –20 | –47 | –52 | –57 |

Reasons:	To bring the Recommendation up to date.