MOD	ASP/25/182

ANNEX  1     (WRC-03)

Limits for determining whether a service of an administration is affected
by a proposed modification to the Region 2 Plan or by a proposed
new or modified assignment in the Regions 1 and 3 List
or when it is necessary under this Appendix to seek
the agreement of any other administration14

(See Article 4)

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of –103.6 dB(W/(m2 · 27 MHz)).

With respect to § 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 is considered by the Bureau as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.

However, an administration in Region 1 or 3 is considered as not being affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values:15

–147	dB(W/(m2 · 27 MHz))		for	0°	   0.23

–135.7 + 17.74 log 	dB(W/(m2 · 27 MHz))	for	0.23	   2.0

–136.7 + 1.66 2	dB(W/(m2 · 27 MHz))		for	2.0°	   3.59

–129.2 + 25 log 	dB(W/(m2 · 27 MHz))	for	3.59	   9

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin16 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

An administration in Region 3, of which the BSS Plan has national coverage only and has been brought in service before [DD.MM.YY], is considered as not being affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values15bis:

–147	dB(W/(m2 · 27 MHz))		for	0°	 θ < 0.23°

–135.7 + 17.74 log θ 	dB(W/(m2 · 27 MHz))	for	0.23°	 θ < 1.8°

–134.0 + 0.89 θ2	dB(W/(m2 · 27 MHz))		for	1.8°	 θ < 5.0°

–129.2 + 25 log θ	dB(W/(m2 · 27 MHz))	for	5.0°	 θ < 9°

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies;

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin16 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including the cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.

(Proposal number ASP/25/183 not used.)

1	4 	With respect to this Annex, except for Section 2, the limits relate to the power flux-density which would be obtained assuming free-space propagation conditions.

With respect to Section 2 of this Annex, the limit specified relates to the overall equivalent protection margin calculated in accordance with § 2.2.4 of Annex 5.

1	5 	For the protection of analogue assignments brought in service before 17 October 1997, the following values shall be used until 1 January 2015:

–147     dB(W/(m2  27 MHz))			for  0°             0.44°

–138 + 25 log      dB(W/(m2  27 MHz))	 for  0.44°        9°.

1	6 	For the definition of the equivalent protection margin, see § 3.4 of Annex 5.

1	5bis 	For the protection of analogue assignments brought in service before 17 October 1997, the following values shall be used until 1 January 2015:

–147     dB(W/(m2  27 MHz))			for  0°             0.44°

–138 + 25 log      dB(W/(m2  27 MHz))	 for  0.44°        9°.

1	6 	For the definition of the equivalent protection margin, see § 3.4 of Annex 5.