MOD	ASP/25/87

18.6-22.21 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.EEE | 21.4-22 FIXED MOBILE | 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.EEE 5.531 |

Reasons:	ITU-R studies have resulted that, in the band 22.21-22.5 GHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the BSS (space-to-Earth) operating in the band 21.4-22 GHz can meet the protection criteria for the VLBI observations only. Thus it is possible to establish mandatory pfd limits, contained in the proposed new footnote No. 5.EEE (ASP/25/88), to the unwanted emissions produced by space stations in the BSS (space-to-Earth) to protect the radio astronomy service in the band 22.21-22.5 GHz, without unduly constraining operations of space stations in the BSS in the band 22.4-22 GHz.