| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 16 to Document 27-E |
|  |  |  | 16 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Arab States Common Proposals |  |
| Proposal for the work of the conference |  |
| Agenda item 1.18 |  |

1.18	to consider a primary allocation to the fixed service in the band 17.3-17.7 GHz for Region 1, taking into account the primary allocations to various services in all three Regions

# 1	Introduction

The 17.3-17.7 GHz band is allocated to the FSS (Earth-to-space) in all three Regions. The use of this allocation by GSO networks is limited to BSS feeder links and subject to the Plans in Appendix 30A.

In Region 2, the band 17.3-17.7 GHz is also allocated (from 1 April 2007) to BSS. There are therefore three sharing issues related to BSS to be considered possible, namely:

1)	Interference from an FS station into BSS feeder-link space station receivers.

2)	Interference from BSS feeder-link earth stations into FS receive stations.

3)	Impact on the Region 2 BSS (space-to-Earth).

Allocation to the fixed service in Region 1 was considered with the major utilization of the band envisaged for PMP, e.g. for cable TV distribution. ITU-R studies showed that no new allocation could be made to the fixed service on a primary basis in Region 1 along with BSS feeder links.

# 2	Proposal

The Arab Administrations support no new allocation proposed to be made to the fixed service (FS) in Region 1 in the band 17.3-17.7 GHz.

ARTICLE  5

NOC	ARB/27A16/1

15.63-18.6 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| 5.514 | 5.514 5.515 5.517 | 5.514 |

Reasons:

1	To protect the current and future allocation of the BSS feeder links in the band 17.3-17.7 GHz.

2	BSS feeder-link earth stations may cause harmful interference to FS receivers. If FS receivers have to be protected from future BSS feeder-link earth stations in Regions 1 and 3, the deployment of future/additional BSS feeder-link users would be significantly constrained.

____________