MOD	ARB/27A2/13

Methodology and data

The software should use:

–	Recommendation ITU-R BS.705 for the calculation of antenna patterns;

–	Recommendation ITU-R P.533 for the prediction of the wanted field strength values at each test point for each wanted requirement;

–	Recommendation ITU-R P.533 for the prediction of the potentially interfering field-strength values from all other co-channel or adjacent channel requirements at each test point for each wanted requirement;

–	Recommendations 517 (HFBC-87) and ITU-R BS.560 for adjacent channel RF protection ratios;

–	Recommendation ITU-R P.842 for the calculation of reliability values.

The set of 911 test points (agreed at WARC HFBC-87) should be used, supplemented where necessary with test points based on a geographic grid.

The software should calculate the wanted and unwanted field-strength values and the fading margins at each test point inside the required service area.

The desired RF signal-to-noise and RF protection ratios should be user selectable, the default values being 34 dB and 17 dB (DSB-to-DSB co-channel case), respectively. In the case of digital emissions, the corresponding values are as provided in Appendix 11. The values for RF protection ratios should be used by the Bureau for its compatibility analyses.

Reasons:	Consequential change to the introduction of digitally modulated emissions.