ADD	ARB/27A2/4

DRAFT RESOLUTION  [RF-HF]  (WRC-03)

Interim RF protection ratio values for analogue and digitally 
modulated emissions in the HF broadcasting service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-03 introduced digitally modulated emissions in the HF broadcasting service;

b)	that Appendix 11 (Rev.WRC-03) gives details of the system parameters and the emission characteristics of the digitally modulated emissions;

c)	that ITU-R is carrying out further studies on the development of HF broadcasting using digitally modulated emissions in the bands allocated to the broadcasting service below 30 MHz;

d)	that the actual values of RF protection are not complete nor accurate;

e)	that ITU-R is conducting the development of a new Recommendation which should go through the approval process of ITU-R;

f)	that there is a need to compile and maintain statistics on administrations capability to introduce digital modulation systems for their HF broadcasting services,

resolves

1	that digital modulation in accordance with Resolution 517 (Rev.WRC-03) may be used in any of the HF bands allocated exclusively to the broadcasting service, this accommodation has to be made with the appropriate amounts of protection given to both analogue and digital emissions as described in the Annex to this Resolution;

2	that the Table in the Annex consists of RF protection ratios for co-channel and adjacent channel conditions, the independent variable in the table is the centre frequency separation in kHz of any pair of emissions, wanted versus unwanted;

3	that the Table in the Annex provides the required relative protection ratios for the DRM mode (Mode B3) that will be used extensively for HF skywave broadcasting in 10 kHz channels, the ratio data are in decibels;

4	that digital modulation governing these protection ratios appears in the Annex and the analogue modulation is double-sideband modulation as summarized in Part A of Appendix 11;

5	that the protection ratio values described in the Annex can be used by administrations as an interim protection ratios and shall be revised by WRC-07 for the purpose of revising the protection ratio figures or for confirmation,

invites ITU-R

1	to continue its studies on digital techniques in HF broadcasting with the purpose to revise the RF protection ratio values for analogue and digitally modulated emissions in the HF broadcasting service as described in the Annex to this Resolution;

2	to report the results of these studies to the World Radiocommunication Conference 2007,

instructs the Director of the Radiocommunication Bureau

to facilitate to the greatest extent possible the completion of these studies.

Annex  to  draft  resolution  [RF-HF]  (WRC-03)

Relative RF protection ratios (dB) between broadcasting systems below 30 MHz, and digital (64-QAM, protection level No. 1) interfered with by digital 
(identical robustness modes and spectrum occupancy types)

| Wanted signal | Unwanted signal | Frequency separation funwanted-fwanted (kHz) |  |  |  |  |  |  |  |  | Parameters |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | BDRM (kHz) | S/I (dB) |
|  |  | –20 | –15 | –10 | –5 | 0 | 5 | 10 | 15 | 20 |  |  |
| AM | DRM_B3 | –47 | –42 | –32 | 3 | 6 | 3 | –32 | –42 | –47 | 10 | – |
| DRM_B3 | AM | –54 | –48 | –40 | –3 | 0 | –3 | –40 | –48 | –54 | 10 | 7 |
| DRM_B3 | DRM_B3 | –53 | –47 | –38 | –3 | 0 | –3 | –38 | –47 | –53 | 10 | 16 |
| AM: DSB AM signal DRM_B3: DRM signal, robustness mode B, spectrum occupancy type 3 BDRM: Nominal bandwidth of DRM signal S/I: Signal-to-interference ratio for a BER of 104. |  |  |  |  |  |  |  |  |  |  |  |  |

Reasons:	The proposed Resolution is required with the addition of § 2.5 of Part C in Appendix 11.