MOD	ARB/27A22/2

5.502	In the band 13.75-14 GHz, an earth station of a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 mand an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW above 2 elevation and 65 dBW below.- An administration planning to use fixed-satellite service earth stations in a geostationary network in this band shall ensure that the single entry power flux-density produced by any earth station operating with an antenna size smaller than 4.5 m within its territory does not exceed:

–	–105.2 dB(W/(m2 · 10 MHz)) not to be exceeded for more than 1% of the time produced at 36 m above sea level at the measurement point to be specified by the concerned administration;

–	–105.2 dB(W/(m2 · 10 MHz)) not to be exceeded for more than 1% of the time produced 3 m above ground at the border of a neighbouring country deploying land mobile radar in this band.

The above power flux-density limits do not apply to earth station antennas with an antenna diameter of 4.5 m or greater.     (WRC-03)

Reasons:	The limits specified above consider a balance in sharing between radiolocation and FSS operating in the same band.