ADD	ARB/27A7/2

1.146ter	spurious domain (of an emission): The frequency range beyond the out-of-band domain in which spurious emissions generally predominate.

Reasons:	Adoption of these two new definitions will allow identifying the frequency ranges within the emission limits in Appendix 3 and recognizing the unwanted emissions based on their separation from the centre frequency of the emission.

2.2	The Arab Group proposes to modify Article 1 and Appendix 3 by providing specific exceptions from the general 2.5 BN boundary for narrow-band systems, wideband systems and other specific cases in various frequency ranges based on Recommendations ITU-R SM.1539 and ITU-R SM.1541 (Method B).

Reasons:	To set mandatory power limits of spurious emission for specified exceptions from the general boundary of 250% of the necessary bandwidth of the emission (2.5 BN) in the RR Appendix 3 in order to avoid or reduce harmful interference as low as possible.