ADD	RUS/30/16

5.516А	In the 17.3-18.1 GHz band assignments to feeder-link stations which are in conformity with the appropriate regional Plan or included in the Regions 1 and 3 List in Appendix 30A may also be used for transmissions in the fixed-satellite service (Earth-to-space) other than feeder links for the broadcasting-satellite service, provided that such transmissions do not cause more interference, or require more protection from interference, than the feeder-link transmissions operating in conformity with the Plan or the List, as appropriate.

APPENDIX  30A  (WRC-2000)

ARTICLE  5     (WRC-2000)

## 5.2	Examination and recording

5.2.1	The Bureau shall examine each notice:

...