MOD	IRN/32/15bis

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AMSS ADD 5.YYY Radio astronomy 5.149 |  |  |

Reasons:	The ITU-R studies carried out under Resolution 216 (Rev.WRC-2000) have demonstrated that appropriately designed and operated AMSS systems can operate on a secondary basis in the 14-14.5 GHz band without causing harmful interference to the primary services operating in the band. Studies have also shown the feasibility of AMSS systems sharing with systems using secondary allocations in the band. The pfd limits taken from ITU-R Recommendation DNR ITU-R M.[AMSS] has been included in a new footnote 5.AMSS and amend the Table of Frequency Allocations in Article 5 to refer to a new footnote 5.YYY that calls up Resolution [AMSS] (WRC-03).