ADD	IRN/32/16

5.AMSS	In the band 14-14.5 GHz, the maximum power flux-density produced at the surface of the Earth by emissions from a single aircraft earth station of an AMSS shall not exceed:

132 + 0.5*	dB(W/m2) in 1 MHz	for		 	40°

112			dB(W/m2) in 1 MHz		for	40° <	 	90°

where  is the angle of arrival of the radio-frequency wave (degrees above the horizontal).

The pfd limits may be exceeded on the territory of any country whose specific agreement has been obtained.

Reasons:	This would ensure the appropriate protection of the fixed service by enforcing the necessary limitations in the RR.