MOD	B/35/15

TABLE  21-4     (WRC-2000)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 3 400-4 200 MHz 4 500-4 800 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 7 250-7 850 MHz | Fixed-satellite (space-to-Earth, geostationary-satellite orbit) Meteorological-satellite (space-to-Earth) Mobile-satellite Space research | –152 | –152 + 0.5( – 5) | –142 | 4 kHz |
| 3 700-4 200 MHz | Fixed-satellite (space-to-Earth, non-geostationary-satellite orbit) | –147 | –147 + 1.15( – 5) | –124 | 1 MHz |
| ... |  |  |  |  |  |

Reasons:	Since there is no special definition for HEO satellites to reflect their particular characteristics, the FS in the 3.7-4.2 GHz band should be protected by the adoption of adequate limits on pfd produced by all types of non-GSO satellites including those in highly-elliptical orbits. These pfd levels should be limited mainly for the lowest angles of arrival in order to avoid situations in which highly-elliptical orbits may repeatedly appear within the main beams of FS antennas for significant duration of time. The proposed limits are identified as Mask C in the ITU-R CPM-02 Report.