ADD	B/35/8

draft  RESOLUTION  [xxx]  (WRC-03)

Procedures in case the operational or additional 
operational limits in Article 22 are exceeded

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-2000 adopted in Article 22 single-entry operational limits (see Tables 22-4A through 22-4C) and single-entry additional operational epfd¯ limits (see Table 22-4A1) applicable to non-geostationary (non-GSO) fixed-satellite service (FSS) systems (space-to-Earth) in certain parts of the frequency range 10.7-20.2 GHz to protect geostationary systems operating in the same frequency bands;

b)	that, taking into account Nos. 22.5H and 22.5I, wherever the limits referred to in considering a) are exceeded by a non-GSO FSS system to which the limits apply, this constitutes an infringement of No. 22.2, except where otherwise agreed between concerned administrations;

c)	that WRC-2000 identified the need for specific procedures that correct in the most expeditious manner any cases where the limits in considering a) are exceeded;

d)	that ITU-R has developed Recommendations ITU-R S.1527 and ITU-R S.1558 to assist administrations in identifying the source of interference in excess of the operational epfd limits and measuring levels of epfd¯ levels to verify compliance with the operational limits, respectively;

e)	that no procedures currently exist in the Radio Regulations to expeditiously address the unique regulatory situation of No. 22.5I,

resolves

that the procedures contained in the Annex be applied in the event of non-compliance with the single-entry operational and additional operational limits in Section II of Article 22 by a non-GSO satellite system in the FSS that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F.

Annex  to  draft  resolution  [xxx]  (wrc-03)

Procedures to be applied in the event of non-compliance with single-entry operational and additional operational limits in Section II of Article 22

1	It is essential that Member States exercise the utmost goodwill and mutual assistance in the application of these procedures for the expeditious elimination of equivalent power flux-density (epfd¯) interference from non-GSO satellite systems in the FSS at levels above the operational epfd¯ limits given in Tables 22-4A, 22-4B and 22-4C and/or the additional operational epfd¯ limits given in Table 22-4A1 (“excess epfd¯ interference”).

2	In securing the expeditious elimination of excess epfd¯ interference, due consideration should be given to all factors involved, including the relevant technical and operational factors.

3	Administrations should cooperate in the detection and elimination of excess epfd¯ interference.

4	Where practicable, and subject to agreement between the administrations concerned, the case of excess epfd¯ interference may be dealt with directly between their operating organizations.

5	When a case of excess epfd¯ interference to a frequency assignment in a geostationary-satellite network is detected at an operating earth station associated with the geostationary-satellite network and such excess epfd¯ interference cannot be accepted by the affected administration, the affected administration should first attempt to identify the source of the excess epfd¯ interference. For purposes of these procedures, the term “affected administration” shall mean the administration on whose territory the receiving earth station associated with the geostationary-satellite network is located or its designee.

6	If an affected administration referred to in § 5 has difficulty in determining the source or characteristics of the excess epfd¯ interference:

a)	It may send a request for cooperation to any administration which has submitted to the Bureau complete advance publication, coordination, or notification information, as appropriate, for non-GSO FSS systems with overlapping frequency assignments that have been brought into use in the frequency bands subject to the limits referred to in § 1, providing all relevant details in a report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd¯) limits in Section II of Article 22 utilizing the form provided in the Appendix to these procedures. A copy of any such request, including the report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd¯) limits in Section II of Article 22, should be sent to the Bureau.

abis)	It may request the assistance of the Bureau to identify the administrations referred to in § 6 a). Upon receipt of such a request for assistance, the Bureau should promptly communicate to the requesting administration the list of administrations which have submitted to the Bureau complete advance publication, coordination or notification information, as appropriate, for a non-GSO satellite system in the FSS in the frequency bands referred to in § 1 with overlapping frequency assignments that have been brought into use. Upon receipt from the Bureau of the list of administrations, the affected administration should then apply § 6 a).

b)	Upon receipt of such a request for cooperation under § 6 a), each administration should, as soon as possible but within 30 days, acknowledge receipt and send to the requesting administration(s), with a copy to the Bureau, information that may be used to identify the source of the excess epfd¯ interference and/or to eliminate one or more non-GSO FSS systems referred to in § 6 a) as the source of the excess epfd¯ interference.

c)	If an administration fails to respond within 30 days of receipt to a request for cooperation under § 6 a), an affected administration may request the assistance of the Bureau, in which case the Bureau should forthwith request the non-responding administration to provide the information referred to in § 6 b) within 30 days of an affected administration’s request for the assistance of the Bureau.

d)	If an administration fails to respond to the Bureau within the time period established in § 6 c) above, the Bureau should:

–	if the procedure of Article 11 has not been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, publish a remark in the IFIC within one month to the effect that the responsible administration did not respond to a request for cooperation regarding an unresolved complaint of excess epfd¯ interference; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the responsible administration did not respond to a request for cooperation regarding an unresolved complaint of excess epfd¯ interference.

7	Upon receipt of the information identified in § 6 a), the Bureau should promptly communicate to all administrations contacted under § 6 a) the identity of any non-GSO FSS systems that, on the basis of determining compliance with the epfd¯ validation limits in Tables 22-1A through 22-1D, the Bureau has concluded to have a maximum epfd¯ lower than the limits referred to in § 1, for all pointing directions towards the geostationary-satellite orbit and therefore would not be responsible for causing epfd¯ interference in excess of the limits referred to in § 1.

8	Once the source(s) of the excess epfd¯ interference have been identified, an affected administration may send a letter, by fax or other mutually agreed electronic means, to the administration(s) concerned and request immediate corrective action. It should give all useful information, including a report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd¯) limits in Section II of Article 22, to enable the responding administration(s) to take such steps as may be necessary to reduce the interference to the epfd¯ levels required in Table 22-4A, 22-4A1, 22-4B or 22-4C, as appropriate, or to higher levels as may otherwise be or have been agreed between concerned administrations pursuant to No. 22.5I. A copy of any such request for immediate corrective action, including the report of excess epfd¯ interference, should be sent to the Bureau.

9	Upon receipt of such a request for immediate corrective action under § 8, an administration should acknowledge receipt to the requesting administration within 30 days, with a copy to the Bureau. Such acknowledgement would not constitute acceptance of responsibility.

10	Within 30 days after receipt of a request for immediate corrective action pursuant to § 8 above, the administration receiving the request should either:

a)	provide the requesting administration and the Bureau with information indicating that no non-GSO FSS system for which it is responsible could have caused the excess epfd¯ interference experienced by the receiving earth station associated with the geostationary-satellite network; or

b)	acknowledge responsibility for causing the excess epfd¯ interference and immediately reduce emissions of the interfering system into the affected receiving earth station associated with the geostationary-satellite network to the epfd¯ levels specified in Table 22-4A, 22-4A1, 22-4B or 22-4C, as appropriate, or to the epfd¯ levels otherwise agreed between concerned administrations pursuant to No. 22.5I, whichever is higher. Full particulars of the action taken by the administration responsible for causing the excess epfd¯ interference should be provided to the requesting administration.

In either case, the Bureau should be informed of the action taken.

11	If an administration fails to act in accordance with § 10 above, an affected administration may request the assistance of the Bureau, in which case the Bureau should forthwith request the non-responding administration to act in accordance with § 10 within 30 days of the affected administration’s request for the assistance of the Bureau.

12	If the administration fails to respond to the Bureau within the time period established in § 11 above, the Bureau should:

–	if the procedure of Article 11 has not been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, publish a remark in the IFIC within one month to the effect that the responsible administration did not respond to a request for immediate corrective action regarding an unresolved complaint of excess epfd¯ interference; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the responsible administration did not respond to a request for immediate corrective action regarding an unresolved complaint of excess epfd¯ interference.

13	If an administration acknowledges responsibility for causing the excess epfd¯ interference pursuant to § 10 b) above, but fails to reduce immediately emissions of the interfering system as required:

a)	It should have an additional ten days to take the necessary action to correct the excess epfd¯ interference situation pursuant to No. 15.21 of the Radio Regulations.

b)	If, after the ten-day period, the administration responsible for the interference has still not reduced emissions of the interfering system as required, the Bureau should:

–	if the procedure of Article 11 has not been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, publish a remark in the IFIC within one month to the effect that the responsible administration is in contravention of its obligations under No. 22.2 and No. 22.5I; or

–	if the procedure of Article 11 has been completed for the frequency assignments of the non-GSO satellite system in the FSS in question, enter a remark in the Remarks column of the Master Register against the relevant frequency assignments of the non-GSO FSS system in question to the effect that the use of the affected frequency assignments by the interfering system is in contravention of its obligations under No. 22.2 and No. 22.5I of the Radio Regulations. Notice of the entry of the remark should be included in the IFIC.

14	The Bureau shall retain any entry in the Remarks column of the Master Register made pursuant to § 6 d), 12 or 13 b) above, which shall remain in place until such time as the non-responding administration responds and/or corrects the excess epfd¯ interference, as appropriate.

Appendix  to  Annex  to  draft  resolution  [xxx]  (Wrc-03)

Report of non-compliance with single-entry operational and additional operational equivalent power flux-density (epfd¯) limits 
in Section II of Article 22

Part I: Particulars concerning the non-GSO satellite system exceeding the operational epfd¯ limits

1)	Name of the non-GSO satellite system, if known, or other means of identification:

2)	Frequency measured:

a)	Date:

b)	Time (UTC):

3)	Class of emission:

4)	Bandwidth (indicate whether measured or estimated):

5)	Measured epfd¯:

a)	Date:

b)	Time (UTC):

c)	Duration (seconds):

d)	Repeatability (Yes/no):

If yes, associated period (seconds):

6)	Applicable epfd¯ limit in Article 22 Tables 22-4A through 22-4C:

7)	Observed polarization:

8)	Class of station and nature of service, if known:

Part II: Particulars furnished by the administration responsible for the operating geostationary fixed-satellite service earth station receiving epfd¯ levels exceeding the operational limits in Tables 22-4A through 22-4C

1)	Name of earth station:

2)	Antenna diameter of the receiving earth station:

3)	Name of the transmitting GSO space station interfered with, if known or other means of identification:

4)	Longitude of the associated geostationary-satellite network:

5)	Orbital inclination of the associated geostationary-satellite network:

6)	Latitude, longitude, elevation and azimuth of the earth station at which the epfd¯ levels exceeded the operational limits in Tables 22-4A through 22-4C:

7)	Nature of interference:

8)	Characteristics of the wanted emission at the receiving station:

a)	Class of emission:

b)	Bandwidth (indicate whether measured or estimated, or indicate the necessary bandwidth notified to the Radiocommunication Bureau):

c)	Frequency measured:

Date:

Time (UTC):

d)	Field strength or power flux-density:

Date:

Time (UTC):

9)	Polarization of the receiving antenna or observed polarization:

10)	Action requested:

Reasons:	This draft Resolution would assist GSO earth station operators in identifying and expeditiously resolving exceedances of the limits in No. 22.5I.