ADD	CME/42/36

draft resolution [xxx] (WRC-03)

Identification of additional spectrum for the 
broadcasting service in the HF bands

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the spectrum allocated to the broadcasting service from 4 MHz to 10 MHz is about 25% of all spectrum allocated to HF broadcasting bands;

b)	that WARC-79 allocated only an additional 125 kHz of spectrum to the HF broadcasting service below 10 MHz (9 775-9 900 kHz);

c)	that WARC-92 allocated an additional 200 kHz HF to the broadcasting service consisting of 100 kHz at 9 MHz, 50 kHz at 7 MHz and 50 kHz at 6 MHz. This additional spectrum will become available to the broadcasting service from [1 April 2007];

d)	that the agenda for WRC-07 includes the revision of allocations to the services in HF bands;

e)	that the results of coordination under Article 12 demonstrate that the broadcast bands below 10 MHz are congested, even when there are high levels of sunspot activity, with little more than half of the broadcasting requirements being satisfied;

f)	that in the most recent periods of seasonal planning, the statistics show that around 250 kHz of additional spectrum is needed to clear the co-channel collisions and up to 800 kHz to clear both the co-channel and adjacent channel collisions in the bands below 10 MHz;

g)	that the introduction and promotion of the new digital technology, that improves spectrum utilization and efficiency, cannot completely solve current congestion problems;

h)	that the broadcasting service, in an era of convergence of services, will play an increased socio-political role,

resolves

that the following conference should conclude on additional spectrum requirement and the identification of additional frequencies for the broadcasting service, taking account of the latest technical, operational and economic results and other studies pertaining to the following bands:

[4 500-4 650 kHz]

5 060-5 250 kHz

5 840-5 900 kHz 1

[7 350-7 650 kHz 1, 2]

[9 290-9 400 kHz 1]

[9 900-9 940 kHz 1]

invites ITU-R

1	to carry out technical studies on this matter, taking into account technical, operational, economic and other relevant factors;

2	to bring the results of these studies to the attention of WRC-07,

urges administrations

to participate actively in the aforementioned studies by submitting contributions to ITU-R,

Reasons:	The implementation of digital techniques will probably make it possible to find better solutions to the problems of congestion in the bands allocated to HF broadcasting.

A clearer understanding could also be obtained of the problems of compatibility with other services.

The wording of draft Resolution [XXX] in the CPM Report concerns the identification of additional bands and not allocations as mentioned in the resolves.

1	Bands adjacent to the HF broadcasting bands governed by Article 12.

2	Band location may need to be revised in light of actions decided in respect of WRC-03 agenda item 1.23.