ADD	CAN/43/12

5.XXX	Design and operation of Earth exploration-satellite service (active) sensors in the band 432-438 MHz shall be in accordance with the provisions set out in [draft revision of] Recommendation ITU-R SA.1260. Additionally, in countries listed in No. 5.281, the Earth exploration-satellite service sensors shall not operate during launch windows of space vehicles to protect the existing range safety command receivers (space operation service) operating in the 433.75-434.25 MHz band.

Reasons:	To identify the constraints to be applied to EESS (active) to ensure the protection of existing services.