| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 46-E |
|  |  |  | 6 May 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| France, Italy, United Kingdom of Great Britain and Northern Ireland |  |
| proposals for the work of the conference |  |
| agenda item 1.11 - amss in the 14-14.5 GHz band |  |

Introduction

Agenda item 1.11 requests the Conference to consider the possible extension of the allocation to the mobile-satellite service (MSS) (Earth-to-space) on a secondary basis in the band 14-14.5 GHz to permit operation of the aeronautical mobile-satellite service (AMSS).

Europe has already submitted a European Common Proposal (ECP) that supports the extension of the allocation to the MSS uplink on a secondary basis in the band 14-14.5 GHz for the aeronautical mobile-satellite service (Earth-to-space) provided that the technical and operational limitations specified in the DNR ITU-R M.[AMSS] are applied to the AMSS in order to provide adequate protection of the terrestrial, space services and the radio astronomy service in the band from harmful interference due to stations operating in the aeronautical mobile-satellite service.

Recognizing that some European administrations operate FS or radio astronomy systems in the 14.25-14.5 GHz band, the ECP on agenda item 1.11 accepts that some of these administrations will propose to WRC-03, on a national basis, country footnotes that would specify the necessary protection in the RR as detailed in DNR ITU-R M.[AMSS] in order to make the technical and operational constraints imposed to AMSS stations that could impact the stations on their territory.

France, Italy and the United Kingdom operate fixed service networks in the band 14.25-14.5 GHz and perform radio astronomy measurements in the 14.47-14.5 GHz band and are of the view that it is necessary to specify in the RR provisions to ensure the protection of these services.

On this basis, France, Italy and the United Kingdom propose, as a complement to the ECP, two country footnotes that reference the provisions of DNR ITU-R M.[AMSS] related to the fixed service and to radio astronomy, making clear that these provisions, mainly pfd limits, are only intended to apply on the territory of administrations deploying, or intending to deploy FS or RAS stations in the 14.25-14.5 GHz frequency range.

Proposals

ARTICLE  5

Frequency allocations

MOD	F/I/G/46/1

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.AAA Space research 5.505 |  |  |

Reasons:	The protection of the RAS in the 14.47-14.5 GHz as given in [draft new] Recommendation ITU-R M.[AMSS], specify, among other, out-of-band emission limits that apply to all AMSS stations in the band 14-14.5 GHz. Footnote 5.BBB is not referenced in this band since there is no FS allocation in France, Italy and the United Kingdom in the band 14-14.25 GHz.

MOD	F/I/G/46/2

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Space research (space-to-Earth) |  |  |

14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile-satellite Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radio astronomy 5.149

Reasons:	The protection of the RAS in the 14.47-14.5 GHz as given in [draft new] Recommendation ITU-R M.[AMSS], specify, among other, out-of-band limits that apply to all AMSS stations in the band 14-14.5 GHz. Footnote 5.BBB is referenced in these bands since FS is operated in France, Italy and the United Kingdom in the band 14.25-14.5 GHz.

ADD	F/I/G/46/3

5.AAA	Aircraft earth stations operating in the aeronautical mobile-satellite service in the band 14-14.5 GHz shall comply with the provisions of Annex 1, Section C of [draft new] Recommendation ITU-R M.[AMSS], with respect to any radio astronomy station performing observations in the 14.47-14.5 GHz band located on the territory of France, Italy and the United Kingdom.

Reasons:	In line with the results of the discussions in ITU-R and the intent of Annex 1, Section C of DNR ITU-R M.[AMSS].

ADD	F/I/G/46/4

5.BBB	In the band 14.25-14.5 GHz, the power flux-density produced on the territory of France, Italy and the United Kingdom by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed the power flux-density limits given in Annex 1, Section B of [draft new] Recommendation ITU-R M.[AMSS] unless otherwise agreed by the affected administration(s).

Reasons:	The pfd limits in Annex 1, Section B of DNR ITU-R M.[AMSS] only apply to emissions in the band of the fixed service. This footnote is limited to the band 14.25-14.5 GHz since there is no FS allocation in France, Italy and the United Kingdom in the band 14-14.25 GHz.

____________