MOD	F/I/G/46/2

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Space research (space-to-Earth) |  |  |

14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile-satellite Mobile-satellite (Earth-to-space) ADD 5.AAA ADD 5.BBB Radio astronomy 5.149

Reasons:	The protection of the RAS in the 14.47-14.5 GHz as given in [draft new] Recommendation ITU-R M.[AMSS], specify, among other, out-of-band limits that apply to all AMSS stations in the band 14-14.5 GHz. Footnote 5.BBB is referenced in these bands since FS is operated in France, Italy and the United Kingdom in the band 14.25-14.5 GHz.