ADD	EGY/48/1

Draft  Resolution  [XXX]  (WRC-03)

Public protection and disaster relief

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the growing telecommunication needs of public agencies and organizations dealing with law and order, disaster relief and emergency response;

b)	that future advanced solutions used by such public protection and disaster relief agencies and organizations will require high data rates;

c)	that there is a need for interoperability and interworking between public protection and disaster relief (PPDR) networks, both nationally and for cross-border operations, in emergency situations and disaster relief;

d)	the importance of the needs of public protection agencies and organizations, including those dealing with emergency situations and disaster relief for:

–	maintenance of law and order;

–	emergency and disaster response;

–	protection of life and property;

e)	that the provision of appropriate spectrum resources to PPDR is becoming increasingly important to the maintenance of a stable and prosperous society;

f)	that current PPDR applications are mostly narrow-band, including voice and low data-rate applications, typically in channel bandwidths of 25 kHz or less;

g)	that although there will continue to be narrow-band requirements, many future applications will be wideband (indicative data rates in the range of 384-500 kbit/s) and/or broadband (indicative data rates in the range of 1-100 Mbit/s) with channel bandwidths dependent on the use of spectrally efficient technologies;

h)	that new technologies for wideband and broadband PPDR applications are being developed in various standards organizations: in particular, a joint standardization programme (known as Mobility for Emergency and Safety Applications (MESA)) between ETSI and TIA has commenced for broadband PPDR;

i)	that there is potential for new technologies such as IMT-2000 and systems beyond IMT-2000 and Intelligent Transportation Systems (ITS) which may support or supplement advanced PPDR applications;

j)	that commercial systems may serve as a complement to dedicated systems in support of PPDR and that such complementary use would be in response to market demands;

k)	that Resolution 98 (Minneapolis, 1998) of the Plenipotentiary Conference urges Member States to facilitate use of telecommunications for the safety and security of humanitarian personnel,

recognizing

a)	the importance of interoperability in the provision of spectrum for public protection and disaster relief;

b)	the benefits of globally and regionally harmonized frequency bands, such as:

–	increased potential for interoperability;

–	a broader manufacturing base and increased volume of equipment resulting in economies of scale and expanded availability of equipment;

–	improved spectrum management and planning;

–	enhanced cross-border coordination;

–	improved cross-border circulation of equipment;

c)	that spectrum planning for PPDR is done at the national level, taking into account the need for interoperability and benefits of harmonization with neighbouring administrations;

d)	the increased benefits of cooperation between countries for the provision of effective and appropriate humanitarian assistance during disasters;

e)	the special needs of developing countries, taking into account the ITU-D Handbook on disaster relief;

f)	the needs of countries, particularly for developing countries, for low-cost communications equipment for public protection and disaster relief agencies and organizations;

g)	that spectrum and innovative spectrum management may be required for the implementation of future advanced solutions;

h)	that the trend is to increase the use of technologies based on Internet protocols;

i)	that currently some bands or parts thereof below 1 GHz have been designated for exclusive use for public protection and disaster relief,

noting

a)	that many administrations use frequency bands below 1 GHz for narrow-band PPDR applications;

b)	applications requiring large coverage areas and providing good signal availability would generally be accommodated in lower frequency bands;

c)	applications requiring wider bandwidths would generally be accommodated in progressively higher bands;

d)	that PPDR agencies and organizations have a minimum set of requirements, including but not limited to, interoperability, secure and reliable communications, sufficient capacity to respond to emergencies, priority access in use of non-dedicated systems, fast response times, ability to handle multiple group calls and ability to cover large areas;

e)	that in most administrations, PPDR applications are provided at several levels, from national down to local, and cooperation between the levels is a domestic matter which harmonized spectrum and interoperability could facilitate;

f)	that, while harmonization may be one method of realizing the benefits stated above, in some countries multiple frequency bands can be a component of meeting the communication needs in disaster situations,

resolves

1	to urge administrations to use globally and regionally harmonized bands for PPDR, to the maximum extent possible;

2	to provide flexibility for disaster relief agencies and organizations by indicating that the identification of bands specifically for PPDR does not preclude the use of these bands by any application in services to which these bands are allocated and does not preclude the use of any other frequencies for disaster relief applications in accordance with the Radio Regulations;

3	to urge administrations, in emergency and disaster relief situations, to satisfy temporary needs for frequencies in addition to what may be normally provided for in agreements with neighbouring administrations;

4	to urge administrations to encourage PPDR agencies and organizations to utilize both existing and new technologies and solutions (satellite and terrestrial), to the extent practicable, to satisfy PPDR interoperability requirements and to further the goals of public protection and disaster relief;

5	that administrations should encourage agencies and organizations to use advanced wireless solutions for providing complementary support to PPDR agencies and organizations1,

resolves to recommend

1	that the following band be identified as bands for PPDR use at WRC-03:

851-869 MHz

(plus, possibly, other frequency bands to be identified at WRC-03);

2	that as necessary, administrations continue to work closely with their public protection and disaster relief community to further refine the identification of future spectrum requirements and possible methods to meet these requirements;

3	that administrations encourage PPDR agencies and organizations to utilize relevant ITU-R Recommendations in planning and implementing spectrum, technology and systems for public protection and disaster relief,

invites ITU-R

to conduct appropriate technical studies in support of the implementation of PPDR applications in the identified bands.

1	 	Some administrations believe that IMT-2000 and ITS are examples of such advanced wireless solutions.