ADD	ISR/56/4

e)	that after a transition period, the broadcasting service may release in Region 1 spectrum in the band 806-862 MHz for mobile services.

Reasons:	Resolution 224 (WRC-2000) “Frequency bands for the terrestrial component of IMT-2000 below 1 GHz” considering a) states “that parts of the band 806-960 MHz are extensively used in the three Regions by first- and second-generation mobile systems. The band 790-862 MHz is presently allocated in Region 1 to the broadcasting service. Many countries in Region 1 do include in RR 5.316 also mobile services on a primary basis; 5.317A states that “Administrations wishing to implement International Mobile Telecommunications-2000 (IMT-2000) may use those parts of the band 806-960 MHz which are allocated to the mobile service on a primary basis”. This band could be implemented by administrations wishing to do so for IMT-2000, when the transition to Digital Video Broadcasting-Terrestrial (DVB-T) is accomplished. This will reduce the total frequencies available to the broadcasting service; however, the reduction is compensated by DVB-T main characteristics. DVB-T technology allows a better use of RF spectrum, compared to analogue TV: