ADD	AFCP/57/32

ADD resolves 2bis of Resolution 228 (WRC-2000):

2bis	to invite ITU-R to make regulatory and technical studies on the usage of frequencies below 600 MHz for IMT-2000 systems, notably assessing their advantages and disadvantages;

African common proposals for agenda item 1.23

1.23	to consider realignment of the allocations to the amateur, amateur-satellite and broadcasting services around 7 MHz on a worldwide basis, taking into account Recommendation 718 (WARC-92)

Introduction

Studies in response to Recommendation 718 (WARC-92) have been ongoing in ITU-R for quite some time.

The purpose of carrying out a realignment of the bands around 7 MHz is to remedy the long-standing difficulties experienced by the amateur service (AS) and the limitations placed on the broadcasting service as a result of the changes made to the frequency bands around 7 MHz.

For the amateur service the usefulness of the allocations around 7 MHz for worldwide links is restricted because only 100 kHz of spectrum between 7 000 and 7 100 kHz is common in Region 2 and Regions 1 and 3. The band 7 100-7 300 kHz is allocated exclusively to the broadcasting service in Regions 1 and 3 and exclusively to the amateur service in Region 2.

Proposal